3GPP TSG-SA WG1 Meeting #111 	S1- 253318r1

25-29 August 2025, Goteborg, Sweden	(revision of S1-253318)

Source:	EDF, Novamint, ITRON

New use case title:	New use case on 6G-enabled Decentralized Local Grid Power Contract

Draft TS/TR:	3GPP TS / TR 22.870

Agenda item:	8.1.8

Document for:	Approval

Contact:	vincent.audebert@edf.fr

Abstract: This document proposes a new use case on 6G-enabled Decentralized Local Grid Power Contract.

---------- Use Case template ----------

## 11.x	Use case on 6G-enabled decentralized grid power contract

### 11.x.1	Description

Production and consumption of electricity in a distribution grid is becoming more and more complex. The behaviour of the customer connected to the grid could vary from importing electricity from the grid or producing electricity to the grid, depending on time of the day and external conditions (market value, weather etc)

There is also a local behaviour, a way to share local production between neighbours in the same LV grid or in a slightly larger aera. Currently, these local communities are energy based, the behaviour of the community is not working in real time and the settlement is done during the billing time. An example of this type of community can be find in France as “collective self-consumption”.

The actors of the energy system are more and more concern by the real-time balance of the grid. It will be a good value for the electricity system if some actor could act in predictable way, following a predictable contracted load, whether it is consumption or production.

This use case shows a need for the 6G to handle a local peer to peer communication between electricity meters, cyber secured and resilient, so the contract of power impact to the energy system could be respected.

Table 11.x.1-1: Potential sustainability impacts of the Decentralized Local Grid Power Contract use case

|  | (the UN SDGs/GDC matching goals of each aspect within 3GPP context) | Potential benefits of the use case (added value) | Potential areas of attention of the use case (risks to be mitigated) |
| --- | --- | --- | --- |
| Environmental sustainability aspects (UN SDGs 12, 13, 14, 15 and indirectly 6, 7 & 11. UN GDC “Develop principles for environmental sustainability of digital technologies”) | Energy resources (UN SDG 7, 11, 12) | - Improved usage of local energy production |  |
|  | Material resources (UN SDG 11, 12) | - Avoid the need to build fast frequency response equipment since the behaviour is predictable by the electricity system |  |
|  | Emissions (UN SDG 6, 7, 11, 12, 13, 14, 15) | - Improve the integration of local renewable system that can avoid building high CO² emission generation based on coal, oil, gas. |  |
| Socio-economic sustainability aspects (UN SDGs 2, 3, 4, 5, 8, 9, 10, 11, 16 & 17 and indirectly 12. UN GDC “Closing Digital Divides and Accelerating SDG Progress” & “Expanding Digital Economy Inclusion” & “Fostering an Inclusive, Safe Digital Space”) | Trustworthiness (UN SDGs 11 and indirectly 3 & 17) | - Avoid the unpredictability of the power consumption of the community |  |
|  | Work & income (UN SDG 8 and indirectly 12) | - Optimal behaviour of the community provides them a better price for electricity |  |
|  | Infrastructure (UN SDG 9) | - Improving stability of the electric grid |  |
|  | TCO reduction (UN SDGs 8, 9 and 12) | - Avoid oversize dimensioning of production or battery in a local distribution grid |  |

### 11.x.2	Pre-conditions

- A community of local users of the grid decide to act coordinate and propose a targeted electricity power value interval to an electricity system actor for a period using the 6G system.

- Through the 6G system, the electricity system actor provided the power limit and timeframe for this community.

### 11.x.3	Service Flows

- During one electricity period (50 Hz – 20 ms or 60 Hz – 17 ms), the meter of each community user calculates his power impact on the grid and broadcast this information to the other meters of the community using 6G network.

- Using the information transmitted by their peers, the meter with his onboarded algorithm (could be classical multi agent or AI) modify the impact on the grid of the premises, using the controlled appliances located inside the premises (water heater, photovoltaic inverter, electric vehicle, heat pump, pool pump, static batteries…). The communication between the meter and the appliances is not part of this document.

- Each meter repeats bullet 1 till the end of the contract period.

### 11.x.4	Post-conditions

- The local community manage to behave the way they have contracted with the electricity system actor.

### 11.x.5	Existing features partly or fully covering the use case functionality

- TS 22.104: Service requirements for ultra-reliable low latency communications (URLLC).

- TS 22.261: Service requirements for the 5G system; includes general support for IoT, URLLC, and industrial use case

### 11.x.6	Potential New Requirements needed to support the use case

[PR 11.x.6-1] The 6G system shall support local secure indirect and direct device  multicast communication under the communication backbone.

[PR 11.x.6-2] The 6G system shall be able to guarantee a maximum latency <1 ms end-to-end locally and to support the following KPIs associated:

.

Table 11.x.2-1: Communication KPI for Decentralized Local Grid Power Contract use case

| Use case # | Characteristic parameters |  |  |  | Influence quantity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value [%] | Communication service reliability: mean time between failures | End-to-end latency: maximum | Service bitrate: user experienced data rate | Message size [byte] | Transfer interval: target value | Survival time | UE speed | # of UEs | Service area |
| 1 | 99.999 | – | < 1 ms | < 100 kbps | < 100 | n/a (note) | – | stationary | <200 | 2 km radius |
| NOTE: event-triggered in 50 Hz or 60 Hz |  |  |  |  |  |  |  |  |  |  |