3GPP TSG-SA WG2 Meeting #170	S2- 2507089

25 - 29 August, 2025, Goteborg, Sweden

Title:	23.700-14: New Solution for Key Issue#4: NRPPa Extension over Direct Connection

Source:	ETRI

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract:	This contribution proposes a solution to support Key Issue #4 by introducing NRPPa-based interface support for sensing session control and efficient data delivery based on Solution 6.1.

Proposal

This contribution proposes a new solution, "NRPPa Extension over Direct Connection", based on Solution 6.1, to be included in TR 23.700-14. The goal is to enhance support for direct interface-based sensing operations between the Sensing Function and the Sensing Entity (e.g., gNB).

The motivation for this proposal arises from the need to address the following Editor's notes stated in Solution 6.1:

Editor's note:	The exact protocols to be used for the direct connection and direct interface are FFS.

Editor's note:	The NRPPa defined in TS 38.455 [6] is an example of such end to end protocol. However, to support the sensing operations over the direct connection, some adaptation or enhancements would be required.

This proposal introduces specific enhancements to the existing NRPPa protocol (as defined in TS 38.455 [6]) to support both control and user plane mechanisms required for sensing session management, capability signaling, and scalable, high-throughput data delivery.

The proposed extensions include:

-	Control Plane Enhancements: Sensing session setup, configuration, and capability exchange, and reporting between Sensing Function and Sensing Entity.

-	User Plane Enhancements: GTP-U-based data tunnel establishment over N3 interface to enable efficient and scalable sensing data transport, especially in scenarios with non-UE-associated traffic.

* * * First Change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 | #6 |
| #1 | X |  | X | X | X |  |
| #2 |  | X |  |  | X |  |
| #3 | X | X | X | X | X | X |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  | X |  |  |
| #6 | X | X | X |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 |  | X |  |  |  |  |
| #10 |  | X |  |  |  |  |
| #11 |  | X |  |  |  |  |
| #12 |  | X |  |  |  |  |
| #13 |  |  | X |  |  |  |
| #14 |  |  | X |  |  |  |
| #15 |  |  | X |  |  | X |
| #16 |  |  | X |  |  |  |
| #17 |  |  | X |  |  |  |
| #18 |  |  |  | X |  |  |
| #19 | X | X |  |  | X |  |
| #20 |  |  |  |  |  | X |
| #X |  |  |  | X |  |  |

6.X	Solution #X: NRPPa Extension over Direct Connection

6.X.0	High-level Solution Principles

Editor's note:	Where possible similar/overlapping solution proposals should be documented as a single solution in the TR.

Documentation of more than one solution from the same company for the same KI is allowed but it shouldn't be expected that all of them will be documented in the TR.

This solution targets Key Issue #4 by enabling end-to-end sensing operations over a direct interface between the Sensing Function and the Sensing Entity (e.g., gNB), building upon the architectural assumptions of Solution 6.1. The approach extends the existing NRPPa protocol (as specified in TS 38.455 [6]) to support sensing functionalities across both the control plane and user plane:

-	Control Plane: Enhancements support sensing session management, capability exchange, event reporting, and data transfer setup, encapsulated within NGAP signaling.

-	User Plane: A non-UE-associated GTP-U tunnel over the N3 interface enables scalable and efficient sensing data delivery, with optional QoS support.

The solution leverages the existing 5G core architecture and standard interfaces (e.g., N2, N3) to ensure seamless integration of sensing capabilities with minimal architectural impact.

6.X.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Clause(s) may be added to capture details.

This solution addresses Key Issue #4: Sensing Data and the Associated Information Collection and Transport by defining NRPPa-based signaling extensions for end-to-end sensing operations over the direct interface between the Sensing Function and the Sensing Entity (e.g., gNB). The proposed solution builds upon the architectural assumptions in Solution 6.1.

To enable sensing service operations over the direct interface, the NRPPa protocol (as defined in TS 38.455 [6]) is extended to support sensing functionalities in both the control plane and user plane as described below.

6.X.1.1	Control Plane Extension of NRPPa

While NRPPa currently supports positioning services, it can be enhanced to support sensing functionalities as follows:

-	Sensing Service Request/Response: Enables the Sensing Function to initiate, configure, or terminate a sensing session at the Sensing Entity. The request may include parameters such as sensing area, desired sensing resolution, data type, and reporting frequency. The Sensing Entity (e.g., gNB) responds with an acknowledgment or rejection, optionally including its sensing capabilities.

-	Sensing Capability Indication: Allows the gNB to report its sensing capabilities to the Sensing Function, including supported sensing modes, maximum detection range, accuracy, and available sensing resources.

-	Sensing Event Reporting: Allows the gNB to send notifications to the SF regarding specific sensing events (e.g., object detection, environmental changes) or to report the status of a sensing session (e.g., completion or error conditions). Event reporting may be periodic or event-triggered.

-	Sensing Data Transfer Configuration: Introduces NRPPa messages to configure user plane data delivery. This includes the exchange of tunnel endpoint identifiers (TEIDs) and IP addresses for the N3 tunnel between the gNB and the UPF. These messages serve to prepare the sensing data path.

These new signaling messages are encapsulated in existing NGAP UE-associated or non-UE-associated NRPPa Transport messages, ensuring backward compatibility and leveraging the existing 5GC control plane architecture with minimal changes to network functions.

6.X.1.2	User Plane Extension of NRPPa

To enable scalable and efficient delivery of high-volume sensing data, this solution introduces a non-UE-associated user plane path over the N3 interface, reusing standard 5G user plane mechanisms:

-	Control Plane Signalling for Tunnel Setup: The NRPPa signaling triggers the setup of a GTP-U tunnel not associated with a UE. The SMF configures the gNB and UPF with necessary tunnel parameters (TEIDs, IP addresses), despite the absence of a UE context.

-	User Plane Tunnel Establishment: The gNB establishes a GTP-U tunnel over the N3 interface toward the UPF. The UPF subsequently forwards sensing data to the Sensing Function, either via a designated internal interface or through a dedicated DNN assigned to sensing services.

-	Encapsulation of Sensing Data: NRPPa-based sensing data is encapsulated in GTP-U packets following standard 5G user plane protocols, ensuring compatibility and extensibility.

-	Dedicated QoS Flow Configuration: A separate QoS Flow is provisioned to meet latency and reliability requirements for sensing traffic. This flow is isolated from regular UE traffic and may utilize existing 5QI parameters or be dynamically managed via PCC rules.

This user plane enhancement extends NRPPa’s utility beyond positioning, enabling high-performance sensing capability integration into the 5G architecture.

6.X.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

The proposed NRPPa enhancements support integrated operation across the control plane and user plane. Control plane signaling for sensing session management is handled using NRPPa messages encapsulated in NGAP, while the user plane transport of sensing data is conducted over the N3 interface using GTP-U, with appropriate QoS handling.

Figure 6.X.2-1: Integrated Control and User Plane Operation for NRPPa Sensing Extension

Figure 6.X.2-1 illustrates the signaling and data flow between Sensing Function and the Sensing Entity (e.g., gNB), as well as coordination with the SMF and UPF for tunnel establishment and data delivery. The procedure is divided into three main steps:

-	Sensing Session Setup: Sensing Function sends sensing service request to gNB using NRPPa encapsulated in NGAP. The request includes session configuration and sensing parameters.

-	Tunnel Setup: Sensing Function requests SMF to establish a non-UE-associated GTP-U tunnel. The SMF configures the UPF and gNB with the necessary tunnel parameters (e.g., TEIDs, IP addresses).

-	Sensing Data Delivery: The gNB transmits sensing data to the UPF via the N3 interface using the GTP-U tunnel. The UPF forwards the data to the Sensing Function, either through an internal path or via a dedicated DNN.

As an alternative to setting up a PDU session, a lightweight GTP-U based mechanism may be used, similar to steps 10 to 16 from PDU Session Establishment procedure in TS 23.502 [10]. In this method, the Sensing Function retrieves tunnel parameters (e.g., TEID, destination IP) from the SMF and relays them to the gNB via control plane signaling (e.g., NGAP). The gNB then transmits sensing data via GTP-U tunnel to the UPF, which is pre-configured (either statically or dynamically via SMF) to forward such non-UE associated traffic to the Sensing Function.

Editor's note: The detailed procedures, message formats, and information elements for the proposed NRPPa extensions will be defined after alignment with RAN3, based on the requirements of the sensing operations.

6.X.3	Impacts on Services, Entities and Interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

Sensing Function:

-	Initiates sensing sessions, collects data, and exchanges signaling with NG-RAN via NRPPa.

Sensing Entity (e.g., gNB):

-	Processes sensing data, exchanges capabilities, and forwards data via either NGAP (N2) or GTP-U (N3).

SMF and UPF:

-	Establish and manage non-UE GTP-U tunnels; support QoS for sensing data forwarding.

* * * End of Changes * * * *