3GPP TSG-WG SA2 Meeting #170	S2-2507150

Goteborg, SE, 25th Aug – 29th Aug, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	[WT#3, AI Training Service], Support of AI Training Service in 6GS

Document for:	Approval

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract: This document explores the opportunities for MNO to offer AI training service in 6GS. AI training service is the service provided by MNO to generate the training data used for AI model training, expose either the generated training data or the trained AI model to UE/third party based on service requirement. This document provides the corresponding justification and lists the related WT scope and KIs to study how to enable such new service.

- Summary

This contribution would like to introduce a new service offered by MNO for AI training. Following is the short summary about this paper regarding the justification and the potential key issues to study how to enable such new service.

Autonomous vehicles and embodied robots are able to perceive environmental data and perform action inference accordingly via the AI model. In order to perform appropriate action, such AI model should be trained with a large amount of high quality training data composed of a large amount of environmental data and the corresponding action data in different situations. However, it is difficult and expensive for third party to obtain such large amount of high quality training data. There are possibilities for MNO to develop new service for AI training taking its advantages to acquire the high quality training data effectively, which can not only help reduce the cost for third party to acquire such training data, but also can bring more revenue to MNO to help improve the third party owned AI model.

Currently, there are two possible options can be offered by MNO provide AI training services.

AI Training Service Option 1: MNO assists to generate training data for third party AI model

In this option, third party can negotiate with MNO regarding the service requirements e.g., training data structure, training data quality etc. Based on the service requirements, MNO gathers required data, generates the training data with MNO owned data generation AI model and exposes the training data to the third party. Detailed description can be found in clause 2.1.1.

AI Training Service Option 2: MNO assists to train third party owned AI model

In this option, third party can negotiate with MNO regarding the service requirements e.g., third party AI model structure, target accuracy, training method etc. Based on the service requirements, MNO gathers required data, trains the third party AI model and exposes the trained AI model to the third party. Detailed description can be found in clause 2.1.2.

Clause 2.1.3 provides gap analysis regarding why existing solution does not support the requirement of AI training service. In short, the AI/ML model training service is used by NWDAF inside the 5GC and the training data is standardized data instead of synthetic data.

Therefore, the potential key issues are listed in the second change of the pCR. The key issues aim to study i) how to generate the AI training data based on the service requirements, ii) how to expose the AI training data based on requirement, iii) how to enable the provisioning of AI model and parameters enabling the AI model training, and iv) how to expose the trained AI model based on requirement.

2. Introduction/Discussion

2.1 Justification to Enable AI Training Service in 6G

Nowadays, autonomous vehicles and embodied robots are becoming widely used. These terminals are equipped with an AI system having sensors and actuators to perceive surrounding environment and generate actions based on the perceived environmental data. This AI system is named as Physical AI. The AI model used for such action inference is called Physical AI model.

Taking autonomous vehicle as an example, in order to make decision accurately for the autonomous driving, the Physical AI model running on an autonomous vehicle should be trained/updated with the training data representing different situations/scenarios. One of the obstacles to the large-scale promotion of Physical AI is the lack of the training data composed of a large amount of environmental data and the corresponding action data, which results in insufficient performance of Physical AI. To tackle this problem, generative AI-based data generation methods, i.e. data generation AI models, have been widely adopted [3], which generate training data including object (e.g. vehicle, robot) environments and corresponding action trajectories aligned with real-world laws for Physical AI.

However, training data generation models still require a large amount of environmental training data containing objects collected from real-world scenarios. This may be difficult and high-cost for the third party to acquire such large amount of training data. For instance, the environmental data may be acquired manually by driving cars across all streets, road, highway etc and use the radar, camera to capture the 2D image information, which will be further used to generate the corresponding action data. The whole process is time-consuming and high labour cost. If the data generation model lacks such large amount of training data, the generated data quality would be degraded, resulting in bad performance of Physical AI training and leading wrong decision making when incident happens. Therefore, how to promote the Physical AI model accuracy with cost-effectively data generation model construction is a key point.

There are possibilities for MNO to develop new service for AI training taking its advantages to solve this issue effectively, which can not only help reduce the cost for third party to acquire high quality training data, but also can bring more revenue to MNO to help improve the third party owned AI model. Figure 1.1.1-1 shows the potentially possible new service for AI training offered by MNO.

Figure 1.1.1-1: AI Training Service

AI Training Service Option 1: MNO assists to generate training data for third party AI model

In this option, UE or third party can negotiate with MNO regarding the service requirements e.g., training data structure, target area, target time, response time, training data quality (e.g. consistency [3], the amount of training data) etc. According to the service requirements, MNO gathers required data based on the target area and/or time, generates the training data with MNO owned data generation AI model and exposes the training data to the UE or third party within the response time. In addition, the MNO can gather the required environmental data based on UE’s location and update the training data to the UE periodically.

AI Training Service Option 2: MNO assists to train third party AI model

In this option, UE or third party can negotiate with MNO regarding the service requirements e.g., third party owned AI model structure, target area, target time, response time, training method etc. According to the service requirements, MNO gathers required data based on the target area and/or time, trains the third party owned AI model and exposes the trained AI model to the UE or third party within the response time. In addition, the MNO can gather the required environmental data based on UE’s location and update the trained AI model to the UE periodically.

Clauses 2.1.1 and 2.1.2 describe the above two options respectively.

2.1.1	AI Training Service Option 1

MNO can own AI models (e.g., data generation model) to generate training data on demand. Taking the above autonomous driving use case as an example. MNO can own a World AI Model which can generate training data aligned with real-world laws for Physical AI model. Such training data can be a set of input data and expected output data (e.g. 3D spatial data [1], video with multiple frames [2], etc.).

Why does MNO have advantages to generate such training data?

In order to generate training data for Physical AI model training, the existing common approach is to collect data manually (e.g., manually driving cars equipped with sensors for driving data acquisition, or manually wearing sensor devices for embodied data acquisition), resulting in high labour expenses.

Following lists the advantages for MNO to generate the training data:

-	Cost-effective data acquisition: Since 6G system has wide-area deployed infrastructures, it is cost-effective for MNO to acquire data e.g., via sensing or positioning. Therefore, 6G system has possibility to acquire more 3D spatial data with lower costs, compared with approaches out of 3GPP scope e.g., manual data acquisition.

-	Abundant data categories: 6G system owns multiple types of data (e.g. positioning of objects, or other data from NFs). It has the possibility to provide more abundant prior information [4] (e.g. complement 3D spatial information) for data generation AI model to improve the quality of training data generation.

-	Some training data need to be generated inside the network: High-resolution 3D spatial data can improve the generated training data quality which may utilize the sensing and/positioning data. This data may not be exposed outside network. Therefore, it is more suitable for MNO to acquire real-world raw data, and generate synthetic training data via data generation AI model owned by MNO inside 6G network.

Based on the above analysis, the possible service flows for this option are illustrated in figure 1.1.1-2.

Figure 1.1.1-2: Illustration of AI training data generation and exposure

Therefore, 6G system will take advantages to provide such physical AI training data generation and exposure services according to the service requirements through data acquisition, MNO owned data generation model training and inference, which helps MNO gain profits while reducing data acquisition expenses for the third party.

In addition, if MNO has such capability to generate training data for third party AI model training, this capability can be further extended to assist to train the AI model supported by other NFs inside the network.

Observation 1: Operator owns generative AI models (e.g., World AI Model) which can be used to generate a high quality of training data via a large amount of data (e.g., sensing, positioning) to assist to train the UE or third party owned AI model.

Proposal 1: Enable the MNO to offer a new service to assist to generate training data and expose the training data to the third party based on service requirement via MNO owned data generation AI model.

2.1.2	AI Training Service Option 2

If the MNO has the capability to generate training data for third party AI model to improve its accuracy, this capability can be easily extended to support the above-mentioned option 2 where MNO can assist to train the third party AI model (e.g., Physical AI model) directly.

In this option 2, third party needs to provision its AI model inside 6G network to perform model training. After model training is completed, the 6G network exposes the trained AI model, rather than the training data, to the third party. Compared with the option 1 illustrated in clause 2.1.1, option 2 is more suitable for scenarios where the 6G network owned AI training data which cannot be exposed to third parties (e.g., if mobility traces or information of UEs who are not third party’s customers can be extracted/derived from the AI training data, the AI training data shall not be exposed to the third party). The 6G network may also optimize the AI model training (e.g., to improve efficiency of model training, use split learning method to train the AI model in multiple network entities) and send/update training performance metrics (e.g., achieved training accuracy) to the third party.

Figure 1.1.2-1: Illustration of Third Party AI Model Training

Observation 2: Operator owns a large amount of training data (e.g., mobility data, sensing, positioning) which can be used to train the AI model (owned by third party) directly inside 6G network.

Proposal 2: Enable the MNO to offer a new service to allow the UE or third party to provision its own AI model to 6G network and the 6G network can assist to train the AI model and expose the trained AI model to the UE or third party based on service requirement.

2.1.3	Gap Analysis & Potential Technical Impacts

Currently, some existing techniques are related with AI model training such as the discussion in AI/ML and the mechanism and procedure specified in TS 23.288 [5]. Following is the analysis why existing techniques cannot meet the requirements mentioned above.

- The consumer of AI/ML model training service is the NWDAF inside the 5GC and the purpose for training the AI/ML model is limited to network data analysis.

- The AI/ML model training defined in TS 23.288 [5] focuses on the standardized data, while the required training data mentioned in the above target scenario is synthetic data.

In short, the consumers of NWDAF enabled AI/ML model management are limited to NWDAF. Neither other NFs in the 5GS nor the 3rd party AFs can use this service. In addition, the missed functionalities for AI training service in existing architecture include:

-	No support of generating and exposing AI training related data (either AI training data or the trained AI model itself) data for AF/NFs/UEs.

-	No support of service metrics (e.g., data quality assurance mechanism) for the exposure AI training related data.

-	No support of framework to enable AI model exposure to AF/UEs.

2.1.4	Reference

[1] Occworld: Learning a 3d occupancy world model for autonomous driving, Tsinghua University, ECCV 2024.

[2] CogVideoX: Text-to-Video Diffusion Models with An Expert Transformer, Tsinghua University, Zhipu AI, ICLR 2025.

[3] Cosmos World Foundation Model Platform for Physical AI, https://arxiv.org/abs/2501.03575, 2025.

[4] M. I. Ismail, A. M. Shaheen, M. M. Fouda and A. S. Alwakeel, "RIS-Assisted Integrated Sensing and Communication Systems: Joint Reflection and Beamforming Design," in IEEE Open Journal of the Communications Society, vol. 5, pp. 908-927, 2024

[5] 3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.801-01.

# * * * * First change * * * *

Annex A.3. WT#3 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

-	Define, support and enable AI training service in 6G architecture.

-	Support the generation of AI training data based on service requirement

-	Support the provisioning of AI model and parameters from UE/third party/MNO for AI model training

-	Support the AI model training based on service requirement.

-	Support the exposure of AI training data or trained AI model based on service requirement.

Annex A.1. WT#1 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

-	NAS: Support interaction between UE and NF supporting AI training services.

-	Exposure: Support exposure framework for training data or trained AI model exposure

-  New Interface: Support interaction among NFs for data collection and training data generation

Annex A.4. WT#4 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

-	Support the exposure of Sensing information inside 6GC to generate training data

Annex A.5. WT#5 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

-	If data framework is proposed, support the data collection via data framework for training data generation.

Annex A.6. WT#6 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

-	If computing service is proposed, the computing services need to provide computing resource for AI training service.

# * * * * Second change * * * *

5.X. Key Issue #X:  Support of AI training service in 6G network

This key issue proposes to study how an operator can provide AI training service. The support of AI training service may consist in the operator assisting the AI training by generating and providing training data or in also supporting the actual training of the third party AI model and expose the trained AI model to the user. The first step is to collect data and generate the training data according to a set of agreed criteria that depends on the AI model to be trained. If the operator is performing or assisting the training of the third party AI model, then besides the generation of the data the operator also takes care of exposing the generated training data to the AI model and to expose the trained AI model to the third party.

The solutions for this key issue should address the following aspects

-	How to acquire the data used to generate AI training data

NOTE:	Coordination with WT#5 may be needed.

-	How to generate AI training data that meet the requirements of the AI model to be trained. The non-exhaustive list to take into consideration include:

-	input and output data structure

-	time and location of the data collected

-	quality of the data

-	How to expose the AI training data taking into consideration:

-	location where the AI training data to be exposed

-	time when the AI training data to be exposed.

-	How to enable the provisioning of the AI model and parameters (e.g., AI model training method, target accuracy) from a third party, UE or MNO for AI model training.

-	How to perform and optimize the AI model training based on the service requirement

-	How to expose the trained AI model taking into consideration:

-	location where the trained AI model to be exposed

-	time when the trained AI model to be exposed

-	target accuracy that trained AI model has reached.

# * * * * End of changes * * * *