SA WG2 Meeting #170	S2-2507447

25 – 29 Aug, 2025, Goteborg , SE

Source:	Samsung, AT&T

Title:	[WT#2] 6G Migration Paths

Document for:	Agreement

Agenda Item:	20.6.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution lists all possible 6G migration paths and considers feasible paths.

1. Justifications

The existing description of WT#2 as in the agreed SID titled as “Study on Architecture for 6G System” [SP-250806] is described as follows:

WT#2: Study migration and interworking, including

-	How to support migration to 6GS

-	How to support interworking with 5GS

-	Whether and how to support interworking with EPS

-	How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

NOTE 2:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 3:  The detailed migration study scope will be coordinated and aligned with RAN.

Observation 1: The 1st bullet, Migration Path / Deployment Scenarios is the most important and the other bullets are dependent on the 1st bullet. Hence, Migration Paths / Deployment Scenarios should be discussed first for WT#2.

Observation 2: 5G Rel-15 introduced too many network architecture options. 5 options (2, 3, 4, 5, 7) are standardized, but only 2 options (2, 3) are deployed. SA deployments are delayed due to the existing NSA operation.

Proposal:  In 6G, it is proposed to consider Standalone architecture only from day one.

Migration/Architecture Options for 6G include MRSS (5G+6G spectrum sharing) and 6G-6G carrier aggregation, as described in the following figure.

Observation 3: Whether MRSS is applied or not, the interfaces in CN sides or the interfaces in UE sides are the same, as shown in the following figures: (a) MRSS is applied, (b) MRSS is not applied.

Observation 4: The candidate migration paths are like the following. Considering bullet 3 in WT#2, whether and how to support interworking with EPS, Path 2, Path 3 and Path 4 need more discussions.

Proposal 2: It is proposed to add Path 1 and others to the Annex of the TR 23.801-01 as candidate migration paths.

**** First Change ****

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

MRSS	Multi-RAT Spectrum Sharing

**** Second Change (all new) ****

Annex A.X. Work Task #2 6G Migration Paths

This clause describes potential migration paths to 6G standalone system.

A.X.1	6G Migration Path 1, from 5G SA into 6G SA

This migration describes the case when 5G standalone deployment migrates into 6G standalone via 5G standalone and 6G standalone interworking with or without MRSS.

6G Migration Path 1 includes the following steps as described in Figure A.X.1-1.

Figure A.X.1-1: 6G Migration Path 1, from 5G SA into 6G SA

1.	Current deployment: 5G SA

2.	Intermediate deployment: 5G SA and 6G SA coexist with or without MRSS. 6G RAN spectrum can be re-farmed, a new band with MRSS, or new FR3 introduction, and can be used with MRSS. Deployment examples with or without MRSS are described in Figure A.X.1-2.

NOTE: In the left figure shows an example deployment using MRSS, the logical representation of the architecture is the same as right figure without MRSS.

3.	Final deployment: 6G SA

Figure A.X.1-2: 5G SA and 6G SA coexist (a) with MRSS or (b) without MRSS

A.X.2	6G Migration Path 2, from 5G NSA into 6G SA

This migration describes the case when 5G non-standalone deployment migrates into 6G standalone via 5G non-standalone and 6G standalone interworking with or without MRSS.

6G Migration Path 1 includes the following steps as described in Figure A.X.2-1.

Figure A.X.2-1: 6G Migration Path 2, from 5G NSA into 6G SA

1.	Current deployment: 5G NSA

2.	Intermediate deployment: 5G NSA and 6G SA coexist with or without MRSS. Radio spectrum used by 6G RAN can be re-farmed, a new band with MRSS, or new FR3 introduction, and can be used with MRSS. The deployment example is described in Figure A.X.2-2.

3.	Final deployment: 6G SA

Figure A.X.2-2: 5G NSA and 6G SA coexist (a) with MRSS or (b) without MRSS

This migration path requires 6GS and EPS interworking.

Editor’s note: Further discussions are needed regarding whether to support interworking with EPS.

A.X.3	6G Migration Path 3, from 5G NSA into 6G SA via 5G SA

This migration describes the case when 5G non-standalone deployment migrates into 6G standalone via 5G standalone and via 5G standalone and 6G standalone interworking with or without MRSS.

6G Migration Path 3 includes the following steps as described in Figure A.X.3-1.

Figure A.X.3-1: 6G Migration Path 3, from 5G NSA into 6G SA via 5G SA

1.	Current deployment: 5G NSA

2.	Intermediate deployment: 5G SA

3.	Intermediate deployment: 5G SA and 6G SA coexist with or without MRSS. Spectrum used by 6G RAN can be re-farmed, a new band with MRSS, or new FR3 introduction, and can be used with MRSS. The deployment example is the same as Figure A.X.1-2.

4.	Final deployment: 6G SA

NOTE 1: This migration path assumes all the 5G non-standalone deployments need to migrate into 5G standalone deployments first, before introducing 6G standalone deployments.

NOTE 2: This migration path does not assume Path 2 and Path 4 are needed.

Editor’s note: Further discussions are needed regarding whether to support interworking with EPS.

A.X.4	6G Migration Path 4, from 5G NSA and 5G SA into 6G SA

This migration describes the case when 5G non-standalone and 5G standalone deployments migrate into 6G standalone via 5G non-standalone and 5G standalone and 6G standalone interworking with or without MRSS.

6G Migration Path 4 includes the following steps as described in Figure A.X.4-1.

Figure A.X.4-1: 6G Migration Path 4, from 5G NSA and 5G SA into 6G SA

1.	Current deployment: 5G NSA and 5G SA

2.	5G NSA and 5G SA and 6G SA coexist with or without MRSS. 6G RAN spectrum can be re-farmed, a new band with MRSS, or new FR3 introduction, and can be used with MRSS. The deployment example is the same as Figure A.X.1-2 and Figure A.X.2-2.

3.	Final deployment: 6G SA

This migration path requires 6GS and EPS interworking.

Editor’s note: Further discussions are needed regarding whether to support interworking with EPS.

**** Third Change (all new) ****

5.X. Key Issue #X: Support of 6G migration and interworking

Key Issue #X for WT#2 Study migration and interworking, including:

- How to support migration to 6GS, including selecting potential migration paths to 6G.

- How to support interworking with 5GS.

- Whether and how to support interworking with EPS

- How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS, depending on bullets b and c.

**** End of Changes ****