SA WG2 Meeting #171	S2-2508353

13 – 17 October, 2025, Wuhan, CN

Source:	vivo

Title:	[WT#1.2, Localized Service Access] Support of Localized Service Access offered by PLMN in 6G

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes to update the WT scope of supporting localized service based on the postponed version (S2-2508125) from pen holder.

Introduction

In Non-public network related features defined in 5G, the UE needs to support at least the following enhancements in order to access the SNPN or PNI- NPN:

(1) SNPN:

According to the clause 5.30.2.3 of TS 23.501, an SNPN-enabled UE is configured with the following information for each subscribed SNPN:

-	PLMN ID and NID of the subscribed SNPN

-	Subscription identifier (SUPI) and credentials for the subscribed SNPN;

-  Some other optional information as defined in this clause.

(2) PNI-NPN:

According to the clause 5.30.3.3 of TS 23.501, the UE needs to support the following configuration for the access to PNI-NPN:

To use CAG, the UE, that supports CAG as indicated as part of the UE 5GMM Core Network Capability, may be pre-configured or (re)configured with the following CAG information, included in the subscription as part of the Mobility Restrictions:

-	 an Allowed CAG list i.e. a list of CAG Identifiers the UE is allowed to access; and

-	each entry of the Allowed CAG list may be associated with time validity information containing one or more time periods;

In general, considering these enhancements on the UE side, the uses need to upgrade the terminals in order to access the SNPN or PNI-NPN for local service access. This is a problem in the real business case, since the cost for the deployment of SNPN or PNI-NPN is relatively high.

In 6G, in order to better support the deployment of localized service access, it’s better to have less impacts on the UE side.

Proposal 1: To support the localized service access in 6G system, the related feature should have less impacts on the UE side.

Considering that the network proving the localized service access should be provided by the operators and under the MNO control, it is proposed to clarify that the network providing localized service access should be within the PLMN network.

Proposal 2:  It needs to be clarified that the network providing localized service access should be within the PLMN network.

**** First Change ****

Annex A.X. WT#1.2 Scope for localized service access

The scope of localized service access is to study the overall architecture design and enablers for localized service access within the PLMN network in 6G, including:

NOTE 1: Support for localized service as defined in 5G is the starting point for the discussion on support of localized service access in 6G.

- authorization and authentication of UE for localized service access

- interface and interaction procedure between networks providing non-localized service and networks providing localized service, or between networks providing localized service:

- Whether and how to support UE mobility between networks providing non-localized service and network providing localized services, or between networks providing localized service

- Whether and how to support service continuity for localized service, including access to localized service after leaving the original network providing localized service.

NOTE 2: For the sake of business success, it is preferred to have less UE impacts in this WT.

**** End of Changes ****