3GPP TSG-SA WG2 Meeting #171	S2-2508391

Wuhan, China, October 13 – 17, 2025										(revision of S2-2507677)

Source:	CATT

Title:	KI#3 interim agreements

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes interim agreements for KI#3.

1. Introduction

Based on S2-2507677 which has been discussed in SA2#170 meeting, the interim agreements for KI#3 “NF (re)selection and/or UP path adjustment based on energy related information” are proposed.

Compared to UPF (re)selection based on the NF profile, which only takes into account of the energy related information of one UPF, UP path adjustment considering DNAI (re)selection takes into account of the overall energy related information (e.g. energy consumption, energy efficiency, and/or renewable energy consumption ratio) of all the UPFs along the UP path for transmitting the service data flows towards the DNAI, so that the most suitable DNAI from energy point of view can be selected for the specific service(s). This can improve energy related performance for supporting services/applications, e.g. when multiple DNAIs can be selected in local networks for the same service(s)/application(s). So it is proposed to add DNAI (re)selection for UP path adjustment taking into account of energy related information in agreed principles.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

# * * * * First change (all new text) * * * *

7.1.3	Agreed Principles for KI#3

The following principles are agreed for KI#3:

-	For NF (re)selection, the NF profile is enhanced to include the energy related information in the table below.

| Information | Description |
| --- | --- |
| EnergySaving /Not EnergySaving State | Value of the energy state as specified in TS 28.310 [9], e.g. notEnergySaving state, Energy Saving state. (NOTE 1) |
| Schedule of EnergySaving/not EnergySaving | The schedule of the EnergySaving/not EnergySaving states, includes the value of EnergySaving or not EnergySaving state which applies at each time interval in the schedule. If this is present, the EnergySaving /Not EnergySaving State attribute is not present. |
| Energy Priority Information | Value of Energy priority relative to other NFs of the same type considering the energy characteristics of the NF. |
| Schedule of Energy Priority Information | The Schedule of the Energy Priority includes the value of Energy Priority information which applies at each time interval in the schedule. If this is present, the Energy Priority information attribute is not present |
| NOTE 1: EnergySaving/not EnergySaving refers to states of a network function which are defined in TS 28.310 [9]. |  |

-	UP path(s) adjustment for PDU Session(s) may be triggered via DNAI (re)selection and/or UPF (re)selection, taking into account energy related information, e.g. the overall energy consumption and/or energy efficiency of the UP path(s) for transmitting the service data flow, and energy related information of the UPF as described above.

NOTE:	Renewable energy consumption ratio aspect can be considered if SA5 indicates support of collection/calculation of such information.

7.2.3	Topics for further consideration for KI#3

The following principles are for further discussions for KI#3:

-	Whether the NF profile needs to include explicit indication of usage of renewable energy or using Energy Priority to implicitly take this into account is sufficient.

# * * * * End of change * * * *