SA WG2 Meeting #171	S2-2508446

13 – 17 Oct, 2025, Wuhan, China	(revision of S2-2508132)

Source:	China Mobile

Title:	[WT#5, Data Framework] Update for WT scope and KIs for 6G data framework

Document for:	Approval

Agenda Item:	20.6.5

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This proposal updates the WT scope and KIs for 6G data framework.

1. Discussion

Globally, operators have invested massive amounts in 5G infrastructure. The 5G system generates vast amounts of data, including AI data, sensing data, user data, network operational data and so on. The 6G system may need to access this data for analysis and network optimization. In addition, the AI service and sensing service in 6G will be provided based on the  data framework. During handover, 5G and 6G systems need to interact to ensure the seamless continuity of data services. Thus, how it achieves interoperability with 5G is a critical issue that requires in-depth research in 6G, which needs to be added in WT#5.

**** First Change ****

Annex A.X. WT#5 Scope

The following is WT#5 in the SA2 6G SID:

WT#5: Study data framework for all aspects related to efficient and scalable data handling including, for example, data collection, distribution, processing, storage, data access and data exposure, with consideration of access control/user consent and privacy where relevant. The example of data may include data for AI and Sensing. This WT can also study any potential enhancements on system and procedure needed for user consent framework.

NOTE 6:	The work split with SA3, SA5 and RAN WGs will require TSG coordination

Data categories can be handled/supported by 6G data framework studied in SA2:

  - Data from Core network and 3rd AF

Option 1:

Editor’s Note: whether the following data can also be handled/supported by 6G data framework in SA2 will be revisited:

  - Data from UE (e.g. for Sensing, UE-sided model training or NF-sided model training, etc.)

  - Data from RAN (e.g. for Sensing, two-sided model training or NF-sided model training, etc.)

  - Data/analytics from OAM (e.g. invoking MDAF service by CN for NF-sided model training)

Option 2:

Editor’s Note: what other data can also be handled/supported by 6G data framework in SA2 will be revisited.

The scope of the WT includes the following aspects:

- Design end to end 6G system architecture for data framework, e.g. defining new NF(s), new data plane/enhancing CP/UP and interfaces, to support efficient and scalable data handling functionalities, e.g.:

- data collection

- data transfer and distribution

- data processing e.g. labelling data for origin, purpose, context and quality/validity as well as handling metadata such as type and structure of data parameters produced by NFs

- data storage within 6G system

- data access

- data exposure within 6G system and to outside of 6G system

NOTE 1:	Coordination with SA5 may be required to determine whether and how to align with the data handling mechanisms supported in the management domain (i.e., OAM) and charging aspects of data exposure.

NOTE 2:	Coordination with RAN WG is required for data collection from or via RAN.

NOTE 3:  Consideration on MNO’s requirements of visibility and controllability for standardized data during data handling processes is required. And data handling processes can be service-differentiated.

- Study from SA2 architectural and system level on user consent, privacy, security and data governance to support the 6G data framework.

NOTE 4:	Coordination with SA3 is required for user consent, privacy, security and data governance related topics.

- Study how to support quality of data which is processed and provided based on the requirements from data requester.

- Study whether and how to exchange data and ensure data service continuity between 5G and 6G networks.

**** Second Change ****

5	Key Issues

Editor's note:	This clause identifies key issues to be addressed.

5.X	Key Issue #X: Data Framework

This key issue aims to provide solutions for data handling with data framework for 6G.

This key issue will investigate the following aspects:

-	How to support efficient and scalable data handling functionalities  in end to end 6G system architecture for data framework, e.g. defining new NF(s), new data plane/enhancing CP/UP and interfaces:

‐data collection

‐data transfer and distribution

‐data processing e.g. labelling data for origin, purpose, context and quality/validity as well as handling metadata such as type and structure of data parameters produced by NFs

‐data storage within 6G system

‐data access

‐data exposure within 6G system and to outside of 6G system

NOTE 1:	Coordination with SA5 may be required to determine whether and how to align with the data handling mechanisms supported in the management domain (i.e., OAM) and charging aspects of data exposure.

NOTE 2:	Coordination with RAN WG is required for data collection from or via RAN.

NOTE 3:  Consideration on MNO’s requirements of visibility and controllability for standardized data during data handling processes is required. And data handling processes can be service-differentiated.

- How to ensure user consent, privacy, security and data governance in the 6G data framework.

NOTE 4:	Coordination with SA3 is required for user consent, privacy, security and data governance related topics.

- How to support quality of data which is processed and provided based on the requirements from data requester.

- Whether and how to exchange data and ensure data service continuity between 5G and 6G networks.

**** End of Changes ****