3GPP TSG-WG SA2 Meeting #171	S2-2508575

20 – 24 January 2025, Wuhan, China

Title:	[DRAFT] LS Reply on N6 measurement exposure

Response to:	LS on N6 measurement exposure (S6-253741)

Release:	Rel-19

Work Item:	eEDGE_5GC_Ph3

Source:	[Huawei to be] SA2

To:	SA6

Cc:

Contact Person:

Name:	Haiyang, Sun

E-mail Address:	sunhaiyang3@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None.

1. Overall Description:

SA2 thanks SA6 for their LS on N6 measurement exposure in S6-253741 and would like to provide the following feedback.

Q1: Based on the discussion above, SA6 would like to ask whether SA2 could consider providing support for exposure of N6 measurement information?

A1:  When SA2 do the eEDGE_5GC_Ph3 work in Rel-19, to enhance the EAS and local UPF (re)selection the N6 delay, which is not measured before, is taken into consideration. To support the N6 delay measurement, the AF provides the remote measurement end information within the EDI IE as described in TS23.501. Per the received AS measurement information, i.e. remote end point of AS, SMF determine which UPF need measure the transmission delay between UPF and remote end point of AS. Per this measurement information, the SMF can determine which UPF is the candidate local UPF. This is reflected as ECS option information when the SMF manipulate the ECS information within the DNS query message. So from SA2 view, the existing N6 delay measurement is under the assumption that this information is customized for local UPF selection, e.g. only specific UPF are selected for this measurement. It is not be considered as general case for any N6 delay measurement. From that dedicated UC, SA2 does not see any need to expose the N6 measurement information to AF.

The N6 delay measurement information is described as (Pair of (UPFx, Remote measurement end of AS), transmission delay). It indeed implicitly reflects the network topology information, e.g. UPF information and its capability. Hence this measurement information is not suitable to be exposed outside, i.e., only for internal usage, e.g. SMF to select local UPF.

Also it need be kept in the mind that the N6 delay measurement is per node level. It is not linked to one specific application flow. However, the normal QoS monitoring in the 5GC network is measured per application flow and may even in different measurement frequency. Hence two parts measurement information are not suitable to be combined directly as e2e round-trip latency information.

Per above consideration SA2 have decided before that this measurement information is only used for local UPF selection and only be aware by the SMF. This is to avoid unnecessary information leakage.

SA2 have concern to expose this information outside SMF. The reasons are as following:

- What is the expected Use Case to use this information? There is no related UC described in the incoming LS on the purpose to use this information. So whether this information exposed outside SMF can fulfil the required scenario is not clear.

- How to avoid the network topology information, e.g. UPF information and its capability, are exposed outside?

Unless SA6 clarifies further on why the AF need this information rather than SMF, SA2 does not plan to expose this information to the AF.

2. Actions:

To SA6:

ACTION: 	SA2 kindly requests SA6 group to take the above responses into account.

3. Date of Next TSG SA WG2 Meetings:

SA2 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-S2.htm