3GPP TSG-SA WG2 Meeting #171	S2-2508679

Wuhan, China, 13 – 17 October 2025

Source:	Ericsson

Title:	KI#1, New Solution: Capability and Identification of the SMS delivery to Emergency Response Centre.

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract of the contribution: This contribution provides a solution for SMS over NAS delivery to Emergency Response Centre KI#1.

# 1.	 Proposal

It is proposed to update TR 23.700-65 with below changes:

* * * Start of change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |
| #x | x |  |  |
| - |  |  |  |
| - |  |  |  |
| - |  |  |  |
| - |  |  |  |

* * * Next change (all new) * * * *

## 6.X	Solution #X: SMS over NAS delivery to Emergency Response Centre by SMS SC

### 6.X.0	High level principles

This solution aims to address KI#1 with the following key principles:

-	The serving network provides indication of support for SMS delivery to Emergency Response Centre and the emergency number list to UE during UE registration/attach and IMS registration (in case of non-roaming), if SMS delivery to Emergency Response Centre is supported by the serving network based on local policy.

-	The SMS delivery to serving network SMS service centre is routed by MME/SMSF and IMS/IP-SM-GW, based on the Relay layer RP-DA which include SMS service centre address/emergency call number, to the serving network SMS service centre.

-	The SMS service centre is responsible to identify different type of emergency service based on the provided emergency number and route the SMS to corresponding Emergency Response Centre.

### 6.X.1	Description

This solution addresses KI#1 with the principles as described in clause 6.x.0 above.

### 6.X.2	Procedures

During the registration/attach and IMS registration procedure (in case of non-roaming), the AMF/MME/S-CSCF provides the indication of the supporting SMS delivery to Emergency Response Centre in the serving network and the emergency number list based on local policy.

For the SMS delivery to Emergency Response Centre, it’s further described in solutions for KI#2/KI#3.

#### 6.x.2.1	Registration Procedures in 5GS

##### 6.x.2.1.1	Registration procedures for SMS over NAS

The following procedures are based on clause 4.13.3 in TS 23.502 [3].

Figure 4.13.3.1-1: Registration procedure supporting SMS over NAS

1.	During Registration procedure in 5GS defined in Figure 4.2.2.2.2-1, to enable SMS over NAS transporting, the UE includes an "SMS supported" indication in Registration Request in step 1-3 indicating the UE's capability for SMS over NAS transport. The "SMS supported" indication indicates whether the UE supports SMS delivery over NAS.

2.	Step 4 to step 14 of the Registration procedure in Figure 4.2.2.2.2-1 are performed. The AMF may retrieve the SMS Subscription data and UE Context in SMSF data using Nudm_SDM_Get. This requires that UDM may retrieve this information from UDR by Nudr_DM_Query. The UDM includes the SMSF information in the Nudm_SDM_Get response message if the stored SMSF belongs to the same PLMN of the AMF. After a successful response is received and if SMS service is allowed, the AMF subscribes to be notified using Nudm_SDM_Subscribe when the SMS Subscription data is modified and UDM may subscribe to UDR by Nudr_DM_Subscribe. For an MPS-subscribed UE, the subscription data may include the MPS priority for Messaging indication. If the AMF receives the MPS for Messaging indication from UDM, the AMF stores the MPS for Messaging indication in the UE context.

The AMF can also receive UE context information containing SMSF Information from old AMF. When AMF re-allocation happens during the Registration procedure, the old AMF transfers SMSF Information to the new AMF as part of UE context in step 5 of Figure 4.2.2.2.2-1.

NOTE 1:	The AMF can, instead of the Nudm_SDM_Get service operation, use the Nudm_SDM_Subscribe service operation with an Immediate Report Indication that triggers the UDM to immediately return the subscribed data if the corresponding feature is supported by both the AMF and the UDM.

3.	If the "SMS supported" indication is included in the Registration Request, the AMF checks in the SMS Subscription data that was received in step 2 whether the SMS service is allowed to the UE. If SMS service is allowed and the UE context received in step 2 includes an available SMSF of the serving PLMN, the AMF activates this SMSF Address and continues the registration procedure. If SMS service is allowed but an SMSF of the serving PLMN was not received in step 2, the AMF discovers and selects an SMSF to serve the UE as described in clause 6.3.10 of TS 23.501 [2]. If there are multiple SMSF instances in the network and the SMS delivery to Emergency Response Centre is only supported by certain SMSF instances, the AMF selects the SMSF instances that supports the SMS delivery to Emergency Response Centre (e.g. based on the SMSF profile that indicates the support of SMS delivery to Emergency Response Centre).

4.	Step 15 to step 20 of the Registration procedure in Figure 4.2.2.2.2-1 are performed.

5.	The AMF invokes Nsmsf_SMService_Activate service operation from the SMSF. The invocation includes AMF address, Access Type, RAT Type, Trace Requirements, GPSI (if available) and SUPI. AMF uses the SMSF Information derived from step 3. Trace Requirements is provided if it has been received by AMF as part of subscription data. When the AMF determines the UE has the MPS for Messaging indication in UE context as specified in step 2, the AMF includes a Message Priority header with a value appropriate for MPS to indicate priority treatment.

6.	The SMSF discovers a UDM as described in clause 6.3.8 of TS 23.501 [2].

7a.	If the UE context for the current Access Type already exists in the SMSF, the SMSF shall replace the old AMF address with the new AMF address.

Otherwise, the SMSF considers this a Registration request from a new Access Type and the SMSF registers with the UDM using Nudm_UECM_Registration with Access Type. As a result, the UDM stores the following information: SUPI, SMSF identity, SMSF address, Access Type(s) in UE Context in SMSF data. The UDM may further store SMSF Information in UDR by Nudr_DM_Update (SUPI, Subscription Data, UE Context in SMSF data).

If the Nsmsf_SMService_Activate request contains two Access Types and one of them is already registered in the SMSF, the SMSF shall replace the old AMF address with the new AMF address for that Access Type. The SMSF shall then register the other Access Type with the UDM using Nudm_UECM_Registration request.

7b-7c	SMSF retrieves SMS Management Subscription data (e.g. SMS teleservice, SMS barring list) using Nudm_SDM_Get and this requires that UDM may get this information from UDR by Nudr_DM_Query (SUPI, Subscription Data, SMS Management Subscription data). After a successful response is received, the SMSF subscribes to be notified using Nudm_SDM_Subscribe when the SMS Management Subscription data is modified and UDM may subscribe to notifications from UDR by Nudr_DM_Subscribe.

SMSF also creates a UE context to store the SMS subscription information and the AMF address that is serving this UE.

NOTE 2:	The SMSF can, instead of the Nudm_SDM_Get service operation, use the Nudm_SDM_Subscribe service operation with an Immediate Report Indication that triggers the UDM to immediately return the subscribed data if the corresponding feature is supported by both the SMSF and the UDM.

8.	The SMSF responds back to the AMF with Nsmsf_SMService_Activate service operation response message. The AMF stores the SMSF Information received as part of the UE context.

9.	The AMF includes the "SMS allowed" indication and “SMS over NAS delivery to Emergency Response Centre supported/allowed” based on local policy (e.g. the function is supported in serving AMF/SMSF) to the UE in the Registration Accept message of step 21 of Figure 4.2.2.2.2-1 only after step 8 in which the AMF has received a positive indication from the selected SMSF.

The "SMS allowed" indication in the Registration Accept message indicates to the UE whether the network allows the SMS message delivery over NAS. The “SMS over NAS delivery to Emergency Response Centre allowed/supported” indicates to UE whether the network allows the SMS over NAS delivery to Emergency Response Centre. The AMF may also include the emergency number list to UE if the “SMS over NAS delivery to Emergency Response Centre allowed” is set.

##### 6.x.2.1.2	Attach procedure in EPS

The SMS over NAS delivery to Emergency Response Centre is similar to 5GS procedure with the change that the MME takes to role of AMF/SMSF.

#### 6.x.2.2	IMS Registration procedure

The following procedures are based on TS 23.204 [8]. This procedure is considered when UE is in non-roaming.

Figure 6.1: Registration procedure

1)	The UE establishes IP connection.

2)	At any time after the establishment of the IP connection, the UE registers at the S-CSCF according to the IMS registration procedures. Based on local policy (e.g. the function is supported in IMS/IP-SM-GW), the S-CSCF also indicates the support of SMS over IP delivery to Emergency Response Centre by including e.g. “+g.3gpp.smsip-delivery to Emergency Response Centre” parameter in the Feature-Caps field header in 200 (OK) response.  The “+g.3gpp.smsip-delivery to Emergency Response Centre” parameter indicates to UE whether the network allows the SMS over IP delivery to Emergency Response Centre.

NOTE 1:	For simplicity, not all messages between UE and S-CSCF and between S-CSCF and HSS are shown in detail.

3)	S-CSCF checks the initial filter criteria retrieved from the HSS during the IMS registration procedure.

4)	After successful IMS registration and based on the retrieved initial filter criteria, the S-CSCF informs the IP-SM-GW (AS) about the registration of the user. IMSI is informed to the IP-SM-GW (AS) when there is no MSISDN in the UE's IMS subscription profile.

5)	The IP-SM-GW (AS) returns OK to the S-CSCF.

6)	The IP-SM-GW (AS) sends IP-SM-GW Register Req to the HSS.

7)	The HSS stores the received IP-SM-GW address if necessary or for MT-SMS without MSISDN (see clause 6.0a.2), uses it as an indication that the UE is available to be accessed via the IMS to trigger an Alert service centre message if the message waiting flag is set, and responses to the IP-SM-GW (AS) with IP-SM-GW Register Res. IP-SM-GW gets the IMPU (SIP URI) for SMS delivery without TEL-URI from registration event package.

NOTE 2:	IP-SM-GW Register Res can include the SC address to be used for this user in the subscriber data (see also clause 6.7).

NOTE 3:	If the IP-SM-GW address stored in the HSS via registration procedure is not the same as the preconfigured IP-SM-GW address (if any), then the short message delivery attempted during registration can be unnecessarily delayed.

8)	After successful registration of the IP-SM-GW address at the HSS the HSS checks whether message waiting data are stored and alerts all SCs using procedures described in TS 23.040 [2] (see also clause 6.5b).

Editor's note:	roaming scenario is FFS.

### 6.X.3	Impacts to Services, Entities and Interfaces

UE:

-	Handling “SMS over NAS delivery to Emergency Response Centre allowed” indication and emergency number list in Registration/Attach Accept messages.

-	Support the handling of SMS over IP delivery to Emergency Response Centre during IMS registration procedure, e.g., support of “+g.3gpp.smsip-delivery to Emergency Response Centre” field header handling.

-	Formulates the RP-DA in the RP-DATA with serving PLMN SMS service centre/emergency number provided by the AMF.

AMF/MME:

-	Support the indication and in Registration/Attach accept with “SMS over NAS delivery to Emergency Response Centre allowed” indication and emergency number list.

S-CSCF:

-	Support the “+g.3gpp.smsip-delivery to Emergency Response Centre” field header handling during IMS registration procedure.

SMSF/MME/IP-SM-GW:

-	Support of SMS delivery to SMS service centre of the serving PLMN based on SMS service centre address/emergency number.

* * * End of change * * * *