3GPP TSG-WG2 Meeting #171	S2-2508856

October 13 – 17, 2025, Wuhan, China		(revision of S2-250xxxx)

Source:	Ericsson

Title:	Interim agreement for KI#3

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution provides agreed principles and topics for further consideration for KI#3

# 1. Discussion

The table below enumerates all solutions identified as having an impact on KI#3. Registration and Discovery specifies the method by which the service is discovered (e.g., via the NRF or other mechanisms). The Selection Criteria defines the parameters used to select SF or SE.

| # | SCF/SF/SeMF/SnCF Discovery |  | SE Discovery |  | Discussion |
| --- | --- | --- | --- | --- | --- |
|  | Registration and Discovery | Selection Criteria | Registration and Discovery | Selection Criteria |  |
| 1 | No procedures are specified NEF performs discovery | Not specified | No procedures are specified | Not specified |  |
| 3 | No procedures are specified NEF performs discovery. | Not specified | No procedures are specified SF performs discovery | sensing service requirement, sensing capability | No details or examples on service requirement and sensing capability. |
| 6 | Not specified due to the use of Gateway sensing NF to select sensing NF. Details about how Gateway sensing function select a (distributed) sensing NF are not specified. | Each sensing NF can be configured to serve several sensing service areas. Sensing service area can be mapped to a list of cells, tracking area, etc. The gateway sensing NF selects a (distributed) sensing NF which serves the target sensing service area. | SE registers capability to the Gateway sensing NF. Gateway sensing NF selects candidate list of sensing entities and includes them in the sensing request to a sensing NF serving a target sensing service area. The sensing NF performs SE selection. | sensing capability at the sensing NF, location supported sensing service area and supported sensing mode and its role (as sensing transmitter, sensing receiver, or both) | No details or examples sensing capability. Role is not applicable for this release as only mono static is support. UE as an SE is described in the solution which should not be in the scope of this release. |
| 13 | NEF selects SeMF through NRF based discovery or according to local configuration. | Sensing area, sensing QoS e.g. the accuracy of sensing result, Sensing service type, e.g. motion monitoring, object detection and tracking as defined in TS 22.137 [2] | SE provides its sensing related info to the AMF (ID, sensing capability) SeMF performs the discovery and selection of the AMF through NRF or based on local configuration. The SeMF selects and reselects the sensing entity(ies) which will participate in the sensing service based on the received sensing entities IDs list from AMF in the Target sensing area. | Sensing capability of a SE, assume the capabilities are sensing QoS, Sensing serve type, although not explicitly mentioned in relevant procedures. | It is arguable whether the sensing capability shall be used to select a SeMF, e.g. a SeMF is responsible for motion monitoring and another SEMF is responsible for object detection. These should be SE’s capability instead. This makes the solution complicated. Therefore, only sensing area may be considered for SeMF selection in normative works. |
| 14 | For untrusted AF, NEF performs NRF based discovery and selection of SF. Trust AF performs NRF based discovery and selection of SF | - Supported sensing service areas (serving areas for which sensing function has the sensing result/data). - Supported sensing modes (sensing modes for which sensing function has the sensing result/data). - Supported sensing types (e.g. tracking, detection). | Not applicable, the solution is only related to SF discovery. | Not applicable, the solution is only related to SF discovery. | It is arguable whether sensing types shall be used to select a SF, e.g. a SF is responsible for motion monitoring and another SEMF is responsible for object detection. These should be SE’s capability instead. This makes the solution complicated. Sensing mode is SE capability and no reason to split sensing data according to the mode. Therefore, only sensing area may be considered for SF selection in normative works. |
| 15 | NRF based SF discovery performed by NEF/(trusted)AF. | Supported area and Sensing service type (e.g. tracking) | SE registers its capabilities to SF directly and SF performs discovery of SE(s). | - Sensing Transmitter and/or Sensing Receiver. - Supported sensing area. | Only mono static is supported in this release. Therefore, the sensing mode shall not be included. It is arguable whether sensing types shall be used to select a SF, e.g. a SF is responsible for motion monitoring and another SF is responsible for object detection. These should be SE’s capability instead. The separation makes the solution complicated. |
| 16 | Not applicable. This solution is only related to SE discovery and selection. | Not applicable. This solution is only related to SE discovery. | SE registers to SF during establishment of an association to the SF. SF performs discovery of SE(s): - Option 1: the SF may select a primary Sensing Entity and delegates further decisions like sensing mode, sensing transmitter(s) and sensing receiver(s) selection to the primary Sensing Entity. - Option 2: the SF may select one or more Sensing Entities to act as sensing transmitter(s) and sensing receiver(s) and the sensing mode. | SF bases on Sensing capabilities and radio serving area (assume to be NG-RAN serving area?) to select SE(s) SF bases on sensing service type and requested KPIs to select option 1 or option 2. | The KPI seems to be SCF internal logic to know which option is better. |
| 17 | NRF based SeMF discovery and selection. | - Target Service Area consisting of geographical co-ordinates-based representation of the sensing service area, one or more Cell ID(s), Tracking Area(s). - Supported Quality of Service information to indicate the accuracy of the sensing e.g. as listed below: - Sensing range. - Sensing resolution (range and angle). - Response time (latency). - Identity of the applications supported by the network. - SeMF load. - Indication of either a single event report or multiple event reports i.e. duration of event reporting. | SE provides its sensing related info to the AMF (Service Area, sensing capability) SEMF performs the discovery and selection of the AMF for through NRF or based on local configuration, for the target sensing area. The SEMF selects the sensing entity(ies) which will participate in the sensing service based on the received sensing entities IDs list from AMF for the are in the Target sensing area. | - Sensing Entity(ies) Capabilities. - Requested Service Area consisting of one or more Cell ID(s), Tracking Area(s). | This solution is like solution#13. |
| 35 | Not applicable. This solution is only related to SE discovery and selection. | Not applicable. This solution is only related to SE discovery and selection. | The Sensing Service Function (SSF) obtains the NG-RAN(SE) information via OAM. The SSF receives sensing service request may include sensing target area information from the NEF or trusted AF. Based on the received sensing target area information and the NG-RAN information configured by OAM, the SSF selects the NG-RAN node(s). | SE capability of providing sensing service, sensing service as Tx or Rx or both Tx and Rx, supported Area. |  |
| 36 | Not applicable. | Not applicable. | No procedures are specified for SE reselection although the title has reselection. | Sensing capability is described as follows: supported sensing mode, supported accuracy of sensing, confidence level, sensing resolution, false alarm probability, missed detection probability, refreshing rate, max sensing service latency, Tx/Rx support indicator, etc | This solution does not have any reselection details and only revocation of a sensing task when an object moves into a Sensing Non-Allowed Area or is out of coverage. This allows the SE to perform a reselection of SE(s). Whether the capabilities mentioned here should be considered for SE discovery should be coordinated with RAN WGs. |

# 2. Text Proposal

It is proposed to agree the following principles to 3GPP TR 23.700-14.

>>>> Start of Changes <<<<

### 7.1.3	Agreed Principles for KI#3 on Sensing Entity and Sensing Function Discovery and (Re-)Selection

For a 3GPP network to select a Sensing Core Function (SnCF) for the sensing service, the following main principles apply:

-	A SnCF shall support NRF-based NF registration and NF discovery procedures. The SnCF shall be provisioned with one or more SnCF Serving Sensing Areas in which it can provide sensing service. The SnCF shall include the provisioned SnCF Serving Sensing Area(s) as part of its NF Profile during registration with the NRF, the SnCF may include additional NF Profile attributes applicable for existing NF registration, e.g., load and priority, etc.

-	Upon receipt of a sensing service request from a third-party AF, the NEF shall map the External Target Sensing Area (e.g. GAD shapes) provided in the request into a Target Sensing Area. The NEF shall select appropriate SnCF instance(s), based on SnCF Serving Sensing Area(s) (in the SnCF NF profile from NRF), that can serve the sensing request within the Target Sensing Area.

For a 3GPP network to select an NG-RAN node for the sensing service, the following main principles apply:

-	The NG-RAN can provide the NG-RAN Serving Sensing Area(s) to the SnCF via a direct association with SnCF. Alternatively, the SnCF can be pre-configured (via OAM) with NG-RAN Serving Sensing Areas.

-	The SnCF selects an NG-RAN node that can serve the sensing request in the Target Sensing Area according to its NG-RAN Serving Sensing Area.

Editor’s Note: Whether and how NG-RAN nodes can provide NG-RAN Serving Sensing Area(s) to the SnCF requires RAN input.

>>>> End of Changes <<<<