3GPP TSG-WG SA2 Meeting #171	S2-2509043

Wuhan, CN, 13th Oct – 17th Oct, 2025	(revision of S2-2507288)

Source:	Huawei, HiSilicon, China Telecom

Title:	[KI#3&4, New Sol] Emergency number provision for UE access to NB-IoT GEO based on UE location

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract: Solution to support emergency number provision for UE access over NB-IoT GEO based on UE location.

# 1. Introduction/Discussion

In order to enable UE detection of an emergency session, MME may provide emergency numbers to the UE at successful registration and subsequent registration updates.

For a UE access to network over NB-IoT GEO satellite, in order to guarantee UE can get correct emergency numbers in the case of a PLMN severing multiple countries, UE location should also be considered.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-19.

# * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| X |  |  | X | X |

# * * * * Second change (all new text) * * * *

## 6.X	Solution #X: Emergency number provision for UE access to NB-IoT GEO based on UE location

### 6.X.0	High level solution Principles

UE location should be considered when providing emergency numbers to UE, as an MME may serve different PLMNs of different countries or a PLMN severing international areas (e.g., MCC 901).

- For a UE access to a PLMN serving a specific country/area over, an emergency number list associated with the PLMN serving country/area may be provided to UE after location verification.

- For a UE access to a PLMN severing international areas, an emergency number list corresponding to its current country/area may be provided to UE after location verification.

MME also should provide an Emergency Service Support indicator in the Attach and TAU procedures to the UE access to NB-IoT GEO.

### 6.X.1	Description

As a NB-IoT GEO satellite may serve multiple countries/areas, when the UE moves between different multiple countries/areas, different emergency numbers should be provided to UE by MME.

-	If the NB-IoT GEO satellite serves for multiple PLMNs of different countries/areas, the MME can provide correct emergency numbers associated with the selected PLMN after location verification.

-	If the NB-IoT GEO satellite serves for a single PLMN (e.g., with MCC 901) covering different countries/areas, the MME may need to know the country/area UE located and provide or update the emergency numbers to UE with UE movement between countries/areas.

If the network supports emergency service, the MME should also provide an Emergency Service Support indicator to UE at successful Attach and TAU procedures.

### 6.X.2	Procedures

The overall procedures of providing emergency numbers to UE based on UE location are illustrated in figure 6.X.2-1.

Figure 6.X.2-1: UE Emergency number provision for UE accessing NB-IoT GEO based on UE location

1.	The UE initiates Attach/TAU Request and may report Coarse Location to MME via NAS SMC for basic UE location verification. The MME performs basic UE location verification as specified in clause 4.13.4 of TS 23.401 [5] which may include steps 2-4 to further get a more accurate UE location.

1a.	MME proceeds with the Attach/TAU procedure and sends Attach Accept or TAU accept to UE with Emergency Support Indicator and Emergency numbers. The emergency numbers are associated with the selected PLMN or corresponding to current UE location (e.g., in the case of the selected PLMN with MCC 901).

Based on operators’ policy, the MME may not provide emergency numbers to UE until it is able to determine the UE location with sufficient accuracy.

2-4.	If the MME is not able to determine the UE location with sufficient accuracy to make a decision or if the received ULI is not sufficiently reliable, the MME initiate UE location procedure after the Mobility Management or Session Management procedure is complete, as specified in clause 9.1.17 of TS 23.271 [xx], to determine the UE location (i.e., a country/area presented by an MCC).

5.	Based on steps 2-4, if the UE is registered to a PLMN (e.g., with MCC 901) that is allowed to operate in the UE location area and the MME determines that UE emergency numbers need to be updated, the MME provide the UE with emergency numbers valid for the UE location (i.e., the country/area presented by an MCC).

If the MME does not provide emergency numbers to UE in step 1a, it may provide emergency numbers to UE in this step.

### 6.x.3	Impacts on Services, Entities and Interfaces

MME:

-	Provides Emergency Support Indicator and Emergency numbers to NB-IoT UE with GEO access.

-	Determine Emergency numbers based on UE location if MME serves for a single PLMN (e.g., with MCC 901) covering different countries/areas.

UE:

-	NB-IoT UE accessing to GEO supports receive Emergency Support Indicator and Emergency numbers.

# * * * * End of changes * * * *