3GPP TSG-WG SA2 Meeting #171  	S2-2509114

# Wuhan, China, 13 – 17 October 2025 	(same as S2-2507358)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6418 | rev | 1 | Current version: | 18.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multi-RAN-vendor support for paging UEs in RRC Inactive, and, correct handling of NB-IoT UE paging capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT, TEI18 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 5.4.4.3 already states correctly that, “…the AMF only infrequently (e.g. at Initial Registration) prompts the NG-RAN to retrieve and upload the UE radio capabilities i.e. UE Radio Capability information to the AMF…” However, the subsequent text does not address the case where the Initial Registration is performed with a, for example, pre-Rel 17 gNB which does not know how to process the UE Radio Access Capabilities in order to generate the RRC Inactive Paging Occasion Determination and other paging related information in the TS 38.331 UERadioPagingInformation that is sent in TS 38.413 signalling messages. For this situation, a Rel 17 capable gNB should also analyse the UE Radio Capability for Paging Information that is sent in every NG-AP INITIAL CONTEXT SETUP REQUEST message to check in the UE Radio Access Capabilities that the UE Radio Capability for Paging Information contains the relevant information, and if it does not, then get them uploaded to the AMF (so that they can then be used in subsequent NG-AP Paging messages). |  |  |  |  |  |  |  |  |
| Summary of change: |  | gNB check for all relevant paging information in the NG-AP INITIAL CONTEXT SETUP REQUEST added, and requirement to update AMF added. Description of NB-IoT paging capability handling added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Paging is unreliable when the UE is in RRC Inactive. The radio coverage for paging might be insufficient for XR devices with only 2 receive antennas. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Yellow highlights indicate the difference in this CR compared to the Rel 17 CR |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.4.4.3	Paging assistance information

The paging assistance information contains UE radio related information that assists the RAN for efficient paging. The Paging assistance information contains:

a)	UE radio capability for paging information:

-	The UE Radio Capability for Paging contains information derived by the NG-RAN node (e.g. band support information) from the UE Radio Capability information. The AMF stores this information without needing to understand or process its contents other than recording whether the information is received from an NB-IoT cell or a wideband NG-RAN cell.

-	If a UE supports both NB-IoT and other RAT(s), then

-	When the UE is using NB-IoT, only the UE Radio Capability for Paging for NB-IoT is signalled between the RAN and the AMF.

-	When the UE is not using NB-IoT, then the UE Radio Capability for Paging for NB-IoT is not signalled between the RAN and the AMF.

As the AMF only infrequently (e.g. at Initial Registration) prompts the NG-RAN to retrieve and upload the UE radio capabilities i.e. UE Radio Capability information to the AMF and the AMF may be connected to more than one wideband NG-RAN RAT, it is the responsibility of the NG-RAN to ensure that UE Radio Capability for Paging (which is derived by the NG-RAN node) contains information on all wideband NG-RAN RATs and all relevant features (e.g. RRC Inactive Paging Occasion Determination, 2 Receivers for XR devices) that the UE supports in that PLMN. To assist the NG-RAN in this task, as specified in TS 38.413 [34], the AMF provides its stored UE Radio Capability for Paging Information in every NG-AP INITIAL CONTEXT SETUP REQUEST message sent to the NG-RAN. If the NG-RAN detects an omission, then the NG-RAN should ensure that an update of the UE Radio Capability for Paging is sent to the AMF.

-	The UE Radio Capability for Paging Information is maintained in the core network, even during AMF reselection and is stored in the UCMF alongside the UE Radio Capability information associated to a UE Radio Capability ID.

b)	Information On Recommended Cells and RAN nodes For Paging:

-	Information sent by the NG-RAN and used by the AMF when paging the UE to help determining the NG RAN nodes to be paged as well as to provide the information on recommended cells to each of these RAN nodes, in order to optimize the probability of successful paging while minimizing the signalling load on the radio path.

-	The RAN provides this information during N2 release.

# * * * * End of changes * * * *