SA WG2 Meeting #170	S2-2506518

25 – 29 Aug, 2025, Goteborg , SE

Source:	Deutsche Telekom, AT&T, China Mobile, Cisco, Intel, KPN N.V., Rakuten Mobile, Sateliot, SK Telecom, Telecom Italia, Vodafone

Title:	[WT#1.2 SBA Framework] Enhanced SBA for 6G

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution covers the SBA framework topic and provides an overview of current limitations fully or partially stemming from the current architecture, based on which a scope for the related (sub-)Work Task (WT) and related Key Issue (KI) are proposed.

# 1. Justifications

In this contribution, we aim at providing an overview of current limitations fully or partially stemming from the current architecture, based on which a scope for the related (sub-)Work Task (WT) and related Key Issue (KI) are proposed. We present justifications reflecting learnings gained during the last years and we propose a WT scope and KI for 6G based on scope described in SP-250806.

It should be noted that the topic of extending the scope of SBA has been discussed in the context of 5G (e.g. see Release 18’s S2-2108938), and no consensus could be found on whether such work should be undertaken for 5G.

Many features are proposed for 6G, and we also consider it important to study how the architectural underpinnings of the Control Plane (CP) can be enhanced. A solid architecture will also enable a more solid foundation on which 6G services can be built.

While surely most, if not all of the individual motivations can be dismissed as not critical by themselves, the combined effect of all these issues, results in architectural limitations that we think can be addressed in the framework of enhanced SBA.

We split this justifications section in sub-section where we present an overview of the motivation for SBA enhancements/application for different use cases.

## RAN, AMF and N2 interface

In this section, we describe the motivation regarding current functionality provided by the N2 interface and between RAN/AMF.

### 1.1.1 Introduction

The architecture for the N2 interface (NG-C as defined in TS 38.300) has been used since previous generations and provides a well-defined RAN/CN split using a single point of entry into the CN, which for 5G was further extended to consider slicing, as:

- A gNB can be connected to multiple AMF instances

- Different AMFs may serve different NSSAIs

- MOCN is widely used to implement network sharing

Our current experience has yielded that AMF scaling cannot be natively supported and is complex to automate. Some improvements are possible, but the architecture is what it is. This is mainly due to the following architectural, functional and protocol-related decisions:

- The association between UE signalling and a specific SCTP connection is sticky (see clause 7 of TS 38.412)

- Any scaling of pods/processes hosting SCTP endpoints results in changes to Transport Network Layer (TNL) endpoints and/or TNL Associations (TNLAs), which trigger an NGAP AMF CONFIGURATION UPDATE procedure to signal the (re-)configuration of the SCTP IP endpoints to all connected gNBs (see clause 8.8. of TS 38.413). In short, it couples any scaling action to generation of signalling load towards RAN

- The AMF performs NAS security termination and is responsible to forward NAS messages to other NAS-capable NFs (see clause 8.2.2.1 of TS 23.501)

- The AMF is also responsible for forwarding N2-SM messages between the gNB and SMF

Due to the fact that an AMF failure would result in a total failure of UE CP and connectivity, AMFs need to be (over-)dimensioned such that no bottleneck is possible, i.e. with enough spare capacity and redundancy to minimize the possibility of an outage.

The issues highlighted are connected to the current possibilities to provide a secure N2 interfaces (see sub-section below) and the NAS functionality split (see S2-2506517).

### 1.1.2 Current situation for AMF deployment

Current cloud-based AMF implementations need to provide an IP termination point for SCTP connections.

The reality is that SCTP, although formally in the list of supported protocols, it is not really supported in current cloud platforms. SCTP being a niche protocol used in telco applications, this situation is not expected to improve, neither its P2P nature.

As such, mechanisms commonly used in container-based deployments such as Services, Gateways, Ingress rules, Ingress controllers), Service mesh and native autoscaling are challenging to use with the current interface design.

Currently, N2 functionality is accomplished by bypassing native mechanisms and requires either:

- Terminating the SCTP connection at a termination point (an “N2 pod”) and then use HTTP (or HTTP-based proprietary means such as gRPC) to distribute load to worker pods. The N2 pod hides the (transparently scalable) pods behind it, and even if it is not, from current experience that resource-intensive, it requires application-level protocol support and adds a further, telco-specific custom layer.

- Terminating SCTP directly at pods. In this case, scaling results in the triggering of signalling via N2 (NGAP AMF CONFIGURATION UPDATE) with updated SCTP endpoint information to the gNB. In current deployments, an AMF may be connected to thousands of gNBs.

Figure 1.1.2-1: SCTP termination at LB pod (left) and SCTP termination at worker pod (right)

For both cases, this results in the need to bind an external IP address to a container within the cluster. In general, this can result in:

- A re-configuration of the AMF IP addresses on the gNB when AMF deployment changes

- Hindering of the autoscaling, leading to a static AMF deployment (in case of SCTP termination at worker pod)

- RAN signalling when an AMF  is scaled, or restarts

- FQDN + DNS resolution when using direct pod termination is not an option because gNBs do not have this function and a DNS service with load distribution capabilities does not exist in production environments

Regardless of the scaling method used, the current linkage between TNLAs, N2-related logic and AMF instances result in the following complexities when trying to deploy and AMF in a cloud-native way, such that it can be automated and scaled:

- IP address(es) of SCTP endpoints need to be included in deployment configurations, such that these IPs can be bound to specific pods bypassing typical cloud mechanisms

- When workloads including SCTP endpoints are scaled, this effectively changes the available TNL endpoints, which in turn triggers an NGAP AMF CONFIGURATION UPDATE including an appropriate TNL Address Weight Factor to all gNBs served by this AMF instance (could be in the order of thousands of gNBs), see clause 8.7.3.2 of TS 38.413. If existing weight factors need to be updated, these need to be signalled to the 5G-AN as well

- Scaling of any workload including SCTP endpoints does not immediately distribute load across AMF instances. While the gNB would apply any updated TNL endpoints and associated weight factors to new associations, an existing NGAP UE-TNLA-binding will only be redistributed by means of a re-establishment, triggered when the AMF releases the NGAP UE-TNLA binding without sending AMF unavailable status indication (see clauses 4.2.7.2.3 and 4.2.7.2.5. of TS 23.502)

### 1.1.3 Topology Aspects

Topology-wise, and connected to the NAS routing issue detailed in S2-2506517, the AMF is responsible for the forwarding of N2 SM information towards/from the SMF (the NG-AP container is transparently transported via SBI), as depicted in Figure 8.2.1.3-1 of TS 23.501 shown below.

Figure 1.1.2-3: Control Plane between the 5G-AN and the SMF, adapted from Figure 8.2.1.3-1 of TS 23.501

A result of the AMF termination mandated by the current architecture is that N2-SM message must be routed via the AMF. With the current architecture, this limitation will also extend to any future functionality using the N2 interface.

Over time, the architecture has been stretched to cover use cases it was not originally designed to cover. In 5G, Non-Public Network (NPNs, see TR 23.734, TR 23.700-07), deployments considered scenarios where localized and centralized deployments are combined. Additionally, Non-Terrestrial Networks (NTNs) raised the question of what core network elements/network functions that should be placed on board the satellite(s) (see TR 23.700-29).

The fact is, that the 5GS aims to cover deployment scenarios ranging between a centralized deployment, and a distributed deployment where network components maybe be tens of thousands of kilometres apart (e.g. terrestrial, LEO, MEO, GEO). 6G is expected to build further on those scenarios and study potential new ones.

Figure 1.1.3-1: Deployment diversity the 3GPP system aims to cover

We consider it a valid point to question whether having the AMF as the point of entry and single-point-of-failure in the CN restricts deployment possibilities and whether 6G could provide enhancements in this regard, both in terms of reliability and deployment topology.

### 1.1.4 Relation to IP backhaul and security

A ramification of the N2 architecture design is the provision of security for the N2 interface. While security is in the remit of SA3, not mentioning this aspect would show an incomplete picture of the E2E consequences of the RAN/CN design.

For some operators, securing the N2 interface is not only a choice but even required by regulators (e.g. without additional N2 security, radio interface cryptographic keys are sent in clear from the AMF to the gNB via N2). At the same time, deployments not required to secure the access backhaul also exist. Current means of protection, as described in clause 9.2 of TS 33.501, are based on deploying an IPsec layer to securely provide a P2P, secure layer between RAN and one or more IP Sec Gateways CN components use to route traffic.

Figure 1.1.4-1 shows an example resulting from the combination of the topology aspects mentioned in the previous section and the security measures available for the N2 interface.

Figure 1.1.4-1: IPsec between CN and RAN depicting the SCTP/N2-MM termination (left) and N2-SM containers (right)

The IPsec layer realizes an overlay network built on top of the operator’s Transport Network (TN). This layer needs to be designed, necessary equipment purchased (gateways and/or line cards), deployed, maintained and operated. Whenever a failure occurs, this can result in the IPsec route being used instead of an optimum routing towards a new node.

Our experience shows that the more layers are stacked on top of each other, the more difficult troubleshooting becomes, as different layers are involved (e.g. route advertisement is performed not only in the TN but also via IPsec tunnels).

There may be potential for reducing operational complexity and potentially cost, if a path forward not requiring IPsec is provided, which necessarily needs to cover the following needs IPsec covers:

- N2, both towards AMF and SMF (this WT description)

- Control Plane being transported over N2 (see S2-2506517)

- User Plane (SA3 remit)

### 1.1.5 SBA for N2 interface

While the current P2P, STCP-based interface is workable for 5G (we do not deny that it does work), we propose to study for 6G to enable native SBI & cloud-native support instead of prolonging    the current architecture ad-infinitum.

We highlight the following aspects:

- By defining NF services (SBI APIs), modularity and extensibility can be achieved. New features can be easily distinguished by means of a different URI (i.e., defining a separate API)

- Load balancing: Message destinations can be identified based on the target URI in the HTTP header, which can contain resource identifiers (e.g. CRUD service operations, see clause 4.6.1.1 of TS 29.501; RPC-style interactions, see clause 5.2.3 of TS 29.501). Message routing (e.g. with a Service mesh) and load-balancing can be performed without need to parse the message payload and without needing support for telco-specific protocols

- Release current association binding: Currently, each UE-associated signalling is bound to a SCTP association (IP source + IP destination), and non-UE-associated signalling is also bound to a SCTP association. With SBA-based N2, resource-based granularity can be achieved, where service operations and created resources can be addressed by a URI containing a host and path component (see clause 4.4.1 of TS 29.501 and section 3 of RFC 3986)

- Backwards-compatible topology: A deployment topology where all N2 communication is terminated at a single entry point can still be realized (e.g. by setting a single Gateway for all SBA-based communication entering the CN from RAN). Endpoint placement would be a deployment choice instead of an architectural requirement.

- Cloud-nativeness: Scaling and automation with HTTP-based services are well understood and in use in current CN deployments. By using SBA, CN scaling need not trigger signalling towards RAN

- Product availability: the Infrastructure-as-a-Service (IaaS) layer is a component of increasing importance for operators’ workload deployments. Since SBA is based on protocols (HTTP) and interactions (API-based) commonly used in cloud deployments, and adapted to telco needs (instead of specifically built for using telco-specific protocols), product (and support) availability for proxies, firewalls, load balancers, observability and operations are more widely available. Currently, load balancing does not integrate well into a cloud-based deployment, requires application-awareness (i.e. payload inspection) and telco-specific protocol support.

- Integrated authentication and authorization to NF service operations by reusing already-defined mechanisms (OAuth2, see clause 13.4 of TS 33.501)

- Deployment flexibility of encryption: SBA provides means to flexibly control deployment of encryption (HTTP over TLS) where needed.

- Reuse operational learnings: There was (and still is) a learning curve to the adoption of SBA. Operators have had to learn how to deploy and operate a layered system where some layers are telco-specific and some others not, as well as how to best use the flexibility and elasticity cloud-based workloads enable. If SBA is applied to other interfaces, these learnings can be reused

- The interaction between RAN and CN is a topic involving both SA and RAN WGs. It is, as of now, not clear whether the full scope of SBA functionality will be deemed necessary for RAN/CN interaction (e.g. bindings, SCP) or whether a possible outcome/compromise is, that only certain SBA aspects are applicable for RAN/CN

### 1.1.6 Gap between current N2/NG-C and SBA

It is clear that a big distance exists between the current architecture and interface between RAN/CN (N2/NG-C) and SBA. This is also reflected in different views prevalent in both the CN and RAN side.

It is hence important to allow for a degree of flexibility, granularity detail and differentiation on how SBA could be applied to between RAN and CN. While SBA has been applied within the CN by and defined SA WGs, application between RAN and CN requires interaction with RAN WGs.

At this point, it is not clear

- whether application of all of the features the SBA framework provides will be seen as necessary by RAN,

- what level of granularity (e.g. NF service split) should apply to existing functionality (e.g. connectivity, sensing),

- whether latency is impacted

- whether Inter-Operability Testing (IOT) will be more complex

- whether security, especially topology hiding aspects (SA3 remit)

and these aspects should be considered in the study phase by the relevant WGs

### 1.1.7 Summary

Architectural Motivation: The functional split and interface between gNB and AMF results in deployment and operations complications, as well as topology limitations. As it is, a cloud-native deployment of the AMF, especially regarding automation and scaling is very challenging to achieve.

Gap Analysis: How SBA could be applied to the N2 interface.

Technical Impact: Following impacts are expected to be covered to resolve this gap:

- Functional split between gNB and CN regarding the exchange of N2 messages, including what SBA features apply

- Interactions between gNB and CN

- Identification of possible security impacts such that SA3 can work on them

- Interworking and migration from 5G to 6G

## 1.2	SMF, UPF and N4 interface

### 1.2.1 Introduction

The current functional split between SMF and UPF requires supporting that User Plane can be exchanged between the SMF and UPF (see clauses 6.2.2 and 6.2.3 of TS.23.501). This mainly related to the forwarding via GTP-U of the following information between an SMF and a UPF (see clauses 4.1 and 5.3 of TS 29.244, which describe data forwarding between the CP and UP functions and their associated protocol stacks) and is basically inherited from the 4G CUPS architecture and functional split:

- DHCP

- DN-AAA

- End Marker

- CIoT packets

- SMF-based UP buffering

This results in SMF/UPF deployment as shown in the figure below:

Figure 1.2-1: SMF/UPF current deployment

Currently, the UPF already supports exposure via SBI, defined during the UPEAS (Release 18) and UPEAS_Ph2 (Release 19) Work Items. However, the N4 interface itself remains based on PFCP/GTP-U

Our experience so far is that the IPs associated to N4 P2P interfaces need to be included in the deployment configurations, which complicates automation. Furthermore, two parallel (sub-)interfaces need to be run in parallel in order to support N4, sometimes even requiring different internal networks/interfaces, which further complicates the deployment.

Since neither PFCP nor GTP-U are natively supported in Kubernetes, mechanisms commonly used in container-based deployments such as Services, Gateways, Ingress rules, Ingress controllers, Service mesh and native autoscaling are challenging or impossible to use.

Additionally, if an operator desires to secure the N4 interface, no integrated solution is possible. As per clause 9.9 of TS 33.501, this operator choice can be realized via IPsec. While this is certainly an option, it requires an additional layer to design, deploy, maintain, and operate (see N2 section for more details). Additionally, if an operator decides to apply granular authorization on their CN deployment, authorization mechanisms are only possible on the SBA interfaces due to the UPF functionality not being SBA-based and thus the lack of support on the N4 interface.

### 1.2.2 Summary

Architectural Motivation: The functional split and interface between SMF and UPF results in deployment and operations complications. As it is, a cloud-native deployment, especially regarding automation is very challenging to achieve.

Gap Analysis: How SBA could be applied to the N4 interface.

Technical Impact: Following impacts are expected to be covered to resolve this gap:

- Functional split between SMF and UPF

- Interactions between SMF and UPF

- Identification of possible security impacts such that SA3 can work on them

- Interworking and migration from 5G to 6G

## 1.3	Existing SBA-based interfaces

The SBA framework was developed for Release 15 and enhanced in the 5G_eSBA WI with features such as NF Set, SCP and bindings. The reality is that SBA Release 16 features are not widely deployed and field experience with those features is not widely available.

Whether SBA needs architectural enhancements or not, is at this moment not clear, and it should not be taken for granted. While protocols have since evolved (e.g. RFC 9114 describing HTTP/3 was release in 2022), protocol-only aspects are the remit of CT4.

Since it is expected that 5GC NFs will, in some degree or another, be re-used and/or enhanced, special care should be taken to allow for backwards-compatibility where needed.

### 1.3.1 Summary

Architectural Motivation: We currently see no need for architectural enhancements to the SBA framework in general.

Gap Analysis: None found.

Technical Impact: Changes to the SBA framework could potentially impact all CN NFs, especially:

- Interactions between NFs

- Interworking and migration from 5G to 6G

## 1.4	Future RAN/CN interfaces

### 1.4.1 Current Situation between RAN/CN

Currently, the addition of new features having RAN impact typically requires updating and maintaining a chain specifying:

- the IEs defined within the NG-AP protocol to support this additional feature,

- the AMF procedures receiving said information (Even if the functionality is unrelated to MM)

- and any other NFs involved in the functionality

Since the N2 interface is monolithically conceived (i.e. in “one piece”), any new functionality ends up invariably tied functionally and architecturally to the AMF and the gNB/AMF SCTP association.

### 1.4.2 Current Situation within CN

Since SBA was introduced in Release 15, we have strived to adhere to the definition of NF service as “a functionality exposed by a NF through a service-based interface and consumed by other authorized NFs”, which is part of a Network Function, “A 3GPP adopted or 3GPP defined processing function in a network, which has defined functional behaviour and 3GPP defined interfaces” as defined in TS 23.501.

SBIs are realized by APIs that provide a standardized, human- and machine-readable, modular and extensible manner of describing functionality a NF exposes to other network elements described via the widely-used OpenAPI specification, and that can be discovered using the SBA framework.

The high number of NF services SA2 defined since Release 15 reflects the many features that can be consumed and/or steered, all while having a modular interface definition.

For an operator, this provides an easy means to not deploy certain functionality (e.g. by not having specific NFs or by stating that certain NF services are not required), as well as add specific functionality when/where needed.

It should be noted that the fact that functionality is defined by means of NF services does not imply that NF services are bundled in a specific way. In fact, products can implement any combination of NF services.

### 1.4.3 SBA for future interfaces between RAN/CN

The aim of explicitly including SBA for future interfaces between RAN/CN is to ensure that a modular, extensible system can be achieved. This goal has been often expressed by operators (e.g. see S2-1910395). SBA enables flexibility and modularity for newly introduced functionalities, and it should be possible to apply this principle to E2E functionality that includes RAN.

While it is, at this moment, unclear, what new services will be finally provided/required by the 6GS, the candidate list contains topics such as Agent-related communication, sensing, AI/ML, data exposure, NAS enhancements and more. By using a modular framework, an important building block towards extensibility and modularity could be provided.

### 1.4.4 Summary

Architectural Motivation: Enable extensibility of the RAN/CN interface.

Gap Analysis: Currently, new functionality relies on extensions of the existing N2 interface.

Technical Impact: Definition of future functionality using the SBA framework

## 1.5	Linkage with interworking aspects

Since the CN for 6G is may end up re-using elements of the 5GC and 5G SA adoption may increase during the next years, we think that it is important to include a linkage to WT#2, such that solutions must either include interworking aspects or explain why interworking is not necessary.

**** First Change (all text new)****

# Annex A.1.2.X WT#1.2 SBA framework

WT 1.2 covers whether and how to support and/or enhance the SBA framework.

This topic is scoped by the WT descriptions in Table A.1.2.X-1.

Table A.1.2.X-1: Work tasks for SBA framework

| Work Task ID | Work Task(s) | Work Task Description |
| --- | --- | --- |
| SBA_WT_#1.2.1 | SBA for the RAN Core Network control plane interaction (N2 interface) | 1.1 How can the SBA be applied to existing functionality over the N2 interface, including what SBA features apply 1.2 Functional split between RAN and CN regarding the exchange of N2 messages (NOTE U) 1.3 Interactions between RAN and CN including NF service definition (NOTE V, NOTE W, NOTE X, NOTE Y) |
| SBA_WT_#1.2.2 | SBA for the control of the user plane (N4 interface) | 2.1 How can the SBA be applied to existing functionality over the N4 interface 2.2 Functional split between SMF and UPF 2.3 Interactions between SMF and UPF including NF service definition (NOTE X, NOTE Y) |
| SBA_WT_#1.2.3 | SBA for future RAN/CN interfaces | 2.1 How can the SBA be applied to future RAN/CN functionality, i.e. functionality not currently existing in 5G (NOTE X) |
| NOTE U: Does not cover other architectural changes (e.g. paging) NOTE V: Topology hiding aspects are to be evaluated by SA3 NOTE W: Work on the N2 interface to be conducted together with RAN3 NOTE X: Interworking aspects for the SBA framework are covered by WT#2. NOTE Y: It is acknowledged that performance impact related to message encoding is an important aspect, which is in CT4 scope |  |  |

Editor’s Note: N2, N4, AMF, SMF, UPF, etc. all refer to functionality and interfaces for 6G. The final terminology (e.g. 6G-N2, eN2, etc.) has yet to be decided.

**** Second Change (all text new) ****

# 5.X. Key Issue #X: SBA framework

The Service Based Architecture is an integral part of the 5GC, and it was designed to provide a modularized function design and enable each Network Function and its Network Function Services to interact with other NF and its Network Function Services.

As defined in TS 23.501 [X], Network Function (NF) is a 3GPP adopted or 3GPP defined processing function in a network, which has defined functional behavior and 3GPP defined interfaces. A NF Service is a functionality exposed by a NF through a service-based interface and consumed by other authorized NFs.

This key issue focuses on defining the remaining interfaces in the control plane as SBIs, which have not yet been considered in 5G, to achieve a full SBA in CP in the 6G architecture. Solutions for this key issue will describe, based on the WT scope described in Annex A.1.2.X WT#1.2 SBA framework:

-	Application of the SBA for the RAN Core Network control plane interaction (N2 interface)

-	Application of the SBA for the control of the user plane (N4 interface)

-	Application of the SBA for future RAN/CN interfaces, i.e. functionality currently not existing in 5G.

Solutions are expected to include interworking aspects (see KI#Y) or argument why interworking aspects are not necessary to be considered.

Editor’s Note: N2, N4, AMF, SMF, UPF, etc. all refer to functionality and interfaces for 6G. The final terminology (e.g. 6G-N2, eN2, etc.) has yet to be decided.

**** End of Changes ****