3GPP TSG-SA2 Meeting #170	S2-2506533

# Gothenburg, Sweden, August 25th-29th, 2025		(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0011 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ADM Service Operations Update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Specified ADM services and service operations require update. - Nadm_DM_Query operation is used for retrieving from ADM either AIOT Device Profile data or AF authorization data. Each ADM data set may require a different data key (AIOT device permanent ID or AF ID). AF ID for AF Authorization data is optional. If not provided, the Authorization Data for all the AFs are to be retrieved. It also aligns with stage 3 specification. - As defined in clause 5.6, AIOTF may perform authorization of AIoT service requested by the AF, using the AF authorization data retrieved from n the ADM or configured locally as described in above Table 5.6-1. When ADM is used, the AIOTF also subscribes to changes of AF authorization data in the ADM for synchronization. The subscribe/notify related operations have not been defined yet in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For the Nadm_DM_Query service operation, an ADM Data Set indication is included as mandatory input while the corresponding data keys are defined as optional input (for AF authorization data, AF ID is optional). - Additional Nadm_DM_Subscribe/Unsubscribe/Notify service operations are defined for the AIOTF to manage notifications about AF authorization data changes in ADM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete/incorrect specification. Incorrect/erroneous interworking with ADM |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.1, 7.5.2, 7.5.3, 7.5.x (new), 7.5.y (new), 7.5.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * Start of Changes * * *

## 7.5	ADM services

### 7.5.1	General

The following table shows the Nadm_DM Service and Service Operations.

Table 7.5.1-1: NF services provided by ADM

| NF service name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nadm_DM | Query | Request/Response | AIOTF, NEF |
|  | Update | Request/Response | AIOTF |
|  | Subscribe | Subscribe/Notify | AIOTF |
|  | Notify |  | AIOTF |
|  | Unsubscribe |  | AIOTF |

### 7.5.2	Nadm_DM_Query service operation

Service operation name: Nadm_DM_Query

Description: NF service consumer may request the AIoT device profile data or the AF authorization data from the ADM.

Inputs, Required: None.

Input, Conditional: AIoT Device Permanent ID (when ADM Data Set Identifier is AIoT device profile data).

Input, Optional:  AF ID (when ADM Data Set Identifier is AF authorization data).

Outputs, Required: the AIoT device profile data or the AF authorization data.

Output, Optional: None.

### 7.5.3	Nadm_DM_Update service operation

Service operation name: Nadm_DM_Update

Description: NF service consumer may update the AIoT device profile data in the ADM.

Inputs, Required: AIoT Device Permanent ID, updated AIoT device profile data.

Input, Optional: None.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure.

Output, Optional: None.

### 7.5.X	Nadm_DM_Subscribe service operation

Service operation name: Nadm_DM_Subscribe.

Description: NF service consumer performs the subscription to notification to data modified in the ADM. The events can be changes on existing data, addition of data.

In particular, this service operation together with the corresponding Unsubscribe and Notify service operations are used by the AIOTF to manage notifications of changes of AF authorization data in ADM.

Inputs, Required: ADM Data Set Identifier, Notification Target Address (+ Notification Correlation ID).

Inputs, Optional: AF ID, Subscription Correlation ID (in the case of modification of the subscription).

Outputs, Required: When the subscription is accepted: Subscription Correlation ID.

Outputs, Optional: None.

### 7.5.Y	Nadm_DM_Unsubscribe service operation

Service operation name: Nadm_DM_Unsubscribe

Description: NF service consumer unsubscribes from the notifications about data modified in the ADM. The events can be changes on existing data, addition of data.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

### 7.5.Z	Nadm_DM_Notify service operation

Service operation name: Nadm_DM_Notify.

Description: ADM notifies NF service consumer(s) about modification of data, when data in the ADM is added, modified or deleted and an NF needs to be informed about this, due to a previous subscription to notifications procedure with the ADM.

Inputs, Required: Notification Correlation ID, Updated Data.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

# * * * End of Changes* * *