SA WG2 Meeting #170	S2-2506621

25 – 29 August, 2025, Goteborg, SE	(revision of S2-25xxxxx)

Source:	LG Electronics, Toyota

Title:	KI#1: Interim agreements

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This paper proposes interim agreements for KI#1.

Discussion

There are 8 solutions for KI#1 "Network energy related information exposure" TR 23.700-67v0.2.0.

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solution # & title | 1 | 2 | 3 |
| 1: Enhancement of Exposure API to include UE categorization data | X |  |  |
| 2: Policy Control Considering Slice or 5GC Energy Consumption | X | X |  |
| 8: Estimation of Renewable Energy consumption | X |  |  |
| 9: Consumption energy source exposure | X |  |  |
| 10: Energy consumption calculation enhancements | X |  |  |
| 11: Energy Consumption estimation considering idle energy consumption | X |  |  |
| 12: Enhancement of energy related information exposure | X |  |  |
| 22: Support of UP path adjustments to maximize the usage of renewable energy | X |  | X |

Observation#1: Sol#2 is considered more related to KI#2 "Support of service adjustments for the UE" because this solution mainly proposes AM/SM/BDT policy adjustment.

Proposal#1: It is proposed to discuss Sol#2 when handling interim agreements/conclusions for KI#2.

Observation#2: Sol#22 is considered more related to KI#3 "NF (re)selection and/or UP path adjustment based on energy related information" because this solution mainly proposes UPF (re)selection by considering energy related information such as renewable energy usage along with the existing parameters.

Proposal#2: It is proposed to discuss Sol#22 when handling interim agreements/conclusions for KI#3.

Other solutions than Sol#2 and Sol#22 can be classified and main proposal for each solution can be summarized as below:

1)	Solutions that mainly propose to expose new/additional energy related information

-	Sol#1: UE categorization data at the desired granularity e.g. percentile of the consumption of the UE at the granularity of the estimate.

-	Sol#12: Application ranking related information, i.e. list of top contributing applications and/or ordered list of contributing applications for a UE or for a PDU Session.

2)	Solutions that mainly propose renewable energy aspect

-	Sol#8: New formula to evaluate renewable energy consumption and renewable energy ratio per the given granularities.

-	Sol#9: Renewable energy share of the full consumption energy calculation per requested granularity.

NOTE:	Renewable energy information per node-level has dependency on SA5.

3)	Solutions that mainly propose new formulas for energy consumption calculation to enhance Rel-19 formulas

-	Sol#10: New formulas taking the energy consumed for maintaining the basic operation and assisting the main tasks at the node (e.g., memory retention or background monitoring) into account.

-	Sol#11: New formulas taking idle energy consumption into account.

NOTE:	These two solutions have some Editor's notes need to be resolved.

Observation#3: Sol#1 can assist EIF service consumer (e.g. AF, NF) to make service/QoS adjustment for UE(s) that consumed large portion of network energy.

Observation#4: Sol#12 can assist EIF service consumer (e.g. AF, NF) to make service/QoS adjustment for application(s) consumed large portion of network energy at certain UE. For example, vehicle UEs can operate various applications related to an AF managed or owned by car OEM such as road safety related applications, applications for In-Vehicle Infotainment (e.g. entertainment, navigation, connectivity with smartphones), AI/ML related applications (e.g. Federated Learning). So, if FL related application for any UE consumed a significant amount of energy within some time period, the AF may not select the UE as member UE for FL for a while. Or if some IVI related application for any UE consumed a lot of energy within some time period, the AF may downgrade the QoS requirements for the application of the UE. In summary, Sol#12 can enable the EIF service consumer efficiently to identify application level energy consumption characteristic and to perform application level service adjustment.

Observation#5: If any AF wants to know energy consumption information for all applications related to it for certain UE, the AF can request to the EIF to provide energy consumption information for each application for the UE. Anyhow, Sol#12 can make the AF request energy consumption information for all applications for the UE simply and efficiently by using the indication to request ordered list of contributors.

Observation#6: Sol#1 and Sol#12 can be applied together which means EIF service consumer (e.g. AF, NF) can use application ranking related information proposed by Sol#12 together with UE categorization information proposed by Sol#1 in order to figure out energy consumption characteristic for certain UEs, i.e. application level energy consumption characteristic. For example, the EIF service consumer can be aware e.g. which applications consumed most energy for the UEs that consumed large portion of network energy. Therefore, the EIF service consumer may perform application level service adjustment.

Proposal#3: It is proposed to capture Sol#1 and Sol#12 as Agreed Principles for KI#1.

Observation#7: Renewable energy aspect, e.g. renewable energy consumption, renewable energy portion proposed by Sol#8 and Sol#9 can be considered in addition to non-renewable energy aspect specified in Rel-19. However, renewable energy information per node-level has dependency on SA5.

Proposal#4: It is proposed to capture Sol#8 and Sol#9 as Agreed Principles for KI#1 with NOTE saying that renewable energy information per node-level has dependency on SA5.

Observation#8: For Sol#10 and Sol#11, some Editor's notes need to be resolved for clarification before evaluating these solutions.

Proposal#5: It is proposed to capture Sol#10 and Sol#11 as Topics for further consideration for KI#1, and to wait and see how Editor's notes to be resolved before deciding these solutions whether to include in agreed principles.

Proposal

It is proposed to agree the following changes into TR 23.700-67.

* * * * Start of 1st Change * * * *

!! All New Texts !!

7.1.Y	Agreed Principles for KI#1

The following principles are agreed for KI#1.

-	The following new/additional energy related information is exposed by EIF:

-	UE categorization data at the desired granularity e.g. percentile of the consumption of the UE at the granularity of the estimate as proposed by Solution#1 described in clause 6.1.

-	Application ranking related information, i.e. list of top contributing applications and/or ordered list of contributing applications for a UE or for a PDU Session as proposed by Solution#12 described in clause 6.12.

-	Renewable energy aspect is taken into account for collecting, calculating and exposing energy consumption information by EIF, as proposed by Solution#8 and Solution#9 described in clause 6.8 and clause 6.9, respectively.

NOTE:	Renewable energy information per node-level has dependency on SA5.

* * * * Start of Next Change * * * *

!! All New Texts !!

7.2.Y	Topics for further consideration for KI#1

The following topics can be considered for KI#1 once Editor's notes for clarification in each solution are resolved.

-	New formulas taking the energy consumed for maintaining the basic operation and assisting the main tasks at the node (e.g., memory retention or background monitoring) into account as proposed by Solution#10 described in clause 6.10.

-	New formulas taking idle energy consumption into account as proposed by Solution#11 described in clause 6.11.

* * * * End of Changes * * * *