SA WG2 Meeting #170	S2-2506690

25 – 29 Aug, 2025, Goteborg , SE

Source:	Qualcomm Incorporated

Title:	[WT#3, AI framework/Agentic AI] AI framework for 6G

Document for:	Agreement

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution discusses the AI framework and proposes the Key issue for WT#3.

# 1.	Justifications

## 1.1	AI framework

The AI function was discussed in different WGs for different use cases in RAN, CN and OAM in 5G. SA2 mainly focused on the model training and inference related to the core network. Considering that the use cases are different in RAN, CN and OAM, it is also beneficial to consider the AI framework that is used for the CN related use cases in SA2 also for 6G. The RAN or OAM related AI function should be discussed in other WGs.

Model training and inference architecture

The 5GC AI function was centrally supported by NWDAF. The NWDAF may contain an MTLF for model training or an AnLF for inference or both logical functions. The other 5GC entities, that act as consumers and request analytics results from an AnLF / NWDAF, take the received analytics results into account based on implementation, as shown in Figure 1.

Figure 1: AI function in 5GC

The LMF-based AI/ML positioning was supported in 5GC since Rel-19. In this case, the inference procedure for location calculation was always performed in LMF but the model training may be performed either in LMF itself or MTLF / NWDAF as shown in Figure 2. In this case, whether and how inferencing is used for location calculation performed in LMF is up to implementation and there is no need for standardization. Therefore, SA2 mainly focused on the model training and model provisioning procedure.

Figure 2: AI function for LMF based AI/ML positioning

In 6G, the core network entities may not only work as consumers to request the inference result from AnLF / NWDAF. Instead, the inference function maybe supported by the core network entities natively and used by the network functions themselves to make the decision based on local data. For example, the 6G-AMF like-function may decide the paging strategy based on local inferencing. The 6G-SMF-like function may decide the SM back off timer based on inferencing performed locally. In order to support these, it is also possible to perform the model training in the core network entities natively (3a), or in the centralized MTLF and distribute the trained model to different network functions (3b), based on different scenarios.

Figure 3: AI in 6G

Figure 3 illustrates the different scenarios:

-	For 3a, the model training and inference are the internal logic in the 6G entity, i.e., whether inferencing is performed is up to implementation of the CN entity and not subject to standards. The main standards work is the data collection for model training and inference, as what we did for case 2a in Rel-19.

-	Case 3b is similar to case 2b, the model provisioning supported by MTLF can be considered to be reused in 6G.

-	Case 3c is similar as 1a and the procedures defined in TS 23.288 can be reused in 6G.

Data Collection for model training and inference

The data collection procedure from different data sources to support model training and inference may mainly be discussed in WT#5: data framework, and we think the data collection procedure defined in TS 23.288 can also be reused. One enhancement compared to 5G is that, since the model training and inference are always performed in NWDAF, the NWDAF is always the data consumer for data collection in 5G. However, considering that we may also support cases 3a and 3b in 6G, other CN NFs may also act as data consumers.

For specific use cases, the input data list for model training and inference should be discussed in WT#3.

Performance Monitoring

The ML model and analytics performance monitoring in based on accuracy in 5G, the enhancement of performance monitoring is required to study for 6G AI.

Multi-vendor interoperation

The multi-vendor interoperation feature is supported in 5GC and the feature is also required to supported in 6G.

## 1.2	Support of agentic AI

During the approval of the SA2 SID on 6G [1] there has been a significant amount of discussion on agentic AI. From the author’s perspective it is essential to keep in mind that agentic AI (AI agents) is a solution that may be proposed for specific problems and use cases.

For this reason there is also no need to have a specific work task on agentic AI or AI agents. In other words, if companies consider agentic AI as an appropriate solution, e.g. for exposure, then companies are welcome to propose a solution based on agentic AI for the respective work task.

In addition, the following aspects are important to consider for any solution based on agentic AI:

-	Remain flexible on agentic AI protocols: Agentic AI and the underlying concepts and protocols are a very dynamically evolving area. While initial versions of protocols such the Model Context Protocol (MCP) [2] and the Agent2Agent (A2A) protocol [2] have been published, it is clear that these will continue to be extended or potentially even replaced since aspects like discovery, security, etc. have not been fully addressed. Discussions at the recent IETF#123 meeting for instance highlighted the need for further work in these areas, which may result in modified or even new protocols for agentic AI [4]. 

For the work on agentic AI in 3GPP this clearly suggests that it essential to remain flexible with respect to protocols for agentic AI. In other words, it is essential to avoid focusing solely on currently available protocols that may be outdated by the time the 6G system gets deployed and instead to aim for 6G solutions that allow for agentic protocols to evolve.

-	Ensure testability and predictability of the 6G system: The SA2 6G SID contains the following statement: “All work tasks aim at supporting multi-vendor interoperable interfaces.” This goal is particularly important in the context of agentic AI, which is inherently non-deterministic. Therefore it is essential to ensure that system procedures remain predictable and testable.

# 2	References

[1]	TSG SA: Study on Architecture for 6G System. SP-250806.

[2]	Model Context Protocol. https://github.com/modelcontextprotocol.

[3]	Agent2Agent protocol. https://github.com/a2aproject/A2A.

[4]	Summary of side meeting on “AI agents protocols” at IETF 123. https://mailarchive.ietf.org/arch/msg/agent2agent/MDU04p-GC54VnJmdkjfr6u3jG8c/

# 3	Proposal

**** First Change ****

# Annex A.X	 WT#3: Study how to support and enable an AI framework for 6G.

This work task will address:

-	The different architectures of ML model function and inference function in 6GC.

NOTE 1:	Whether and how to perform local inference by a 6G network function is an implementation decision and beyond the scope of 3GPP.

-	Whether and how to enhance the ML model provision procedure based on the corresponding procedures defined in TS 23.288 [x].

-	Whether and how to enhance the performance monitoring procedure based on the accuracy monitoring defined in TS 23.288 [x].

-	Whether and how to enhance the multi-vendor interoperability based on the corresponding mechanism defined in TS 23.288 [x].

NOTE 2:	Data collection as defined in TS 23.288 is assumed to be sufficient for model training. Allowing network functions other than NWDAF to act as a consumer for data exposed by other network functions, e.g. for the purpose of model training, is not precluded.NOTE 3:	Agentic AI/AI Agent is a solution that may be proposed for specific problems and use cases. For this reason, agentic AI is not covered by this WT. Instead, agentic AI-based solutions may be proposed as solutions for other WTs if deemed appropriate.

NOTE 4: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

**** Second Change ****

# 5.X. Key Issue #X: AI framework for 6G

This key issue will address:

-	The different architectures of ML model function and inference function in 6GC.

NOTE 1:	Whether and how to perform local inference by a 6G network function is an implementation decision and beyond the scope of 3GPP.

-	Whether and how to enhance the ML model provision procedure based on the corresponding procedures defined in TS 23.288 [x].

-	Whether and how to enhance the performance monitoring procedure based on the accuracy monitoring defined in TS 23.288 [x].

-	Whether and how to enhance the multi-vendor interoperability based on the corresponding mechanism defined in TS 23.288 [x].

NOTE 2:	Data collection as defined in TS 23.288 is assumed to be sufficient for model training. Allowing network functions other than NWDAF to act as a consumer for data exposed by other network functions, e.g. for the purpose of model training, is not precluded.

NOTE 3:	Agentic AI/AI Agent is a solution that may be proposed for specific problems and use cases. For this reason, agentic AI is not covered by this WT. Instead, agentic AI-based solutions may be proposed as solutions for other WTs if deemed appropriate.

NOTE 4: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

**** End of Changes ****