3GPP TSG-SA2 Meeting #170	S2-2506715

Goteborg, Sweden, 25 – 29 August, 2025

Source:	Samsung

Title:	Update on Solution #3: IP Type IMS PDN connection over User Plane with NIDD voice transfer between UE and PGW

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: The contribution proposes to clarify overhead reduction for NIDD voice transfer between UE and PGW.

Discussion

The proposed solution is updated to solve some ediotr’s note. Especially for how to convert the RTP/UDP/IP header from the short brand-new header, which is not only cover RTP header only but also UDP/IP header information, which is used to retrieve the original RTP/UDP/IP header. and such context information for coversion can be from UE via PCO also.

Additionally, it is expected that a dedicated voice codec is used over the GEO satellite b/w UE and NW, which can be assumed that voice data transcoding is normally required in this situation and IMS-AGW is used for that. So, the IMS-AGW can be a better entity for header conversion together in place of PGW. In this case we assume that a tunnel between UE and IMS-AGW is pre-configured.

Proposal

It is proposed to add this solution for KI#1 in the TR 23.700-19.

Start of 1st Change

6.3	Solution #3: IP Type IMS PDN connection over User Plane with NIDD voice transfer between UE and PGW

6.3.0	High-level solution Principles

This solution is to address Key Issue #1.

This solution uses a single IMS APN PDN connection with two EPS bearers. The IMS PDN is IP type (e.g. IPv4, IPv6 or IPv4v6) PDN Connection. This solution is a User Plane based solution, i.e. both the default bearer and the dedicated bearer are using S1-U data transfer.

In order to reduce the header overhead to the most extreme level:

-	it is proposed to transmit the IMS voice traffic as non-IP (NIDD) traffic over dedicated EPS bearer; and

-	the PGW will translate the NIDD voice packets to IP/UDP/RTP packets before forwarding them to the IMS eneity in the DN, e.g. the AGW (AGW is not aware of the NIDD transport between PGW and UE); and

at the same time in order to reduce the impact to the IMS control plane ("I"MS network is designed based on IP) and IMS entity in the IMS DN, e.g. P-CSCF;

-	it is proposed to transmit the IMS Signalling (e.g. SIP INVITE ... etc.) over default EPS bearer using IP.

6.3.1	Description

Figure 6.3.1-1: IP Type IMS PDN connection over User Plane with NIDD voice transfer between UE and PGW

NOTE 1	Assuming data rate is 1Kbps and voice codec rate is 0.7Kbps, then there is only 0.3Kbps remaining for voice packet "header" overhead. Assuming ptime = 80ms, which means there will be 12.5 voice packets per second to consume the 0.3Kbps remaining bandwidth, which means each voice packet header part can only use 300bits/12.5 == 24 bits, i.e. each voice packet can only tolerate maximum 3 Bytes header size, these overhead are shared by multiple layers including RTP/UDP/IP/NAS(if go with CP)/RRC(if go with CP)/PDCP(if go with UP or go with CP after DRB is established)/RLC/MAC headers.

The IMS PDN (for voice over GEO) is used to transmit both IMS signalling and IMS voice.

-	For IMS signalling:

-	the main consideration is to minimize the impact to the IMS control plane network ("I"MS network is designed based on "IP") so the default EPS bearer for IMS PDN connection is configured as IP transmission (i.e. IMS Signalling over IP, no special handling).

-	For voice packets, on the other hand, it is proposed to use non-IP (NIDD) type transmission.

Editor's note:		In the context of using "Non-IP" voice packet transfer, only one call or more than one call needs to be supported is FFS based on SA WG1 discussion.

Editor's note:		In the context of using "Non-IP" voice packet transfer, what are the essential RTP header fields for the minimum information that need to be provided in RTP for IMS voice over GEO is FFS based on SA WG4 discussion.

Thus the ideal IMS PDN configuration for IMS voice over GEO would be:

-	the PDN Type for this IMS PDN is IP Type; and

-	a dedicated EPS bearer is configured to use NIDD to transmit voice packets.

6.3.2	Procedures

6.3.2.1	Establishment and Usage of IP Type IMS PDN connection over User Plane with NIDD voice transfer between UE and PGW

In order to support:

-	IP Type IMS PDN connection over User Plane with NIDD voice transfer between UE and PGW, which support to use:

-	IP (IP(v4), IPv6 or IPv4v6) type traffic for default EPS bearer; and

-	non-IP type traffic for dedicated EPS bearer;

the UE supports the this feature indicates support of "IP and non-IP Hybrid Transfer" in UE network capability in Attach request, and the MME supports the "IP and non-IP Hybrid Transfer" indicates to accept to use it in Attach accept; and during Dedicated EPS bearer context activation procedure the network indicates, for a dedicated bearer to be activated for voice, the traffic type to be sent over the bearer is non-IP traffic.

6.3.2.2	NIDD voice transfer between UE and PGW

6.3.2.2.1	Uplink Voice Data Transmission using NIDD over dedicated EPS bearer

NIDD voice transfer between UE and PGW is can be done by missing or reducing the fixed or duplicated or tracackable field of RTP/UDP/IP header in the new packet header. So, the PGW can retrieve the RTP/UDP/IP packets from the NIDD packet based on the new packet header. For example, some fixed information can be configured between UE and P-GW via PCO of SM NAS messages or the very initial voice data packet e.g. source/destination UDP port info, UDP packet length(i.e. fixed for voice over NTN GEO) IP version, header length of IP packet, total length of IP packet(i.e. fixed for voice over NTN GEO), source/destination IP address, RTP payload type, SSRC id, etc.

6.3.2.2.1.1	UE side consideration

For the uplink voice packets to be sent over a dedicated bearer using NIDD, i.e. without IP/UDP header the existing Packet filter component type identifier (as specified in TS 24.008 [13] clause 10.5.6.12) is not able to indicate the "voice traffic" needs to be steered to the dedicated EPS bearer context, thus a new Packet filter component type identifier may be introduced, e.g. "(IMS) voice traffic type", to indicate the UE to steer NIDD voice traffic to the dedicated EPS bearer context. The UE sends the "Voice with minimum header info (e.g. 1 Byte headerincluding short RTP SN) packet" over the dedicated EPS bearer context associated with this "(IMS) voice traffic type" Packet filter component type identifier.

6.3.2.2.1.2	Network side consideration

When the PGW receives uplink voice packets from the dedicated EPS bearer, the PGW converts the "Voice with minimum headerinfo (e.g. 1 Byte header including short RTP SN) packet" to "Voice over RTP/UDP/IP packet", where PGW can track the original RTP SN by the delivered short lsb RTP SN based on keeping and tracking the context for whole RTP SN value.

In order to build the context of the conversion operation, when the PDN GW triggers the dedicated EPS bearer creation procedure as specified in step 2-11 in clause 5.4.1 of TS 23.401 [5], the PGW also builds and stores the context (e.g. IP(es) and Port(s)), based on information received from PCRF or PCO from UE or the very intial voice packet, in order to perform the conversion, i.e. to construct the RTP/UDP/IP headers for the received uplink voice packet and then forward the conversed voice packets (with RTP/UDP/IP header) to the DN.

As an alternative way, instead of PGW, IMS-AGW can perform conversion of packet header from the minimum header to RTP/UDP/IP header of the voice data packets, where the IMS-AGW may obtain the context information for conversion from P-CSCF(including IMS-ALG) and it is assumed that a tunnel for NIDD data delivery is pre-configured between PGW and IMS-AGW.

6.3.2.2.1	Downlink Voice Data Transmission over NIDD over dedicated EPS bearer

For the downlink voice packet, the PGW may still leverage the current DL TFT as usual, but for DL traffic steered to the NIDD dedicated EPS bearer the PGW converts the DL Voice over RTP/UDP/IP packets to Voice with minimum header info (e.g. 1 Byte header including short RTP SN) packet. The conversion can be simply removing the RTP/UDP/IP headers and insert 1 Byte header including short RTP SN (e.g. use the short RTP SN value originally indicated in the removed RTP header) as the converted header part of the voice packet before forwarding the voice packet to SGW, where UE can track the original RTP SN by the delivered short lsb RTP SN based on keeping and tracking the context for whole RTP SN value.

As an alternative way, instead of PGW, IMS-AGW can perform conversion from RTP/UDP/IP header of the voice data packets to the minimum header, where the IMS-AGW may obtain the context information for conversion from P-CSCF(including IMS-ALG) and it is assumed that a tunnel is pre-configured between PGW and IMS-AGW.

6.3.3	Impacts on Services, Entities and Interfaces

Editor's note:	Whether a new GBR or non-GBR QCI is needed for the dedicated EPS bearer used for voice traffic is FFS. Whether an existing GBR or non-GBR QCI can be used for the dedicated EPS bearer used for voice traffic is FFS.

UE:

-	support dedicated EPS bearer for NB-IOT.

-	indicate support of "IP and non-IP Hybrid Transfer" to the MME.

-	new Packet filter component type identifier, e.g. "(IMS) voice traffic type", to let the UE indicated to steer NIDD voice traffic to the dedicated EPS bearer context.

-	transfer IMS voice packet over dedicated bearer without adding IP/UDP/RTP header.

-	transfer IMS voice packet over dedicated bearer with minimum header information in the header (e.g. 1 Byte header including short RTP SN).

MME:

-	indicates support of "IP and non-IP Hybrid Transfer" to the UE.

RAN:

-	support dedicated EPS bearer for NB-IOT.

-	support NIDD bearer for the "IP and non-IP Hybrid Transfer" PDN connection.

Editor's note:	Detail impacts on RAN, e.g. whether and how to enable/disable RoHC in the RAN for the voice bearer, is FFS.

PGW:

-	support dedicated EPS bearer for NB-IOT.

-	new Packet filter component type identifier, e.g. "(IMS) voice traffic type", to indicate the UE to steer NIDD voice traffic to the dedicated EPS bearer context.

-	for uplink voice packets, convert the NIDD voice packets into IP packets before forwarding the packets the IMS DN.

-	for downlink voice packets, convert the IP/UDP/RTP voice packets into NIDD voice packets before forwarding the packets to the UE.

-	building conversion parameters (e.g. source IP, destination IP, source port, destination port) for voice packets by leveraging existing UL and DL TFT parameters and PCO used/negotiated during dedicated EPS bearer creation/modification procedure or from the very initial voice data packet.

IMS-AGW:

-	it works only when header conversion between RTP/UDP/IP header and minimum header in place of PGW.

End of Change