3GPP TSG-SA2 Meeting #170	S2-2506839

# Goteborg, Sweden, 2025-08-25 – 2025-08-29

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0025 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PLMN terminology alignment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In SA#108, SA sent an LS (SP-250852) to SA2, SA3 and CT1, which provide a guidance for AIoT device credentials storage which is included in SP-250851: 1) for Rel-19, the AIoT system is defined as private network (isolated network deployment that does not interact with a public network) e.g. SNPN, and the AIoT device credentials storage follows 3GPP defined requirements, the exact mechanism is out of scope of 3GPP (similar to Annex I.2.2 of TS 33.501). This bullet means that no interconnection exists between AIoT systems and PLMNs ; 2) For Rel-20, if the AIoT system is defined as public network i.e. PLMN, the AIoT device credentials storage shall use UICC. NOTE 1: Revisiting the above requirements in bullet 1 and/or 2 can be anticipated in future releases following the normal working procedures including in Rel-20. NOTE 2: In case UICC is used, the exact form factor and whether it is removable, non-removable or integrated is out of scope of 3GPP. Based on the guidance provided by SA, in Rel-19, the AIoT system is defined as an SNPN without PLMN interaction. This guidance needs to be reflected in TS 23.369 TS 23.369 mentions PLMN ID multiple times, with the SA guidance, only SNPN is in scope. Hence it is proposed to change PLMN ID into SNPN ID as no PLMN is involved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the guidance from SA which impacts TS 23.269. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Stage 2 specification is not aligned with SA guidance - Network identifier format not aligned with current specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.2, 5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>>BEGINNING OF CHANGES <<<<

### 5.7.2	AIoT Device Permanent Identifier

In order to support the AIoT feature in 5G System, a globally unique AIoT Device Permanent Identifier shall be allocated to each AIoT Device. An AIoT Device Permanent Identifier is assigned either by an operator or by a third party. The AIoT Device Permanent Identifier is used to identify an AIoT Device and locate the entity where the AIoT Device related information is stored.

NOTE 1:	How to configure an AIoT Device with the AIoT Device Permanent Identifier is out of scope of this specification.

The AIoT Device Permanent Identifier includes the following components:

-	The ID Type, including:

-	Information indicating whether a SNPN ID i.e., PLMN and NID are included.

-	Information indicating whether a third party identifier is included.

-	Identification Information Type, indicating whether the Identification Information contains an  EPC or unstructured information.

-	The Domain Information includes none, one or more of the following:

-	An SNPN Identifier (i.e., MCC and MNC with NID) as specified in TS 23.003 [6] clause 12.7 when the information in the ID type indicates it is included. Self-assignment NID model should not be used in order to achieve global uniqueness.

-	A third party identifier used to identify a third party when the information in the ID type indicates it is included.

-	The Identification Information is used to distinguish different AIoT Devices within the scope identified by Domain Information (if available) and can contain either:

-	An EPC, as defined in clause 14 of GS1 TDS Release 2.1 [7].

-	Unstructured information, where the content is defined by the allocator.

Figure 5.7.2-1: AIoT Device Permanent Identifier Structure

An operator allocated AIoT Device Permanent Identifier should include the identifier of the network for the operator. The identifier of the network is present as either a PLMN Identifier, NID or both in the AIoT Device Permanent Identifier.

A third party allocated AIoT Device Permanent Identifier may include none of the following information or include any combination of at least one kind of the following information: an SNPN ID or the third party identifier.

NOTE 3:	The length of ID Type, SNPN ID (if present) and the third party identifier (if present) components is fixed. The length of the Identification Information is variable. The details are specified in TS 23.003 [6] and TS 29.xxx [xx].

Editor's note:	The reference in NOTE 3 needs to be updated, when the appropriate stage 3 document is identified.

NOTE 4:	When the Domain Information is empty, the AIOTF uses, e.g., Identification Information (i.e EPC) to discover and select the ADM instance or the external server for the AIoT Device Profile Data.

The following lengths are supported for the Identification Information in an AIoT Device Permanent Identifier: 96 bits, and 128 bits.

NOTE 5:	The encoding for the length of the Identification Information enables additional shorter or longer fixed lengths to be supported in the future.

## 5.8	Filtering Information

The filtering information is used to identify or filter multiple AIoT Devices and is constructed by one or multiple components(i.e. ID Type, SNPN ID, third party identifier and Identification Information) of the AIoT Device Permanent Identifier as defined in clause 5.7.

The filtering information includes a list of filtering elements. A filtering element includes:

-	Information indicating a component of the AIoT Device Permanent Identifier that is used to match the bitstring.

-	A bitstring which is used to compare with the component.

-	An Offset from the beginning of the indicated component, which indicates the start location in the indicated component to be used to compare with the corresponding bitstring.

When the indicated component is SNPN ID or Third Party Identifier, the Offset is not included in the filtering information, as the whole component is compared with the bitstring, otherwise the Offset is always included.

There may be multiple filtering element within the Filtering Information for the Identification Information.

Each bitstring is corresponding to one component of the AIoT Device Permanent Identifier.

NOTE 1:	It is assumed that the AIOTF includes at most one filtering element information within the Filtering Information for ID Type, SNPN ID or Third Party Identifier. It is assumed that the AIoT Device does not validate the number of filtering element(s) for any indicated component within the Filtering Information.

NOTE 2:	The maximum number of the filtering element for Identification Information is left to stage3.

Editor's note:	Whether and how to secure the filtering information is up to SA WG3.

Figure 5.8-1: The example of the comparing the bitstring to the component

To determine whether an AIoT Device Permanent Identifier matches the filtering information, it is compared with every filtering element information within filtering information by comparing the whole bitstring within a filtering element information with the indicated component of its AIoT Device Permanent Identifier. If all the compared bitstrings match the AIoT Device Permanent Identifier then an AIoT Device Permanent Identifier matches the filtering information. If an AIoT Device Permanent Identifier does not contain an indicated component then it does not match the filtering information.

>>>>END OF CHANGES<<<<