3GPP TSG-SA2 Meeting #170	S2-2506870

Goteburg, Sweden, 25th – 29rd August,  2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1507 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cleanup of SCP signaling in 23.288 6.22.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Oracle |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-08-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is a need to clean up the SCP signaling in 23.288 Table 6.22.2-8 – both editorial and content. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clean up the SCP signaling in 23.288 Table 6.22.2-8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SCP signaling in 23.288 Table 6.22.2-8 will have some vaugue text as well as missing parameter(s). Consequently, CT3 will not be able to properly update TS 29.570 (SCP services stage 3). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.22.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.22.2	Input data

The NWDAF collects the UE related context data, NF context, AF data, MDAF data and SCP signaling information as listed in Tables 6.22.2-1, 6.22.2-2, 6.22.2-3, 6.22.2-4, 6.22.2-5, 6.22.2-6 and 6.22.2-7.

The NWDAF can collect UE behavioural information per UE or group of UEs as specified in clause 6.7.5.2.

The NWDAF may use output of abnormal UE behaviour analytics and UE dispersion analytics as specified in clause 6.7.5 and clause 6.10.3 as input data for analytics on signalling storm. This information may be collected from a different NWDAF (e.g. distributed architecture).

The NWDAF may collect a subset of input data based on implementation.

Table 6.22.2-1: UE related Context Data collection

| Information | Source | Description |
| --- | --- | --- |
| UE ID |  | Identifies a single UE. |
| Signalling feature data |  | NF procedures containing signalling exchange information related to a particular UE or session from the Connection, Registration, Mobility and Session Managements procedures (NOTE 1). |
| > Request type and number from UE/RAN (0..max) | AMF | Request type of N1 or N2 interface (such as received Initial Registration Request, Mobility and Periodic Registration Request, Service Request, etc.) and the number of requests corresponding to the request type. |
| >> Time duration from receiving request from UE/RAN to response to UE/RAN | AMF | Time duration between the request from UE/RAN and response to UE/RAN. |
| >> Number of successful responses of UE/RAN | AMF | Number of successful responses associated to their initial requests, such as Registration Response, etc. |
| >> Number of failed responses of UE/RAN | AMF | Number of failed responses associated to their initial requests, such as Registration Reject, Service Reject, etc. |
| >> Reason of failed responses of UE/RAN | AMF | Reasons of failed responses associated to their initial requests, e.g. reject, no-response, etc. |
| >> A posterior Request type of UE/RAN (0..max) | AMF | A posterior Request types triggered from UE/RAN, for NF Service request, or request to UE/RAN (NOTE 3). |
| > Request type and number from NF (0..max) | NF | Request type (i.e. identification of NF service instance) received from NF, e.g. Namf_N1N2Trans, Namf_comm, etc., as well as the number of requests received from NF (NOTE 2) (NOTE 3). |
| >> Time duration from sending a request to receiving NF to response from the NF | NF | Time duration between the request from an NF and response to the NF. |
| >> Number of successful responses from NF | NF | Number of successful responses associated to their initial requests (NOTE 2). |
| >> Number of failed responses from NF | NF | Number of failed responses associated to their initial requests (NOTE 2). |
| >> Reason of failed responses from NF | NF | Reasons of failed responses associated to their initial requests, e.g. reject, no-response, etc. |
| >> Number of redundant signalling of NF | NF | Number of received redundant signalling. The redundant signalling means the signalling which is transmitted multiple times (NOTE 2) (NOTE 3). |
| >> A posterior Request type of NF (0..max) | NF | A posterior Request types triggered from NF, for NF Service request (NOTE 3) (NOTE 4). |
| > Public Warning information | AMF | Public Warning information as defined in the TS 23.041 [52], such as WRITE-REPLACE WARNING REQUEST, etc. |
| > Frequent Mobility Registration Update | AMF | The number of Mobility Registration Updates N within a period M may be an indication for abnormal ping-pong behaviour, where N and M are operator's configurable parameters. |
| > Number of receiving Session Report from UPFs | SMF | Number of receive Session Report from UPF triggered by DL packet in case of PDU Session is in 5GCM-idle state. |
| UE Context in NF |  |  |
| > State transition information | AMF, SMF | UE related state transition information such as transition type, frequency of CM state changes, etc. State transition identifier: - "Access Type change to 3GPP access"; - "Access Type change to non-3GPP access"; - "RM state change to RM-DEREGISTERED"; - "RM state change to RM-REGISTERED "; - "CM state change to CM-IDLE"; - "CM state change to CM-CONNECTED"; - "Handover"; - "Mobility Registration Update"; or - "Frequent Mobility Registration Update"(table 6.7.2.2-1). Or, PDU Session related state transition information such as transition type, frequency SM state changes, etc. State transition identifier: - "PDU Session Establishment"; - "PDU Session Release"; - "Communication failure"; or - "PLMN change". It can be reported for a group of UEs (e.g. the number of total transitions or percentage of the group UEs who have transitions). |
| > timer information | AMF, SMF | Timer information which has been set for the UE, such as timer type, duration. |
| NOTE 1: NWDAF can optionally provide transaction dispersion analytic information for MM and SM transactions. NOTE 2: This data is provided per time interval that is indicated by the NWDAF. NOTE 3: As a centric routing/proxy entity, SCP might be involved with some signalling that may not be visible to the NF sending the SCP's ingress traffic, such a signalling could be associated with actions such as selection & discovery, retransmissions, re-directions, re-selection, throttling, etc. NOTE 4: A posterior request is a request sent from NF1 to NF2 (or from NF2 to NF1) that was triggered by a previous request received by NF1. For example, in clause 4.3.2.2 of TS 23.502, the request sent by AMF in step 3 and the request sent by SMF in step 11 are both posterior requests of the request received by the AMF in step 1. These posterior requests can be collected by NF1 (the AMF in this example). |  |  |

Table 6.22.2-2: NF Context Data collection

| Information | Source | Description |
| --- | --- | --- |
| NF instance ID |  | NF instance ID |
| > NF profile | NRF | NF Profile information such as allowed NF information for the NF and NF Service(s), NF capacity, NF load, NF priority, NF service instance capacity, NF service instance load, NF service instance priority. |
| > NF load information | NRF | Load information indicates the current load of the NF and NF Service(s) (NOTE 1). |
| NF heart-beat related information | NF, OAM | NF heart-beat related information such as responding time, Number of retransmissions, heart-beat intervals. |
| > Unexpected operational status indicator | NF, OAM | The parameter indicates unexpected operational status occurs. It is an indication of deviation from the normal operations, based on thresholds or rules configured by operator at the NF or OAM (e.g. for energy consumptions, loads, etc.). |
| NOTE 1: If this information is not available in the NRF, it can be obtained from the OAM. |  |  |

Table 6.22.2-3: NF Specific Data collection

| Information | Source | Description |
| --- | --- | --- |
| NF instance ID |  | NF instance ID |
| > Usage information of UE IP address resources | SMF | Usage information of UE IP address resources (dynamic and static, V4, V6, etc.) for CP or UP allocation, such as number, usage, number of UE IPs, which prohibit allocation during certain time interval, etc. This parameter is valid for SMF only. |
| > Load information of connected UPFs | SMF | Load information of connected UPFs such as using PFCP Load Control Information. This parameter is valid for SMF only. |

Table 6.22.2-4: Application activation time information

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | Identifies the application providing this information. |
| User activation time information (1…max) | AF | Information of activation time for the users (e.g. IoT users) per application. |
| > Number of UEs | AF | The total number of UEs |
| > Active Time | AF | The time stamp of the users per application switch to active, or the start and end time of the users activity per application. |
| > Inactive Time | AF | The time stamp of the users per application switch to inactive, or the start and end time of the users inactivity per application, if applicable. |
| > UE type ID | AF | Identifies a group of UEs, e.g. external group ID, or a list of UE IDs. |

Table 6.22.2-5: Data collection from OAM related to NRF operations as defined in clause 5.10 of TS 28.552 [8]

| Information | Source | Description |
| --- | --- | --- |
| NRF ID |  | NRF instance ID. |
| Number of NF service registration requests | OAM | Number of registration request received at the NRF. |
| > Number of successful NF service registrations | OAM | Number of successful registrations. |
| > Number of failed NF service registrations due to encoding error of NF profile | OAM | Number of failed registrations. |
| > Number of failed NF service registrations due to NRF internal error | OAM | Number of failed registrations. |
| Number of NF service update requests | OAM | Number of update request received at the NRF. |
| > Number of successful NF service updates | OAM | Number of successful updates |
| > Number of failed NF service updates due to encoding error of NF profile | OAM | Number of failed updates. |
| > Number of failed NF service updates due to NRF internal error | OAM | Number of failed updates. |
| Number of NF discovery requests | OAM | Number of discovery request received at the NRF. |
| > Number of successful NF discoveries | OAM | Number of successful discovery attempts. |
| > Number of failed NF service discoveries due to unauthorized NF Service consumer | OAM | Number of failed discovery attempts. |
| > Number of failed NF discoveries due to input errors | OAM | Number of failed discovery attempts. |
| > Number of failed NF discoveries due to NRF internal error | OAM | Number of failed discovery attempts. |

Table 6.22.2-6: Data collection from MDAF/MDAS of control plane congestion analysis as defined in clause 8.4.7.1.3 of TS 28.104 [45]

| Information | Source | Description |
| --- | --- | --- |
| affectedObject | MDAF | Indication of 5GC NFs where congestion issues occurred or potentially may occur. |
| cPCongestionIssueID | MDAF | This field holds the ID of the control plane congestion issue which is reported. |

Table 6.22.2-7: Data collection from MDAF/MDAS of fault information as defined in clause 8.4.3.1.3 of TS 28.104 [45]

| Information | Source | Description |
| --- | --- | --- |
| failurePredictionObject | MDAF | Indication of NFs where the failure related issues occurred or potentially occur. |
| potentialFailureType | MDAF | This field holds the ID of the control plane congestion issue which is reported. |
| eventTime | MDAF | This field holds the time of potential failure predicted |
| issueID | MDAF | This filed holds the ID of this failure prediction which is reported. |
| perceivedSeverity | MDAF | This field holds the value to indicate relative level of urgency for operator attention. |

Table 6.22.2-8: SCP Signalling Infomation

| Information | Source | Description |
| --- | --- | --- |
| NF instance ID of a consumer or/and NF instance ID of a producer |  | NF instance ID(s) of the service producer or/and consumer. |
| > Number of requests sent to an NF Service instance | SCP | Number of requests received by the SCP for an NF service instance producer, such as Nudm-sdm, Namf-loc, etc. |
| > Number of requests sent by an NF instance | SCP | Number of requests received by the SCP from an NF instance consumer. |
| > Number of successful responses of different NF Services of the NF instance | SCP | Number of successful responses received from NF service instance producer, such as Nudm-sdm, Namf-loc, etc. |
| > Number of total failure responses of different NF Services of the NF instance | SCP | Number of total failure responses received from NF service instance producer of different NF Services, such as Nudm-sdm, Namf-loc, etc. |
| > Number of failed responses of a specific NF Service of the NF instance | SCP | Number of failed responses received from NF service instance producer of a specific NF Service, such as Nudm-sdm, Namf-loc, etc. |
| > Reason of failed responses of different NF Services of the NF instance | SCP | Reasons of failed responses received from NF service instance producer of different NF Services, e.g. reject, no-response, etc. |
| > Number of failure responses SCP generated towards consumer NF instance. | SCP | Number of total failure responses generaterd by SCP towards request sender NF instance i.e., consumer NF. |
| > Reason of failure responses SCP generated towards consumer NF instance. | SCP | Reasons of failed responses of SCP generated towards request sender NF instance i.e., consumer NF instance, e.g. target nf not reachable, no-response, etc. |
| > Number of times SCP reselect NF instance that is originally destined to that NF instance producer | SCP | Number of times the SCP has reselect NF instance producer, it happens when e.g. SCP realizes that the original NF instance is not reachable. |
| > Reason of SCP reselection of NF instance | SCP | Reason of SCP reselection of NF instance producer as defined in clause 6.3.1.0 of TS 23.501 [2]. |
| > Status of the connection to the NF instance | SCP | The connection status (i.e. ACTIVE or INACTIVE) between SCP and NF instance. |
| > Average time duration from sending request to the NF instance to receiving response from the NF instance. | SCP | Average time duration between SCP sending the request to the NF instance and SCP receiving the response from the NF instance. |

# * * * * End of changes * * * *