3GPP TSG-WG Meeting #170	S2-2507194

Goteborg, SE, 25-29 Aug, 2025

Source:			CATT

Title:			WT#2, Migration and Interworking, and study proposal

Document for:			Approval

Agenda item:			20.6.2

Work Item / Release:			FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes the study of migration and IWK under WT#2.

1．Introduction

Migration and IWK with the legacy 5GS and/or 4G/EPS are fundamental for the operator and user to maintain seamless communication services between 5GS/EPS and 6GS.

2. Gap Analysis

From the 4G/EPS migration to the 5GS, the collocated HSS+UDM, PCF, SMF+PGW-C, and UPF+PGW-U are shared by the EPS and 5GS. There are numerous collocated network nodes between the EPS and 5GS, which introduce significant changes to the EPS network deployments. How to minimize the number of collocated network nodes between the 5GS and 6GS to support migration from 5GS to 6GS needs to be studied.

To support IWK between EPS and 5GS, the N26 interface between the 4G MME and 5G AMF is defined to transfer and map the UE Context between the EPS and 5GS. With the N26 interface, and UE Context transferring and mapping, the single registration mode can be supported which is simply implemented in the UE. However, if there is no N26 interface, the UE can register to both EPS and 5GS and can keep the EPS and 5GS context separately. During the UE moves back and forward between the EPS and 5GS, the UE can use the previously stored context to decrease the time of the TAU and PDU Session switching. However, keeping both types of UE context in the UE makes the UE implementation complex and expensive (and fewer types of UE support dual registration?), and the time of PDU Session switching does not decrease so much. How to support a N26-like interface between the 5GS and 6GS, and how to support single registration with a single USIM, needs to be studied.

Still, a lot of operators have a big EPS network with no or a small 5GS network, whether and how to support EPS migration to the 6GS, especially to support the VoIMS over EPS seamless handover to 6GS, is fundamentally important for the operators and users.

A lot of operators (only) provide 4G/5G NTN/satellite access; these 4G/5G NTN/satellite access subscription users are critically important for the operators. How to support interworking between 6GS and 4G/5G NTN/satellite access that uses EPS/5GS needs to be studied.

The 5GS/EPS and 6GS can be within the same PLMN, or between different PLMNs of the same operator, or between different PLMNs of different operators. All the migration and IWK in these scenarios need to be studied.

3．Proposed Changes

* * * First Change * * * *

Annex A.X. WT#2: Study migration and interworking

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

Study migration and interworking, including

- How to support migration to 6GS

- How to support interworking with 5GS

- Whether and how to support interworking with EPS

- How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

NOTE 2: Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 3:  The detailed migration study scope will be coordinated and aligned with RAN.

* * * Next Change * * * *

5.X. Key Issue #X (WT#2): Study migration to 6GS and interworking with 5G/EPS

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

This key issue (KI) will study migration to 6GS and interworking with 5GS/EPS for the cases where the 5GS and 6GS are within the same PLMN, different PLMNs within the same operator, or different PLMNs between different operators.

- How to support migration to 6GS with minimized the co-located NF(s) between the 6GS and 5GS, e.g. with single USIM and single registration in 5GS/6GS.

- How to support interworking with 5GS with or without N26 like interface.

- Whether and how to support interworking with EPS, e.g., the VoIMS service from/to EPS.

- How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

NOTE 2: Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 3:  The detailed migration study scope will be coordinated and aligned with RAN.

* * * End of Changes * * * *