3GPP TSG-WG SA2 Meeting #170	S2-2507270

# Goteborg, SE, 25th Aug – 29th Aug, 2025		(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0058 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add AF Authorization Data storage into UDR functionality |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | UDR description only supports one functionality “Storage of AIoT device profile data”. It is observed that clause 5.6 “AF authorization for the AIoT Services” introduces another type of Ambient IoT data “AF Authorization Data”, and the NF can use this type of data can be either the ADM or AIOTF. In general, Ambient IoT data is considered as new dataset stored in the UDR, in addition to subscription data, policy data, structured data for exposure, and application data. ADM can access Ambient IoT data from UDR. In detail, Ambient IoT data include the following two sub-dataset: - AIoT device profile data. - AF authorization data |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update clause 4.5.8 UDR functionality to store Ambient IoT data - Storage of Ambient IoT data, including - AIoT device profile data. - AF authorization data |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear what type of data the AF Authorization Data belongs to. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.8, 7.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.5.8	UDR

In addition to the functions defined in TS 23.501 [3], the UDR may support the following functions:

-	Storage of Ambient IoT data, including:

-	AIoT device profile data;

-	AF authorization data.

# * * * * Second change * * * *

7.X	UDR Services

7.X.1	Nudr_DataManagement (DM) Service

7.X.1.1	General

The ADM makes use of the Nudr_DM service to access Ambient IoT data, in order to perform the authentication for AIoT Device, or authorizaiton of the AF request for AIoT services.

Ambeint IoT data keys are defined in Table 7.X.1.1-1.

Table 7.X.1.1-1: Ambient IoT data stored in the UDR

| Category | Information | Description |
| --- | --- | --- |
| AIoT device profile data | Last known AIOTF information | Indicate the last known AIOTF that serves the AIoT device, or unknown |
| AF authorization data | Allowed area | Indicate the allowed area for the indicated AF to perform the AIoT services operations. |
|  | Allowed service operations | Indicate the allowed service operation (s) for the indicated AF, e.g. inventory, read, write, permanent disable. |
|  | Allowed target AIoT Devices | Indicate the allowed AIoT Device(s) for the indicated AF. |

Table 7.X.1.1-2: Data keys

| Data Set | Data Subset | Data Key | Data Sub Key |
| --- | --- | --- | --- |
| Ambient IoT Data | AIoT device profile data | AIoT Device ID |  |
|  | AF authorization data | AF ID |  |

# * * * * End of changes * * * *