SA WG2 Meeting #171	S2-2508283

13 – 17 October 2025, Wuhan, China

Source:	LG Electronics

Title:	KI#1: Interim agreements

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This paper proposes interim agreements for KI#1.

Discussion

For KI#1 "System Architecture to Support Sensing", 12 solutions have been proposed. An analysis and summary of these solutions are provided in the following table.

| Sol.# | Sensing System Architecture | New vs. Existing Sensing Functions | SE-SF interaction |
| --- | --- | --- | --- |
| #1 | Direct SF–SE interface introduced | Unclear (maybe a possible new or existing sensing function) | Point-to-point direct connection between SF and SE |
| #3 | Sensing framework with AF-NEF–SF–SE baseline; Optional sensing function split (SCF, SPF) | New sensing function (optionally split into SCF and SPF) | Unclear (maybe a direct connection between SF and SE?) |
| #4 | Distributed sensing management per coverage; NEF handles service provisioning/exposure | New Sensing Management Function (SeMF) | Sensing data collection via CP (N1/N2) or UP (N3) |
| #5 | SF-AMF-SE framework; AMF selects SEs and forwards requests; SF selects AMF(s) and processes sensing data | Unclear (maybe a new sensing function) | Indirect via AMF (N1/N2 enhancements needed) |
| #6 | Sensing NF manages services per sensing service area | New sensing function; optional Gateway Sensing NF | Unclear (maybe a direct connection between SF and SE); sensing NF collects sensing data via CP/UP |
| #7 | SSMF-AMF-SE framework; SSMF includes SCF and re-used DCCF | New sensing service management function (SSMF) (with SCF + reused DCCF components) | Indirect via AMF over N2 (NGAP) |
| #19 | Generic sensing request/exposure procedure; sensing management and sensing processing split | New sensing management function (SeMF); new sensing processing function (SPF); new sensing policy and authorization function (SPAF) | Direct connection between SE and SPF; sensing data processed either at SPF or at SE; |
| #27 | Sensing control function and sensing processing function split; supports on-demand, periodic, or event-based result exposure | New sensing function (with SCF + SPF role) | Direct connection between SF and SE; |
| #28 | Sensing framework with AF-NEF-SnCF-NG-RAN flow | New sensing core function (SnCF); | Direct connection between SnCF and NG-RAN; NG-RAN provides measurements or results exposure to SnCF; |
| #31 | Dual-layer approach (central GSMF, distributed SSMF); using sensing result storage NF | New gateway sensing management function (GSMF); new sensing service management function (SSMF); new storage NF | Unclear (maybe a direct connection between SSMF and SE); GSMF handles exposure via CP or data connection |
| #34 | Based on Sol#3 baseline; direct connection SE-SF over data connection | New gateway sensing function (GSF); | Direct SF–SE connection |
| #38 | New SDM function introduced to store/update sensing authorization information | New SDM (may be co-located with existing NF) | Not addressed (focus on AF-NEF-SDM path for authorization data) |

Summary

- 10 solutions (Sol#3, Sol#4, Sol#6, Sol#7, Sol#19, Sol#27, Sol#28, Sol#31, Sol#34, Sol#38) agree on introducing a new sensing function (optionally split into processing, management/control, or authorization functions). - 8 solutions (Sol#1, Sol#3, Sol#6, Sol#19, Sol#27, Sol#28, Sol#34, Sol#38) support direct connection between SF and SE, while 2 solutions prefer an indirect connection via AMF. - Some solutions (Sol#6, Sol#31, Sol#34) assert the necessity of a new sensing gateway function. - Some solutions (Sol#4, Sol#31) support CP-based (N1/N2) and/or UP-based (N3) sensing data collection.

Proposal

It is proposed to agree the following changes into TR 23.700-14.

* * * * Start of 1st Change * * * *

!! All New Texts !!

7.1	Agreed Principles

7.1.Y	Agreed Principles for KI#1

The interim agreements on principles for KI#1 are as follows:

-	In the 5G Core Network, a new Sensing Function is defined to support sensing services. The Sensing Function may include logical components such as sensing control and sensing processing functionalities.

-	Sensing data may be transmitted between the Sensing Function and the Sensing Entity(ies) via a direct connection, where the Sensing Function requests the sensing service operation from the Sensing Entity(ies).

-	The Sensing Function provides the sensing result to the sensing service consumer (e.g., AF) according to the sensing service request.

7.2	Topics for further consideration

7.2.Y	Topics for further consideration for KI#1

Topics for further consideration for KI#1 are as below:

-	Whether and how to apply logical decomposition of the Sensing Function (e.g., sensing control function, sensing processing function) and define the possible interactions between such logical functions.

-	How to establish the direct connection and which protocol is used for the connection between the Sensing Function(s) and the Sensing Entity.

* * * * End of Changes * * * *