SA WG2 Meeting #171	S2-2508495

13 - 17 October, 2025, Wuhan, China	(Revision of S2-2506568)

Source:	Samsung

Title:	KI#4 New Sol: Sensing Data and the Associated Information Collection and Transport

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC /Rel-20

Abstract of the contribution: This contribution proposes a new solution for Sensing KI#4 on Sensing Data and the Associated Information Collection and Transport.

# Discussion

As it has been documented in clause 5.4 of TR 23.700-14, for KI#4 Sensing Data and the Associated Information Collection and Transport, the following questions are to be studies by SA2:

-	How to collect sensing data (and the associated information, if any) for sensing result generation?

-	Which Sensing Function performs sensing result generation?

-	Whether and what the above associated information is required for sensing result generation

NOTE:	Implications to RAN or RAN dependent aspects will be coordinated with RAN WGs.

As it has been agreed in SA#108, the scope of this topic will cover only gNB-based monostatic sensing in this release.

This contribution provides the end-to-end solution for KI#4 Sensing Data and the Associated Information Collection and Transport to support gNB-based monostatic sensing.

# Proposal

It is proposed to adopt the following changes into TR 23.700-14.

*** Start of the change ***

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 | #6 |
| #1 | X |  | X | X | X |  |
| #2 |  | X |  |  | X |  |
| #3 | X | X | X | X | X | X |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  | X |  |  |
| #6 | X | X | X |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 |  | X |  |  |  |  |
| #10 |  | X |  |  |  |  |
| #11 |  | X |  |  |  |  |
| #12 |  | X |  |  |  |  |
| #13 |  |  | X |  |  |  |
| #14 |  |  | X |  |  |  |
| #15 |  |  | X |  |  | X |
| #16 |  |  | X |  |  |  |
| #17 |  |  | X |  |  |  |
| #18 |  |  |  | X |  |  |
| #19 | X | X |  |  | X |  |
| #20 |  |  |  |  |  | X |
| #21 |  |  |  | X |  |  |
| #22 |  |  |  | X |  |  |
| #23 |  |  |  | X | X |  |
| #24 |  |  |  | X |  |  |
| #25 |  |  |  | X |  |  |
| #26 |  |  |  | X |  |  |
| #27 | X | X |  |  | X | X |
| #28 | X |  |  |  | X |  |
| #29 |  |  |  |  | X |  |
| #30 |  |  |  |  | X |  |
| #31 | X |  |  |  | X |  |
| #32 |  |  |  |  | X |  |
| #33 |  | X |  |  | X |  |
| #34 | X |  |  | X |  | X |
| #35 |  |  | X |  |  | X |
| #36 |  | X | X |  |  |  |
| #37 |  | X |  |  | X |  |
| #38 | X | X |  |  |  |  |
| #39 |  |  |  | X |  |  |
| #x |  |  |  | X |  |  |

*** Next change (all new text) ***

## 6.X	Solution #X: Support of Sensing Data and the Associated Information Collection and Transport

### 6.2.0	High-level Solution Principles

The solution is based on the following principles to support sensing data and information collection and transport:

- Based on request from sensing service consumer, sensing function (SSF) determines the configuration information of the corresponding services and trigger sensing data collection and transfer towards selected sensing entities.

- Establishing direct connection between SSF and sensing entity for sensing data transfer.

- Based on the sensing data collection and/or sensing data transfer request from SSF, the sensing entities collect sensing data and transfer the sensing data to SSF via direct connection as configured.

### 6.X.1	Description

This contribution provides the end-to-end solution for KI#4 Sensing Data and the Associated Information Collection and Transport to support gNB-based monostatic sensing scenarios. Sensing data and/or associated information needs to be collected by sensing entities and transported to SSF (Sensing Service Function) to support sensing results calculation. In this solution, the sensing entities/ gNBs may collect sensing data and report the corresponding sensing data to SSF, based on SSF request. The sensing data might be transferred directly (e.g. based on sensing protocol) from sensing entities to SSF. To transfer the sensing data, direct connections between SSF and each sensing entities are established.

### 6.X.2	Procedures

Figure 6.x.2-1 procedure of Sensing Data and the Associated Information Collection and Transport

0.	A sensing service consumer may send sensing service request to SSF (via NEF).

In the sensing service request, the sensing service consumer may include: AOI or target sensing area, required accuracy level and latency of sensing results, required detection probability and false alarm probability of sensing results, candidate list of sensing entities (if available), sensing objects and other assistance information that might be helpful for SSF to determine sensing service configuration or sensing results.

1.	Based on service consumer’s request, SSF may determine to trigger sensing related procedures within 5GS.

The SSF may determine the configuration and control information for the corresponding sensing service.

The SSF may determine to trigger data collection towards selected sensing entities.

NOTE 1: sensing entity discovery and selection is addressed in KI#3.

The SSF may also determine to trigger direct connection for sensing data transfer towards selected sensing entities, if there is no direct connection between SSF and those sensing entities. If direct connection between SSF and the select sensing entity is available, step 2-4 can be skipped.

2.	The SSF may send direct connection establishment request to the selected sensing entities via AMF. The SSF may also trigger sensing data collection in parallel.

The SSF may perform AMF discovery and selection based on AMF service area, AMF capability, sensing service area, sensing objects or candidate sensing entity list (if given), etc.

2.1.	SSF sends direct connection establishment request to AMF, e.g. by invoking Namf_communication_N1N2MessageTransfer or other service operation. The SSF may include IP address/FQDN of the SSF to indicate the sensing data transfer via direct connection in N2 container. The IP address/FQDN of the SSF is the destination of sensing data transfer. The SSF may also include binding information (e.g. sensing service ID, sensing event ID, etc.) to associate the direct connection to the corresponding sensing service or SSF.

2.2.	Based on in the information indicated by SSF, the AMF may forward the direct connection establishment request to sensing entities via N2 message.

NOTE 2:	the N2 message between AMF and NG-RAN is in RAN3 scope.

3.	Upon receiving the direct connection establishment request, each sensing entities establish direct connection between sensing entity and SSF, e.g. based on the received IP address/FQDN information of SSF. The sensing entity maybe also associated the direct connection to the corresponding SSF or sensing service using the received binding information.

Editor’s Note:	it is FFS which protocol (e.g. sensing protocol, based on IP addresses, etc.) for data transfer between sensing entity and SFF will be used.

4.	Sensing entities may send acknowledgement to SSF via AMF to indicate the successful establishment of direct connection for sensing data and information transfer.

4.1.	Sensing entity sends acknowledgement to AMF via N2 message.

4.2.	the AMF forwards the acknowledgement to SSF, e.g. via Namf_Communication_N2InfoNotify or other service operation.

5.	[Conditional] if SSF does not trigger sensing collection in step 2, the SFF may determine to trigger sensing data collection towards sensing entities in this step. The SSF may also indicate the determined configuration and control information for the corresponding sensing service to sensing entities.

Sensing data collection request maybe sent via direct connection, if established. Alternatively, the SSF may send the Sensing data collection request via CP through AMF; then the AMF forwards the request to sensing entity.

6.	Sensing entities perform sensing data collection, as configured by SSF.

7.	[Optional] the SSF may request the sensing entity to report sensing data and information.

8.	Sensing entities report the collected sensing data and associated information to SSF via direct connection.

Sensing entities may transfer sensing data based on explicit data transfer request from SSF in step 7, periodically (e.g. based on the periodicity configured by SSF), or other criteria (e.g. event triggered, data collection reaches a certain volume, etc.).

9 - 10.	SSF determines sensing results based on the sensing data and optional the sensing assistance information reported from sensing entities. The SSF may notify the sensing results to sensing service consumer as request in step 0.

NOTE 3:	Sensing Result Exposure is discussed under KI#5.

### 6.X.3	Impacts on services, entities and interfaces

SSF:

-	Trigger sensing data collection and/or transfer.

-	determine sensing related configuration and indicate it to sensing entities.

-	Reception of sensing data reported from sensing entities.

-	Derive sensing results based on (aggregated) sensing data.

-	Expose sensing results to sensing service consumer.

AMF:

- Support direct connection estalishment for sensing data and optional other related inforamtion transfer.

NG-RAN:

- Support direct connection estalishment for sensing data and optional other related inforamtion transfer.

- Tranfer sensing data and optional other related inforamtion via direct connection to SSF.

NOTE :	interface between AMF and NG-RAN node is in RAN3 scope.

*** End of the change ***