SA WG2 Meeting #171	S2-2509120

13 – 17 October, 2025, Wuhan, China	(revision of S2-250xxxx)

Source:	CATT

Title:	KI#4, Sol#21: Update solution#21

Add a term for Sensing QoS

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This pCR is proposed to update the solution#21 to solve the editor’s note.

- Introduction

The solution#21 includes three editor’s notes, as follows.

Editor's note:	The definition of sensing QoS is FFS.

To solve this editor’s note, a pCR proposing to add a new term for sensing QoS is also submitted in this meeting. So this editor’s note can be removed simply from the solution.

Editor's note:	The information included in sensing response is FFS and can be determined based on coordination with RAN.

Editor's note:	The information included in sensing report is FFS and can be determined based on coordination with RAN.

The sensing response and sensing report are sent by RAN to SF, so the ENs above is related to the information sent from RAN to SF. The information has been discussed in RAN1#122 meeting, different options have been discussed and summarized in R1-2506480, as follows:

NOTE:	The measurements for sensing in the Figure above refer to the information a BS transmits to core network upon a sensing request as described in R1-2506480.

Which option(s) to be supported is still under discussion in RAN1 WG. Based on the status above, it is proposed to leave it to RAN WGs to decide what information is included in the sensing response and sensing report and to revise the ENs to Notes as follows:

Note:	The information included in sensing response is to be determined in RAN WG(s).

Note:	The information included in sensing report is to be determined in RAN WG(s).

2. Proposal

Based on the information in clause 1, it is proposed to update the solution#21.

**** First Change ****

6.21	Solution #21: Sensing Data Collection and Transport via the Direct Interface between NG-RAN and SF

6.21.1	High level principles

This solution aims to address key issue#4 Sensing Data and the Associated Information Collection and Transport.

The principles of the solution include:

-	The Sensing Function (SF) and NG-RAN acting as sensing entity supports direct interface to transfer sensing message.

6.21.2	Description

The architecture with direct interface between NG-RAN and SF is shown in Figure 6.21.2-1.

Figure 6.21.2-1: The architecture with direct interface between NG-RAN and SF

SF selects NG-RAN to act as sensing entity and uses Nx interface to transfer sensing message with the selected NG-RAN.

NOTE:	The protocol stack for the Nx interface can be determined based on coordination with RAN WGs and CT WGs.

6.21.3	Procedures

Figure 6.21.3-1: Sensing Message Transport procedure

0.	The OAM configures the SF with the information of NG-RAN which supports the Sensing Entity function, e.g. the IP address of NG-RAN.

1.	The sensing service consumer/NEF/GW function sends Nsf_SensingService Request to SF. The request includes sensing area, sensing QoS, sensing service type (e.g. object detection, object tracking), sensing target type.

If the consumer requests periodic sensing report, the request also includes the time interval between successive sensing reports and the total number of reports.

2.	[Conditional] If the consumer requests periodic sensing report, the SF sends the Nsf_SensingService Response to the sensing service consumer/NEF/GW function to acknowledge that the request is received.

3.	The SF selects NG-RAN to act as Sensing Entity.

4.	The SF sends DL Service Message Transfer to the selected NG-RAN. The message includes correlation ID and sensing request message. The correlation ID is allocated by SF to identify the sensing service request received in step 0.

NOTE:	The sensing request is in scope of RAN WG3.

5.	The NG-RAN sends UL Service Message Transfer to SF. The message includes correlation ID and sensing response. The correlation ID is received in step 4.

NOTE:	The information included in sensing response is to be determined in RAN WG(s).

6.	The SF sends Nsf_SensingService Response including the sensing result to the sensing service consumer/NEF/GW function. The sensing result includes the position and velocity of the detected object/target.

7.	[Conditional] Based on the sensing request, the NG-RAN sends UL Service Message Transfer to SF periodically. The message includes correlation ID and sensing report. The correlation ID is received in step 4.

NOTE:	The information included in sensing report is to be determined in RAN WG(s).

8.	[Conditional] If Sensing Report is received, the SF sends Nsf_SensingService Notify including the sensing result to the sensing service consumer/NEF/GW function. The sensing result includes the position and velocity of the detected object/target.

6.21.4	Impacts on Services, Entities and Interfaces

SF, NG-RAN:

-	Support sensing message transfer via the direct interface between RAN and SF.

**** End of Change ****