3GPP SA WG2 Meeting #171	S2-2509532

Wuhan, CN, 13 – 17 Oct 2025	(revision of S2-2508565)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5584 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PCF service update to identify Hosting Network in case of INS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20_NetShare_Ph2-ARC |  |  |  |  |  | Date: |  |  | 2025-10-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the description of NetShare_ph2-ARC WID, the NFs of the participating operator’s network (e.g. UDM, SMF, PCF) to detect the Indirect Network Sharing case and identify the hosting operator’s network. While based on the current description in clause 5.18.1 of 23.501, the serving PLMN ID is set to the selected PLMN ID as below. This means the NFs of the participating operator’s network (e.g. UDM, SMF, PCF) may not detect the Indirect Network Sharing case and identify the hosting operator’s network. “For interaction between the Network Functions in the hosting operator network and the participating operator network (e.g. AUSF, UDM), the NF of hosting operator network sets the serving PLMN ID to the selected PLMN ID.” Similarly, when the UE moves between hosting operator networks, or between hosting operator network and participating operator network, the NFs of the participating operator’s network (e.g. UDM, SMF, PCF) may not detect the Indirect Network Sharing case and identify the hosting operator’s network as well. This contribution updates the PCF service to include hosting PLMN ID in case of INS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the PCF service to include hosting PLMN ID in case of INS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The participating operator’s network may not be able to identify the hosting operator network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.16.11.1, 5.2.5.4.2, 5.2.5.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.16.11	UE Policy Association Establishment

4.16.11.1	General

The UE Policy Association Establishment procedure, which may be performed for a UE registered in the same AMF or different AMFs for 3GPP access and non-3GPP access, concerns the following scenarios:

1.	UE initial registration with the network.

2.	The AMF relocation with PCF change in handover procedure and registration procedure.

3.	UE registration with 5GS when the UE moves from EPS to 5GS and there is no existing UE Policy Association between AMF and PCF for this UE.

4.	A change of the value of the UE Policy Association indicator received from UDM from "disabled" to "enabled" while the UE is registered.

The H-PCF may interact with the CHF in HPLMN to make a decision about UE Policies based on spending limits.

Figure 4.16.11-1: UE Policy Association Establishment

This procedure concerns both roaming and non-roaming scenarios. The AMF stores the UE Policy Container in the UE Context when received from the UE.

In the non-roaming case the V-PCF is not involved and the role of the H-PCF is performed by the PCF. For the roaming scenarios, the V-PCF interacts with the AMF and the H-PCF interacts with the V-PCF:

1.	The AMF establishes UE Policy Association with the (V-)PCF considering the value of the UE Policy Association Indicator from UDM, the reception of the UE Policy Container from the UE and the AMF local configuration as defined in clause 6.1.2.2.5 of TS 23.503 [20].

NOTE 1:	In roaming scenario, the AMF local configuration can indicate whether UE Policy delivery is needed based on the roaming agreement with home PLMN of the UE as defined in TS 23.503 [20].

2.	The AMF sends a Npcf_UEPolicyControl Create Request with the following information: SUPI, may include Access Type and RAT, PEI, ULI, UE time zone, Serving Network (PLMN ID, or PLMN ID and NID, see clause 5.34 of TS 23.501 [2]), the PLMN ID of the hosting operator (see clause 5.18.1 of TS 23.501 [2]), the Internal-Group-ID-list and UE Policy Container (the list of stored PSIs, operating system identifier, Indication of UE support for ANDSP, indication of UE capability of reporting URSP rule enforcement to network, indication of support of URSP delivery in EPS). In roaming scenario, based on operator policies, the AMF may provide to the V-PCF the PCF ID of the selected H-PCF. The V-PCF contacts the H-PCF. In roaming case, steps 3 and 4 are executed, otherwise step 5 follows.

If the AMF, based on configuration, is aware that the UE is accessing over a gNB using satellite backhaul, the AMF includes the Satellite backhaul category as described in clause 5.43 of TS 23.501 [2].

If during UE initial registration the AMF receives from the UE the indication of support of N3IWF selection based on the slices the UE wishes to use over untrusted non-3GPP access and/or of UE support for TNGF selection based on the slices the UE wishes to use over trusted non-3GPP access, the AMF shall indicate to the PCF that the UE supports N3IWF selection based on the slices the UE wishes to use over untrusted non-3GPP access and/or UE support for TNGF selection based on the slices the UE wishes to use over trusted non-3GPP access. The AMF may request to the PCF to receive a notification when the provisioning in the UE of ANDSP/WLANSP with slice information has finished.

3.	The V-PCF forwards the information received from AMF in step 2 to the H-PCF. When a UE Policy Container is received at initial registration, the H-PCF may store the PEI, the OSId, indication of UE capability of reporting URSP rule enforcement to network, indication of support of URSP delivery in EPS or the indication of UE support for ANDSP in the UDR using Nudr_DM_Create including DataSet "Policy Data" and Data Subset "UE context policy control data".

3a.	The H-PCF may invoke the Nudr_DM_Query service to the H-UDR. The H-UDR may respond with the requested policy control subscription data as in clause 6.2.1.3 of TS 23.503 [20] and/or application data as in clause 6.2.1.6 of TS 23.503 [20].

The V-PCF may retrieve the Application guidance on URSP Rule for inbound roamers of the PLMN of the SUPI, if not available, using Nudr_DM_Query or Nudr_DM_Subscribe including the Data Set "Application Data" and Data Subset "Service Specific Information" and DataKey set to "PLMN ID(s) of inbound roamers".

3b,	If the H-PCF determines that the decision about UE policy control depends on the status of the policy counters available at the H-CHF and if such reporting is not established for the subscriber, the H-PCF initiates an Initial Spending Limit Report Retrieval as defined in clause 4.16.8.2. If policy counter status reporting is already established for the subscriber and the H-PCF determines that the statuses of additional policy counters are required, the H-PCF initiates an Intermediate Spending Limit Report Retrieval as defined in clause 4.16.8.3.

NOTE 2:	The Nudr_DM_Query in step 3a may include the Spending Limit Information, i.e. the policy counters and their latest status. Thus the PCF can provide the UE policy to the AMF before contacting the CHF. The PCF may need to update the AMF depending on the statuses of the policy counters provided by the CHF.

NOTE 3:	Potential inconsistencies between the policy counter and its status in the H-UDR and in the H-CHF can happen given that the H-CHF may update the policy counter and its status at any time, as such it is recommended that the H-PCF contacts the H-CHF if the policy counters and its status stored in the H-UDR is used, to be able to receive updated information from the H-CHF.

4.	The H-PCF sends a Npcf_UEPolicyControl Create Response to the V-PCF. The H-PCF may provide the Policy Control Request Trigger parameters in the Npcf_UEPolicyControl Create Response.

The (H-)PCF in roaming and the PCF in non-roaming may register to the BSF as the PCF serving this UE, if not already registered at the AM Policy Association establishment. This is performed by using the Nbsf_Management_Register operation, providing as inputs the UE SUPI/GPSI and the PCF identity.

5.	The (V-) PCF sends a Npcf_UEPolicyControl Create Response to the AMF. The (V-)PCF relays the Policy Control Request Trigger parameters in the Npcf_UEPolicyControl Create Response.

The (V-)PCF also subscribes to notification of N1 message delivery of policy information to the UE using Namf_Communication_N1N2MessageSubscribe service which is not shown in this figure.

6.	The (H-)PCF gets policy subscription related information and the latest list of PSIs from the UDR using Nudr_DM_Query service operation (SUPI, Policy Data, UE context policy control data, Policy Set Entry) if either or both are not available and makes a policy decision. The (H-)PCF may get the PEI, the OSId, indication of UE capability of reporting URSP rule enforcement to network or the indication of UE support for ANDSP in the UDR using Nudr_DM_Query including DataSet "Policy Data" and Data Subset "UE context policy control data" if the AMF relocates and the PCF changes. In the roaming scenario, the H-PCF may provide the indication of UE support for ANDSP to the V-PCF, if the indication was not present in the Npcf_UEPolicyControl Create request from V-PCF and the H-PCF gets this information from the H-UDR. The (H-)PCF may get the 5G VN group data and 5G VN group membership for each Internal-Group-ID received from the AMF using Nudr_DM_Query (Internal-Group-Id, Subscription Data, 5G VN Group Configuration). The (H-)PCF may store the 5G VN group data and 5G VN group membership for later use for other SUPIs that belong to the same Internal-Group-ID. The (H-)PCF may request notifications from the UDR on changes in the subscription information by invoking Nudr_DM_Subscribe (Policy Data, SUPI, DNN, S-NSSAI, Notification Target Address (+ Notification Correlation Id), Event Reporting Information (continuous reporting), UE context policy control data) service. The (H-)PCF may request notifications from the UDR on changes in the 5G VN group data or 5G VN group membership associated to each of the Internal-Group-Id provided to the PCF by invoking Nudr_DM_Subscribe (Subscription Data, 5G VN Group Configuration, Internal Group ID, Notification Target Address (+ Notification Correlation Id), Event Reporting Information (continuous reporting)) service. The (H-)PCF creates the UE policy container including UE policy information as defined in clause 6.6 of TS 23.503 [20] and in the case of roaming H-PCF provides the UE policy container in the Npcf_UEPolicyControl_UpdateNotify Request. In the non-roaming case, the PCF may subscribe to Analytics from NWDAF as defined in clause 6.1.1.3 of TS 23.503 [20].

7.	The V-PCF sends a response to H-PCF using Npcf_UEPolicyControl_UpdateNotify Response.

NOTE 2:	Step 6 (and step 7) can be omitted. Then the (H-)PCF creates the UE policy container including UE polices in step 2 (in the case of non-roaming) or step 3 (in the case of roaming). This means that the potential interactions with UDR as in step 6 will have to be executed in step 2 (non-roaming) or step 3 (roaming).

8.	The (V-)PCF triggers UE Configuration Update Procedure in clause 4.2.4.3 to sends the UE policy container including UE policy information to the UE. The (V-)PCF checks the size limit as described in clause 6.1.2.2.2 of TS 23.503 [20].

9.	If the V-PCF received notification of the reception of the UE Policy container then the V-PCF forwards the notification response of the UE to the H-PCF using Npcf_UEPolicyControl_Update Request.

If the V-PCF is notified by the V-UDR about the Service Specific Information applicable to inbound roamers from the HPLMN of the UE as specified in clause 4.15.6.10, the V-PCF provides the Service Parameters to the H-PCF.

10.	The H-PCF sends a response to the V-PCF. If the V-PCF received a UE Policy Container step 8 will follow.

# * * * * Second change * * * *

5.2.5.4.2	Npcf_SMPolicyControl_Create service operation

Service operation name: Npcf_SMPolicyControl_Create.

Description: The NF Service Consumer can request the creation of a SM Policy Association and provides relevant parameters about the PDU Session to the PCF.

Inputs, Required: SUPI (or PEI in the case of emergency PDU Session without SUPI), PDU Session id, DNN, S-NSSAI and RAT Type.

Inputs, Optional: Information provided by the SMF, such as PDU Session Type, Request Type, Access Type, the IPv4 address and/or IPv6 prefix, PEI, GPSI, User Location Information, UE Time Zone, Serving Network identifier (PLMN ID, or PLMN ID and NID, see clause 5.34 of TS 23.501 [2]), the PLMN ID of the hosting operator (see clause 5.18.1 of TS 23.501 [2]), Charging Characteristics information, Session-AMBR, subscribed default QoS information (5QI, 5QI Priority Level, ARP), UE support of reflective QoS (see TS 23.501 [2], clause 5.7.5.1), Number of supported packet filters for signalled QoS rules for the PDU Session (see TS 23.501 [2], clause 5.7.1.4), UE's capability to support (S)RTP Multiplexed Media Identification Information in IP Packet Filters (see clause 4.3.2.2.1), 3GPP PS Data Off status, Trace Requirements and Internal Group Identifier (see clause 5.9.7 of TS 23.501 [2]), DN Authorization Profile Index, DN authorized Session AMBR, Framed Route information (as defined in Table 5.2.3.3.1-1), MA PDU Request indication, MA PDU Network-Upgrade Allowed indication, ATSSS capabilities of the MA PDU Session, QoS constraints from the VPLMN (as defined in clause 5.7.1.11 of TS 23.501 [2]), Satellite backhaul category, list of NWDAF instance Ids (used by AMF, SMF, UPF) and corresponding Analytics ID(s), PVS IP address(es) and/or PVS FQDN(s) and Onboarding Indication in the case of ON-SNPN (see clause 5.30.2.10.4.2 of TS 23.501 [2]), URSP rule enforcement that including Connection Capability, PCF binding information (address(es) of PCF for UE, instance id of PCF for UE), HR-SBO support indication (see clause 6.2.1.2 of TS 23.503 [20]), Alternative S-NSSAI (see clause 5.15.19 of TS 23.501 [2]), URSP delivery in EPS support indication, Local Offloading Management indication.

NOTE 1:	If SMF receives the DN authorized Session AMBR from the DN-AAA at PDU Session establishment, it includes the DN authorized Session AMBR within the Session-AMBR, instead of the subscribed Session-AMBR received from the UDM, in the request.

NOTE 2:	It is up to stage 3 to determine whether the corresponding supportedFeature in Npcf_SMPolicyControl can be reused as URSP delivery in EPS support indication.

NOTE 3:	When Local Offloading Management is allowed, SMF provides Local Offloading Management indication to PCF.

W-5GAN specific PDU Session information provided by the SMF is specified in TS 23.316 [53].

Outputs, Required: SM Policy Association ID defined in TS 29.512 [57]. Success or Failure.

Outputs, Optional: Policy information for the PDU Session as defined in TS 23.503 [20] and Policy Control Request Trigger(s) of SM Policy Association as defined in clause 6.1.3.5 of TS 23.503 [20].

See clause 5.8.2.2 of TS 23.501 [2] for allocation of IPv4 address and IPv6 prefix. The IPv6 prefix length is /64, or is shorter than /64 when Prefix Delegation applies.

See clause 4.16.4 for the detail usage of this service operation.

See clauses 4.22.2.1 and 4.22.3 for detailed usage of this service operation for ATSSS.

# * * * * Third change * * * *

5.2.5.6.2	Npcf_UEPolicyControl_Create service operation

Service operation name: Npcf_UEPolicyControl_Create

Description: NF Service Consumer can request the creation of a UE Policy Association by providing relevant parameters about the UE context to the PCF.

Inputs, Required: Notification endpoint, SUPI.

Inputs, Optional: H-PCF ID/address (if the NF service producer is V-PCF and AMF is NF service consumer), information provided by the AMF as define in clause 6.2.1.2 of TS 23.503 [20], such as Access Type, Permanent Equipment Identifier, GPSI, User Location Information, UE Time Zone, Serving Network (PLMN ID, or PLMN ID and NID, see clause 5.34 of TS 23.501 [2]), the PLMN ID of the hosting operator (see clause 5.18.1 of TS 23.501 [2]), RAT type, LBO Information (see clause 6.1.2.2.4 of TS 23.503 [20]), UE policy container including the list of PSIs, OS id, the indication of UE support for ANDSP, UE capability of reporting URSP rule enforcement to network (see clause 6.6.2.4 of TS 23.503 [20]), UE indication of support of URSP delivery in EPS and Internal Group (see TS 23.501 [2]), Satellite backhaul category (see clause 5.43 of TS 23.501 [2]), "5GS to EPS Mobility" indication, request to update the UE policies, request to be notified when updated UE policies have been provided to the UE, the indication of UE support for N3IWF selection based on the slices the UE wishes to use over untrusted non-3GPP access, the indication of UE support for TNGF selection based on the slices the UE wishes to use over trusted non-3GPP access.

Outputs, Required: Success or Failure, UE Policy Association ID.

Outputs, Optional: Policy Control Request Trigger of UE Policy Association. In the case of H-PCF is producer, UE policy information (see clause 5.2.5.6.1), Request for notification of SM Policy Association establishment and termination, PCF binding information.

Request for notification of SM Policy Association establishment and termination and the PCF binding information apply when the BSF is not deployed.

# * * * * End of changes * * * *