3GPP TSG-SA2 Meeting #171	S2-2508642

Wuhan, China, October 13-17 2025	(revision of S2-2507677)

Source:	Nokia, Lenovo

Title:	KI#3: proposed agreement of principles

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract: This document proposes to agree principles for KI#3.

# 1.	Introduction

This paper proposes a n initial set of principles for agreement and also outstanding topics that deserve further consideration. (yellow text is new text from last meeting version in S2-2507677)

In addition, Nokia has as a pen holder generated from these papers this version text including commented text in the P-CR and also an issue list is provided in clause 1.a.

| S2-2508377 | Interim principles for KI3. | Vivo |
| --- | --- | --- |
| S2-2508391 | KI#3 interim agreements. | CATT |
| S2-2508616 | Agreed principles for key issue #3. | China Mobile |
| S2-2508695 | KI #3, New Agreed Principles: Agreed Principle for Key Issue #3. | InterDigital Inc |
| S2-2508760 | WT#3: conclusion and agreement. | Huawei, HiSilicon |
| S2-2508765 | [KI#3] proposed agreement of principles. | [Nokia] NTT DOCOMO |
| S2-2508824 | Key issue#3, the analysis and interim agreement. | ZTE |
| S2-2508893 | KI#3: proposed agreement of principles. | Ericsson |
| S2-2509003 | Interim agreements for KI#3. | MediaTek Inc. |
| S2-2509091 | KI#3: Proposal for agreements and conclusion. | Apple |

# 1.a	Issues list

-	PDU Session Modification procedure is reused for UP path adjustment.

-	The NRF may store NF Profiles for N3IWFs and TNGFs. N3IWF and TNGF profiles may include Energy Information (e.g. Energy Priority Information or Schedule of Energy Priority Information). The AMF may query the NRF for N3IWF and TNGF profiles so that the information can be considered for N3IWF or TNGF reselection. The N3IWF and TNGF can provide their profiles to the NRF by invoking the Nnrf_NFManagement_NFRegister service or the N3IWF and TNGF profiles can be provided to the NRF via OAM.

-	Renewable energy support The value indicates whether the NF is powered with renewable energy and it is configured by OAM. Configure supported/not supported or ratio.

-	Docomo proposes to remove the statement that the N profile update proposed in this paper requires SW upgrade of functions that use this additional Information. In NF profile.

-	The energy policy configured by operator to select the correct energy priority in array of energy priority needs to be made concrete: is this the energy saving indicator (remember this is a scalar operator can set)

-	Ericsson paper only assumes AMF selection can used of energy state . the rest of NF profile updates is FFS. Some other papers question the need of schedule.

-	Energy Consumption at a slice level and based on a long term trend may be a trigger for reselecting NFs.

-	Energy Consumption at a granularity of a UE or a PDU Session is not used as a trigger for reselection of NFs.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-67.

# * * * * First change (all new text) * * * *

### 7.1.3	Agreed Principles for KI#3

The following principles are agreed for KI#3:

-	For NF selection, the NF profile is enhanced to include the parameters in the table below. This is a feature requiring the software to be upgraded in NRF and its consumers.

| Information | Description |
| --- | --- |
| EnergySaving /Not EnergySaving State | Value of the energy state as specified in TS 28.310 [9], e.g. notEnergySaving state, Energy Saving state. (NOTE 1). When the NF profile indicates that the NF is in EnergySaving state, the NF cannot be discovered nor selected. |
| Schedule of EnergySaving/not EnergySaving | The schedule of the EnergySaving/not EnergySaving states, includes the value of EnergySaving or not EnergySaving state which applies at each time interval in the schedule. If this is present, the EnergySaving /Not EnergySaving State attribute is not present. NRF and NF consumers should avoid selecting NFs that include a scheduled EnergySaving State if the NF consumers require the NF to be available (e.g. SSC mode 1) |
| Energy Priority Information | Value of Energy priority Set by the operator according to own policies. It indicates the priority relative to other NFs of the same type considering the energy characteristics of the NF (including aspects like the power source characteristics/cost used for the hosting of the NF) |
| Schedule of Energy Priority Information | The Schedule of the Energy Priority includes the value of Energy Priority information which applies at each time interval in the schedule. If this is present, the Energy Priority information attribute is not present. NRF and NF consumers should avoid selecting NFs that include a scheduled shift to a lower priority that is likely to cause reselection of the NF if the NF consumer require the NF being used to be stable (e.g. SSC mode 1) |
| NOTE 1: EnergySaving/not EnergySaving refers to states of a network function which are defined in TS 28.310 [9]. |  |

-[	[CONDITIONALLY AGREED] For NF profile, and specifically for UPF selection, additional parameter such s renewable energy may be considered if SA5 indicates support. ..

or

-	[CONDITIONALLY AGREED] The usage of renewable energy: Value (Boolean type) of the usage of renewable energy is to represent the energy source is renewable or non-renewable and this value is configured by operator with the information obtained from external sources. Clause 8.3.2 EnergySourceInfo in TS 28.310 defines the energy resource type. The usage information of the renewable energy can be obtained from OAM and can be included as the NF profile information for NF selection and reselection. This can be confirmed with SA5.

-	For UP path selection and reselection, the UPFs are selected/reselected by using NF discovery and selection procedures which may be Energy-aware if NF profile is updated as per bullets above. Changes in NF profile of a UPF may trigger UPF reselection subject to SSC mode of the PDU sessions.

- 	The Energy Priority Information is defined in solution #20 clause 6.20.1.

-	OAM provisions and updates as needed the information within the NRF profile. The above information are not known by the NF itself.

-	The NF Discovery procedure, based on operator policy, takes into account any Energy-related NF profile attributes query parameters. Both the NRF and the NF service consumer can take into account the Energy-related NF profile attributes if available, as part of the NF Discovery procedure.

-	A NF service consumer takes into account any Energy-related NF profile attributes from the NF profiles discovered from NRF using the NF Discovery procedure (and subsequent updates if the NF subscribed to NF profile updates), for the selection of a target NF.

-  The schedule information may be taken into account in the process of the selection of the NF by the consumer of the information in order to select the appropriate NF considering the future behaviour, if needed. The criteria for taking into account the schedule information is implementation specific.

### 7.2.3	Topics for further consideration for KI#3

The following principles are for further discussions for KI#3:

-	whether the NF profile needs to include explicit indication of usage of Renewable Energy or using Energy Priority to implicitly take this into account is sufficient. Note that this depends on availability of renewable energy information at a NF which for now is FFS.

-	Whether considering the energy consumption of a PDU sessions or of a NF over a Path should be a trigger for reselection of UPFs (including embedding this information in DNAIs )

-	In case of multiple configured energy policies in network, the "energy priority" is an array of priorities. Each entry of the array is the priority of the NF in the corresponding energy policy configured by the operator.

# * * * * End of change * * * *