SA WG2 Meeting #170	S2-2507055

25 – 29 Aug, 2025, Goteborg , SE

Source:	Nokia (pen holder), AT&T, T-Mobile US,….

Title:	[WT#1.1, all topics] Work task #1.1 Support of 6G Control plane signaling

Document for:	Approval

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: Scope of Work task 1.1.

# 1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

Work task 1.1 scope as agreed in 6G System Study item S2-2506096:

.1.	Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

# 2. Documents proposing WT#1.1

| Tdoc | Title | Source |
| --- | --- | --- |
| S2-2506393 | [WT#1.1] Support of Control Signalling in 6GS | OPPO |
| S2-2506465 | [WT#1.1, control signalling] 6G NAS for connectivity and beyond connectivity services | China Telecom |
| S2-2506480 | [WT#1.1] WT and KI for enhancement of UE and Core Network interaction | China Mobile |
| S2-2506505 | [WT#1.1, all topics] Control Signalling Design for 6G | MediaTek Inc. |
| S2-2506517 | [WT#1.1] NAS for 6G | Deutsche Telekom, AT&T, Cisco Systems, KPN N.V., Rakuten Mobile, Sateliot, SK Telecom, T-Mobile USA, Telecom Italia |
| S2-2506521 | [WT#1.1, beyond connectivity services] Support for an Identity Service | InterDigital Inc. |
| S2-2506523 | [WT#1.1, control signalling] NAS functionality for 6G | Ofinno |
| S2-2506555 | WT#1.1, Support for NAS Signalling in the 6G Architecture | InterDigital Inc. |
| S2-2506577 | [WT#1.1, NAS functionalities] Scope and New Key Issue for NAS functionalities | LG Electronics |
| S2-2506611 | [WT#1.1, 6G NAS] Support of 6G New NAS | vivo |
| S2-2506752 | [WT#1.1] Discussion and proposal on key issue for work task#1.1 | CATT |
| S2-2506775 | [WT#1.1, NAS] Discussion and proposal for NAS | ZTE |
| S2-2506822 | [WT#1.1] UE-Core Network interaction in 6G | CEWiT |
| S2-2506840 | [WT#1.1, all topics] Support of control signaling for connectivity and beyond connectivity services | Samsung |
| S2-2506868 | [WT#1.1, Control Signalling for 6G] Scope and Key Issue proposal for UE-Core Network interaction | IIT Bombay |
| S2-2506871 | [WT#1.1, Non-Access Stratum] Scope and Key Issue for Non-Access Stratum | Futurewei |
| S2-2506903 | [WT#1.1] Introduction of NAS functionality without impacting other NAS functionalities | Apple |
| S2-2506942 | [WT#1.1, Control signalling for 6G] UE-CN control signalling communication | Ericsson |
| S2-2506975 | [WT #1.1, 6G NAS] Discussion and key issues for distributed NAS | TCL |
| S2-2506988 | [WT#1.1] Update to WT#1.1 for clarification on flexible addition of new services in 3GPP | Qualcomm Incorporated |
| S2-2507055 | [WT#1.1, all topics] Work task #1.1 Support of 6G Control plane signaling | Nokia, AT&T, T-Mobile US |
| S2-2507124 | [WT#1.1, NAS] New NAS and Interaction between UE and Network | Huawei, HiSilicon |
| S2-2507135 | [WT#1.1] Technical Scope of WT#1.1 Control Signaling in 6GS | ETRI |
| S2-2507200 | [WT#1.1, NAS] Key Issues on NAS and operator services in 6G | Lenovo |
| S2-2507353 | [WT#1.1] KI on UE-CN interaction for control signalling for 6G | Xiaomi |
| S2-2507378 | WT#1.1, Study proposal to enhance a NAS functionality | NEC |

S2-2506480, S2-2506577, S2-2506611, S2-2506752, S2-2506775, S2-2506840, S2-2506903, S2-2506975, S2-2507124, S2-2507378:

- WT1.1 scope as agreed in the 6G study item

S2-2506393:

WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network:

NOTE  1:	The duplication of functionality in RAN and CN will be avoided, while maintaining the existing RAN and CN functionality split.

1.1.	Study the support for control signalling for 6G for connectivity and/or beyond connectivity services, including at least the following:

- a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities; How to route the NAS signalling for basic connectivity between UE and the according NFs with less NAS signalling routing delay and avoiding potential NAS signalling congestion via a centralized AMF.

- b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

- c.	How to support the control signalling (i.e., beyond connectivity service message) interaction for beyond connectivity services (e.g., LCS, Sensing, AI, etc.) between UE and the according NFs; Whether and how to develop generic mechanisms for UE-Core Network interaction to support beyond connectivity services.

S2-2506465:

Study high-level 6G NAS functionalities to support connectivity and beyond connectivity services. These NAS functionalities meet the requirement of well-structured modularity, e.g. introduction of a new NAS functionality without impacting other NAS functionalities. The detailed NAS procedures are to be studied in related WT, e.g. WT 3, 4 and 6.

To support beyond connectivity services, study generic 6G NAS delivery and transmition mechanism to meet  the requirement of  signalling scalability and efficiency. This study may have RAN dependancy.

S2-2506505:

- WT1.1 scope as agreed in the 6G study item. In addition, consider the following aspects:

In particular, this WT addresses at least the below versus 5GS design as the baseline:

- How to route control messages for connectivity, legacy services (i.e., NAS) between UE and CN

- How to route control messages for beyond connectivity, new services between UE and CN

- Impact on termination point of interfaces between UE and CN

- Define NAS architecture and system procedures (e.g., registration, connection, mobility, session management, etc.)

- Where and how to handle authentication, security management and integrity protection

NOTE 1: Potential impact on Interworking to be coordinated with KIs related to WT#2.

NOTE 2: UE-CN interactions for beyond connectivity services to be coordinated with KIs related to WT#5, WT#6.

NOTE 3: Potential security and privacy impact to be coordinated by SA3.

S2-2506517:

- WT1.1 scope as agreed in the 6G Study item. In addition, consider the following aspects:

This topic is scoped by the WT descriptions in Table A.1.2.X-1.

Table A.1.2.X-1: Work tasks for non-access stratum functionality

| Work Task ID | Work Task(s) | Work Task Description |
| --- | --- | --- |
| NAS_WT_#1.1.1 | NAS modularization | - How can other NAS functionality be decoupled from NAS-MM - Interactions between UE and CN - Interactions between UE and RAN, as well as between RAN and CN necessary to achieve 1.2 (NOTE V, NOTE W, NOTE X, NOTE Y) |
| NAS_WT_#1.1.2 | NAS message routing | 2.1 How NAS messages be routed between the UE and a target NF in the NF 2.2 Interactions between UE and CN 2.3 Interactions between UE and RAN, as well as between RAN and CN necessary to achieve 2.2 (NOTE V, NOTE W, NOTE X, NOTE Y) |
| NOTE V: Topology hiding and security aspects are to be evaluated by SA3 NOTE W: Work on the N2 interface regarding transport of NAS messages to be conducted together with RAN3 NOTE X: Interworking aspects for NAS are covered by WT#2. NOTE Y: It is acknowledged that performance impact related to message encoding is an important aspect, which is in CT1 scope |  |  |

S2-2506521:

# WT#1.1 Scope of a Beyond Connectivity Service for Identities

One of the beyond connectivity services that is in scope of work task 1.1 is support of an identity service. This part of the work task includes studying the following aspects of a 6G identity service.

-	Definition of the architectural assumptions that are necessary to support associating an identifier with all of a UE’s traffic or only the traffic of a UE application.

-	What information is stored as part of an identity profile (e.g., an identifier, associated security credentials, associated devices, user specific QoS settings). Including how profiles are acquired, stored, and updated in the network.

-	Whether and how identifiers are linked and unlinked (i.e., associated) with subscriptions in an operator-controlled manner.

-	Whether and how identity specific QoS settings are considered in order to provide service differentiation when providing communication services.

-	How identities are verified and authorized.

NOTE 1:	Aspects of the study of this service will depend on interaction with SA WG3.

NOTE 2:	Solutions to this part of the work task do not need to be NAS based (i.e. an identity service does not need to be NAS based).

S2-2506523:

WT#1.1: Study how to extend the Non-Access Stratum (NAS) functionality to support control signalling for 6G services. The 6G services to be supported by the new NAS functionality can be distinguished such as the basic connectivity services for communications (e.g. mobility management such as registration, paging, session management), and the beyond connectivity services (e.g. the operator services that can be existed over N6 such as AI/ML services or Sensing services). The following aspects will be, at least, considered:

- How to introduce a new NAS functionality while considering minimum impacts on other NAS functionalities, and how to minimize congestion/overhead in the existing control-plane-based NAS signalling.

- How to identify/discover a NF which is supporting the 6G NAS functionality.

- How to identify a minimal set of NAS functionalities for UEs using basic connectivity services in general, and additional sets of NAS functionalities for certain types of UEs (e.g. NB-IoT) and/or certain type of operator services (e.g. AI/ML services, Sensing services).

- How to support the new NAS functionality in roaming scenarios.

S2-2506555:

The objective of this work task is to study support for control signalling in 6G for connectivity services and/or beyond connectivity services, including at least the following:

- Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

The study will consider:

-	Whether and how to redefine the functional split between the Access and Mobility Management Functionality and other Core Network Functions (NFs).

-	Whether and how to remove dependencies between NAS features in order to support independent evolution and deployment.

-	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

The study will consider:

- Whether and how the 6G system can support distributed NAS design principles to enable minimal configurations.

- Whether and how to determine functional independence among NAS module within the distributed NAS design.

- Whether and how new NAS functionalities can be integrated dynamically without introducing backward compatibility or dependency issues on the core minimal set.

-	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

The study will consider:

-	Whether and how to allow for flexible UE-CN and RAN-CN interactions.

-	Whether and how to support diverse 6G deployment scenarios (e.g., distributed, NPNs, NTNs) through adaptable signalling paths and transport protocols.

NOTE: 	Security aspects for distributed NAS aiming at reducing other NAS functionalities tight coupling with Access and Mobility Management Functionality are to be evaluated by SA3.

S2-2506822:

WT#5	Study the scope for the development of a new NAS functionality and generic mechanisms for UE-Core Network interactions in 6GS, which include:

- Preserve the access-agnostic NAS model.

- Independent NAS modules for newly introduced NAS functionalities like connectivity and beyond connectivity services.

- Identifying the minimal set of NAS functionalities suitable for all devices.

- Efficient and secured NAS message transmission  reducing signalling overhead.

S2-2506868:

Objective for “WT #1.1.c - Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.” includes:

- Study efficient mechanisms for UE-Core Network interaction in 6G systems for signalling/data collection to support connectivity services and/or beyond connectivity services for handling higher signalling load and to reduce service delivery latency

- Study the placement and functionalities of network functions to support efficient  mechanisms for UE-Core network interaction

S2-2506871:

WT 1.1 Enhanced NAS:

Study the general architectural aspects for control signaling and connectivity services in 6G that can support a range of connectivity services with connections (PDU sessions) and beyond connectivity (non-session connectivity)

- Investigate new/enhanced functionality for handling routing of NAS signalling that is separate from functionality handles basic registration and mobility management, and other functionality for enhanced services.

- Investigate enhanced mechanisms and procedures for signalling for paging and reduce congestion in the control plane.

S2-2506942:

The study of the control signalling communication between UE and CN (i.e. Non-Access Stratum level communication) covers the following aspects

-	Identify if there is any potential improvement (e.g., minimized UE impact for verticals, lower number of messages both OTA, between RAN/CN and also inside CN) on the support for the UE connectivity-based functionalities in addition to the re-use of existing basic 5GS NAS mobility and session management handling mechanisms.

-	Investigate the UE-CN communication to support the UE interworking between 6G system and 5G system.

-	Investigate the possibility of designing a new common UE and CN communication mechanism for any new services without dependency to the UE connectivity-based functionalities.

NOTE: Potential dependency to RAN WGs and SA3 WG for security.

S2-2506988:

1.1.	Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services offered over the control plane.

S2-2507055:

WT#1.1 Study the support for control plane signalling for 6G for connectivity services and/or beyond connectivity services, including the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of independent non-access stratum functionalities depending on the type of typical user equipment (e.g. IoT vs eMBB).

c.	Whether and how to develop generic mechanisms (e.g. authorization, discovery) for UE-Core Network interaction to support operator services.

S2-2507135:

This Work Task investigates enhancements to the 6G control signaling framework, with a particular focus on NAS modularity and extensibility. The objective is to enable scalable, extensible, and service-aware signaling models that support both existing and future services.

This Work Task considers the following aspects:

- Whether and how to enable the introduction of a new non-access stratum functionality without impacting other NAS functionalities.

- Whether and how to identify a minimal set of NAS functionalities that does not get impacted by additional ones.

- Whether and how to develop generic mechanisms for UE–CN interaction to support operator-defined services.

- Definition of modular architecture and composition rules for NAS and control plane signaling.

- Mapping of modular control plane components to deployment environments (e.g., cloud-native CNFs, edge-distributed functions).

- Support for future signaling paradigms including intent-based and AI-assisted interaction models.

S2-2507200:

The technical scope for WT#1.1 include the following aspects:

1.1	Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to design the non-access stratum (NAS) to enable the introduction of a new NAS functionality (over control plane) without impacting other NAS functionalities. This includes how to efficiently route NAS message between UE and core network NFs and how to enable direct security procedure (e.g. authentication, authorization) between the UE and the target NF.

a.1.	Whether and how to identify a minimal set of NAS functionalities that does not get impacted by additional NAS functionalities.

b.		How to develop a generic mechanism for UE-Core Network interaction to support operator services beyond connectivity, e.g. to support positioning, Sensing, AI/ML, data collection, etc.

S2-2507353:

WT#1.1 focuses on UE-CN interaction for control signalling. The 6G design for UE-CN interaction for control signalling should consider the following:

- the system is architecturally modular, and implementation modular with as little independency from each self-contained module as possible;

- the system supports control signalling for connectivity services and beyond connectivity services;

- the system supports UE-CN interaction for control signalling over control plane;

- the system supports UE-CN interaction for control signalling over user plane;

- the design UE-CN interaction should be conceived considering at least the following aspects: cloud native, sustainability and energy efficiency, robustness and resiliency, etc.

S2-2506611, S2-2506975 proposes architectural requirements, architectural assumptions and/or key issues related to WT1.1 but no WT scope proposal thus it is assumed that the papers assume the WT scope as in the agreed SID.

**** First Change ****

# Annex A.X. WT#1.1

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#1.1 Study the support for control signalling for 6G for connectivity services and beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

**** Second Change ****

For Terminology clause:

Connectivity services: 3GPP connectivity offered to the UE and managed by the 3GPP network, including Registration Management (RM), Connection Management (CM), Mobility Management (MM), Session Management (SM), Reachability Management and Paging, Authentication and Security Management, Transport of UE Policy, essential and regulatory services.

Beyond connectivity services: all 3GPP services not related to connectivity services

**** End of Changes ****