SA WG2 Meeting #170	S2-2507889

25 – 29 Aug, 2025, Goteborg, SE	(was S2-2506925r2)

Source:	China Telecom

Title:	[WT#1.2, Localized Service Access] Support of Localized Service Access offered by PLMN in 6G

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution analyses the need of supporting support localized service offered by PLMN in 6G and proposes the scope and key issues for TR 23.801.

1. List of contributions for [WT#1.2 Localized service access]

The following is proposed as baseline as starting point for drafting the WT and KI(s) for Localized Service Access based on the list of Tdocs:

Table 1. List of contributions for WT#1.2 Localized Service Access

| S2-2506481 | 23.801-01: [WT#1.2] Working tasks and key issues for 6G localized network | China Mobile | Rel-20 | FS_6G_ARC |
| --- | --- | --- | --- | --- |
| S2-2506722 | 23.801-01: [WT#1.2, localized service access] Work task #1.2 scope for localized services access | Nokia | Rel-20 | FS_6G_ARC |
| S2-2506776 | 23.801-01: [WT#1.2, Localized network access] Discussion and proposal for Localized network access | ZTE | Rel-20 | FS_6G_ARC |
| S2-2506925 | 23.801-01: [WT#1.2, Localized Service Access] Support of Localized Service Access offered by PLMN in 6G | China Telecom | Rel-20 | FS_6G_ARC |
| S2-2507163 | 23.801-01: [WT#1.2, Local Networks] Introduction and justification of required studies for local networks | ROBERT BOSCH GmbH | Rel-20 | FS_6G_ARC |
| S2-2507298 | 23.801-01: [WT#1.2, Architecture Simplification and Impacts for Localized Service Access] Justifications and Proposals for 6G architecture simplification | Huawei, HiSilicon | Rel-20 | FS_6G_ARC |
| S2-2507300 | 23.801-01: [WT#1.2, localized service access] Support Distribute Subnetwork in 6GS | CATT | Rel-20 | FS_6G_ARC |

Capture of WI Scope and KI Description of the papers are listed in below:

Table 2. WT Scope and KI Description from Contributions to Localized Service Access

| Tdoc# | Source | WT Scope | KI Description |
| --- | --- | --- | --- |
| S2-2506481 | China Mobile | 1.2. Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access. (same as in SP-250806) | This key issue will address the general system architecture and function requirement to support localized network service. Different verticals always have different requirements for the localized network. The 6G network should be able to provide customized local network for different verticals, e.g., localized authentication for vertical UEs. For NPN network, this cannot be used currently because of NID, which is not supported by UE. Therefore, for localized network service access, there should be no UE impact. What’s more, network simplification should be considered when deployed the localized network. The aspects should be studied in this key issue include: - Whether and how to support the localized NFs to support basic network mechanism. - How to support the authentication and authorization locally, and how to keep the information alignment between the local subscription information and central subscription information. - How to support the UE access to localized network. - How to decide that the UE should be able to access to localized network or centralized network. - During the mobility, how to decide that the UE that should be access to localized network or centralized network, and how to support the service continuity. |
| S2-2506722 | Nokia | WT#1.2.x Scope for localised service access - Study enablers for localized services access via PNI-NPN/SNPN. NOTE: The idle mode mobility and service continuity between 5G PNI-NPN/SNPN and 6G PNI-NPN/SNPN is assumed to be covered in WT#2 - Study whether and how to support UE onboarding for NPN in 6GS. | (same as WT scope) |
| S2-2506776 | ZTE | No update in the Working task description No need for sub-WT. | KI#X: Study any optimization to support localized service access in 6G, this includes the following aspects: - How to access SNPN via PLMN, or access PLMN via SNPN, or access SNPN via SNPN, without dependency on N3IWF. - Whether and how to simplify the network when the network start or stop the localized service based on demand, e.g. without impact on the UE - How to improve the system that allow the operator to provision localized service on demand without any impact on existing services KI#Y: Study how to enhance the network resilience via localized service access. This includes the following aspects: - How to support localized service for commercial user when backhaul is not available/limited. - How to support disaster roaming between PLMN and SNPN, or between SNPN and SNPN. KI#Z: Study how to discover network for localized service. This includes the following aspects: - How to manage the localized service information supported by the network - How to use the information above to discover and select the network for proper localized service |
| S2-2506925 | China Telecom | WT#1.2 Scope for localized service access network from PLMN Operators Localized service access enables network operators to create customized and cost-effective solutions for the emerging 6G applications of diverse requirements. The target of localized service access offered by PLMN in 6G is to achieve: - Lightweight deployment in localized network, i.e., minimization of the numbers of NFs required for localized network operation with flexible scalability; - Failure resiliency in localized network, i.e., one localized network will not be impacted by the failure in the central network under specific conditions; - Interaction for functionalities complement between localized networks and central network, i.e., UEs can access NFs in the central network or other localized networks if not available in the access localized network; - Disaster recovery between localized networks and central network, i.e., when a localized network fails, the central network and other localized networks can take over its operations. | Solutions for this key issue will study how to support the localized service access from PLMN operator requirements defined in TR 22.870[z] and in normative stage 1 specifications (when available), including but not limit to: - Localized service network architecture - Identify the fundamental functionalities and capabilities within localize network to provide local service independently - Define interface and interworking procedures between localized networks and central network for coordination - Localized service/NF discovery and selection - Define effective mechanism to support efficient localized service/NF discovery and selection - Authorization and authentication of UE for localized service access - To support a mechanism to authorize and authenticate UEs to access localized services within the localized network from the PLMN operator without interaction with central network - To support a mechanism to authorize and authenticate UEs to access services outside the localized networks, i.e., from the central network or from other localized networks - Network resiliency for localized service network - To support fast recovery of services by providing localized networks when the PLMN of the same operator due to urgent events |
| S2-2507163 | ROBERT BOSCH GmbH | It is proposed to capture in the scope of WT1.2 in SP-250806 “1.2. Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.” the following aspects of local networks: - Study limited core network functionalities of local networks to achieve partial autonomy from the mobile network - Study intra-local network coordination, i.e. resource management within the local network - Further studies on architecture enhancements to enable local networks in 6G | (Not provided) |
| S2-2507298 | Huawei, Hisilicon | WT#1.2 Scope Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex. Define the overall 6G architecture as collection of capabilities and high-level functionalities considering the following sub work tasks to support 6G access network: - Study the 6G architecture simplification for lean and streamlined design, and architecture impacts for localized service access, see clause 5.X and 5.Y. Editor's Note: The scope is to be updated due to impact from other WTs, such as WT#3, #4, #5, #6. Annex A.X. WT#5 Scope Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex. - Support the management of the UE context type data. | 5.X Key Issue #X: Architecture simplification to support lean and streamlined design The existing NFs interdependency, stateful NF design and serial procedures create challenges on the path to improve network resilience, network scalability and network simplification. Roll out of new features or new services, network upgrades, network failure recovery, and scaling in/out, might produce a chain of affected network functions/functionalities, association update signalling storm, inefficient and imbalanced message distribution etc. In order to create lean and streamlined standards it is advantageous to simplify the overall system by dimensioning an appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations. This key issue will study the simplification of the 6G system architecture taking into consideration the following aspects: - how to support dimensioning an appropriate set of functionalities to minimize the inter-functionality or inter-function dependencies, e.g., minimize the signalling latency by adopting parallel processing for orthogonal functionality and inter-function interaction reduction via high cohesive network functions. - how to support uniform message forwarding and routing, NF discovery and selection. - how to support separation of NF data and logic, for example to minimize the signalling arising from network upgrade, network dynamic scale-in /scale-out, new feature/ service delivery, network failure recovery etc. Editor's Note: New KIs, derived from other WTs, such as WT#3, 4, 5, 6, are to be added. 5.Y Key Issue #Y: Architecture impacts to support localized service offered by PLMN Localized services can be offered via PLMN to support several services in public area, for example: 1) Provisioning local data connectivity service and voice service in the disaster area in response to an urgent event (e.g., disaster, emergency and DDoS events) to improve network resilience, as described in clause 5.6.2 of TR 22.870[xx]. 2) Verification of new network services (e.g., XR services) rolling out on the local area network, running test on the selected users in response to new business contract for fast service delivery, as described in clause 5.9.5 of TR 22.870[xx]. 3) Offering customized local area network service (e.g., customized capabilities, and more localized management) for vertical scenarios like factories or campuses, adapting to differentiated and changing demands on network services of various local stakeholders, to simplify the network and improve the network flexibility, as described in clause 5.9.6 and 11.9 of TR 22.870[xx]. To offer localized services, the operators need to configure part of the PLMN as a local area network. The local area network should be easy to deploy and use, provide adequate performance, be cost effective, sufficiently flexibile to meet demands etc. This key issue will study how to enable the 6G architecture to support certain level of local control for PLMN offered localized service access, including: - How to enable the architecture to support local control for control plane signalling, user subscription, policy management, topology management etc. - How to minimize the impacts towards PLMN when performing NF discovery and selection, message forwarding and routing between localized services dedicated network functions and PLMN services dedicated network functions. - How to support fast delivery of the new or updated services/capabilities for localized services with no/minimal disruption to existing services. - How to enable a UE to access the localized services, which are provided by dedicated network function instances within the PLMN. - How to authorize the UE, that is subscribed to PLMN and to localized services, to access the localized services when UE is in specific area, with minimum impacts to UE’s home network. Editor's Note: New KIs, derived from other WTs, such as WT#3, 4, 5, 6, are to be added. |
| S2-2507300 | CATT | It proposes to keep WT#1.2 as it is. | Key Issue #X: Support Distribute Subnetwork in 6GS Distributed Subnetwork is one of the solutions to support localized service access, which is an extension of the PLMN network. It provides network services with a certain degree of autonomy, localization, simplification, local authorization, topology hiding, cross domain collaboration and diversified functions in specific areas during specific time periods based on the needs of individual customers. In order to support Distribute Subnetwork in 6GS, the following aspects are to be studied: - Whether and how the Architecture, NFs and procedures enhancements to support Distribute Subnetwork in 6G. - Whether and how to simply the network to support Distribute Subnetwork start or stop base on the users demand. - Whether and how to support the subnetwork discovery and selection. - Whether and how to support the UE select and access the subnetwork. - Whether and how to support the subnetwork switch to guarantee the services continuity for UE. - Whether and how to manage the Distribute Subnetwork supported by the network. |

2. Summary of contributions for [WT#1.2 Localized service access]

The proposed KIs are focus on the following enhancement to meet requirement of localized service in TR22.870:

a. Localized network architecture to support different use cases and requirement

b. Interaction between localized networks and central network

c. Simplify the network when the network start or stop the localized service based on demands

d. Discovery and selection for localized network

e. The authentication and authorization mechanism

f. Network resilience

g. Control of UE's selection and access to localized network

h. Service continuity during mobility and interworking

Table 3. Category of KIs from contributions to Localized Service Access

| Tdoc# | Source | 5G features mentioned in KI | a. Localized network architecture | b. Interaction | c. simplify the network | d. Discovery and selection | e. The authentication and authorization | f. Network resilience | g. Control of UE's selection and access | h. Service continuity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S2-2506481 | China Mobile |  | X |  |  |  | X |  | X | X |
| S2-2506722 | Nokia | SNPN, PNI-NPN |  |  |  |  |  |  | X | X |
| S2-2506776 | ZTE | SNPN, PNI-NPN | X |  | X | X |  | X |  |  |
| S2-2506925 | China Telecom |  | X | X |  | X | X | X |  |  |
| S2-2507163 | ROBERT BOSCH GmbH |  | X | X |  |  | X |  |  |  |
| S2-2507298 | Huawei, Hisilicon |  | X |  | X | X | X |  | X |  |
| S2-2507300 | CATT |  |  |  | X | X |  |  | X | X |

Paper S2-2506722 (Nokia) and S2-2506776 (ZTE) specify optimization and enhancement of PNI-NPN/SNPN in KIs, while the rest papers do not.

It is suggested to describe the scope and KI for localized service access in TR23.801 as following:

For WT scope:

- Option 1.1: keep the original WT#1.2

- Option 1.2: if other topics in WT#1.2 break down to detailed sub-WT, then the same as Localized service access

WT scope pending for further discussion:

- whether to reuse/enhance PNI-NPN/SNPN or to identify new feature to support local network or localized service access in 6G.

For KI description:

The KI description should avoid solution preference.

- Option 2.1: use the same wording as WT scope if it takes option 1.2.

- Option 2.2: use different wording from WT scope.

**** First Change ****

OPTION 1.1 (keep the description in SP-250806):

Annex A.X. WT#1.2 Scope

The study includes the following high level work tasks, and the conclusion will consolidate the 6G architecture among all work tasks:

WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network:

1.2.	Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

(As proposed by China Mobile (S2-2506481), ZTE(S2-2506776), CATT(S2-2507300))

OPTION 1.2 (with detailed description):

Annex A.X. WT#1.2 Scope for localized service access

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

The scope of localized service access includes:

- Study the overall architecture design and enablers for localized service access in 6G

- Study network resilience via localized service access

WT scope pending for further discussion:

The baseline for WT/KI discussion

Option 1: use general description

-	Study the 6G architecture simplification for lean and streamlined design, and architecture impacts for localized service access, see clause 5.X and 5.Y. (Huawei, S2-2507298)

Editor's Note: The scope is to be updated due to impact from other WTs, such as WT#3, #4, #5, #6. (Huawei, S2-2507298)

- Lightweight deployment in localized network, i.e., minimization of the numbers of NFs required for localized network operation with flexible scalability; (China Telecom, S2-2506925)

- Failure resiliency in localized network, i.e., one localized network will not be impacted by the failure in the PLMN network under specific conditions; (China Telecom, S2-2506925)

- Interaction for functionalities complement between localized networks and PLMN network, i.e., UEs can access NFs in the PLMN network or other localized networks if not available in the access localized network; (China Telecom, S2-2506925)

- Disaster recovery between localized networks and central network, i.e., when a localized network fails, the PLMN network and other localized networks can take over its operations. (China Telecom, S2-2506925)

- Study limited core network functionalities of local networks to achieve partial autonomy from the mobile network. (BOSCH, S2-2507163)

- Study intra-local network coordination, i.e. resource management within the local network. (BOSCH, S2-2507163)

- Further studies on architecture enhancements to enable local networks in 6G (BOSCH, S2-2507163)

Option 2: use PNI-NPN/SNPN

- Study enablers for localized services access via PNI-NPN/SNPN. (Nokia, S2-2506722)

NOTE:	The idle mode mobility and service continuity between 5G PNI-NPN/SNPN and 6G PNI-NPN/SNPN is assumed to be covered in WT#2. (Nokia, S2-2506722)

- Study whether and how to support UE onboarding for NPN in 6GS. (Nokia, S2-2506722)

**** Second Change ****

Option 2.1 (use the same wording if WT scope takes option 2)

Option 2.2 (use different wording from WT scope):

5.X. Key Issue #X: Support for localized service access

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

Solutions for this key issue will study how to support the localized service access defined in TR 22.870[z] and in normative stage 1 specifications (when available), including but not limit to:

- Network architecture to support local control

  - Identify the fundamental functionalities and capabilities to provide local service access independently

  - How to support fast delivery of the new or updated services/capabilities for localized services with no/minimal disruption to existing services

  - Enable UE to access to localized service

  - Authorization and authentication of UE for localized service access

    - To support a mechanism to authorize and authenticate UEs to access localized services without interaction with PLMN network

    - To support a mechanism to authorize and authenticate UEs to access services provided by PLMN network

  - Define effective mechanism to support efficient localized service/NF discovery and selection

  - Study enablers for localized services access via PNI-NPN/SNPN. (Nokia, S2-2506722)

  - Study whether and how to support UE onboarding for NPN in 6GS. (Nokia, S2-2506722)

- Network architecture to support interworking

  - Define effective mechanism to support efficient localized service/NF discovery and selectionDefine interface and interworking procedures between localized service dedicated network functions and PLMN services dedicated network functions

  - How to access SNPN via PLMN, or access PLMN via SNPN, or access SNPN via SNPN, without dependency on N3IWF. (ZTE, S2-2506776)

  - How to support mobility and service continuity

NOTE:	The idle mode mobility and service continuity between 5G PNI-NPN/SNPN and 6G PNI-NPN/SNPN is assumed to be covered in WT#2. (Nokia, S2-2506722)

  - How to support disaster roaming between PLMN and SNPN, or between SNPN and SNPN. (ZTE, S2-2506776)

- Network resiliency via localized service access

  - To support fast recovery of services by providing localized service when the PLMN of the same operator fails due to urgent events

Study whether and how to support UE onboarding for NPN in 6GS. (Nokia, S2-2506722)

**** End of Changes ****