SA WG2 Meeting #170	S2-2507936

# 25 – 29 Aug, 2025, Goteborg , SE

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0738 | rev | 2 | Current version: | 19.3.0 |  |
| 3 |  |  |  |  |  |  |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures of data collection for NG RAN-sided models |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi, CEWiT, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, NR_AIML_air-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1) The following procedure is agreed in R3-255095 7.x.2 Positioning Data Collection Procedure Figure 7.x.2-1 shows the Positioning Data Collection procedure used to retrieve positioning information for a UE that is undergoing a positioning process. Figure 7.x.2-1: Positioning Data Collection Procedure 1. The LMF sends the NRPPa MEASUREMENT REQUEST message to one or more gNBs according to UL related positioning procedures described in Clause 8. 2. The gNB(s) determines that data collection is needed for the UE being positioned. 3. The gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and indicates that data collection is needed for the UE being positioned. Note: Steps 1 to 3 may occur while the LMF performs one or more of the positioning procedures described in clause 8. 4. The LMF sends a NRPPa POSITIONING DATA COLLECTION REPORT message to the gNB(s) which indicated in step 3 that positioning data collection is needed. The message includes information related to UE location and correlation information that enables the gNB to correlate the information related to UE location with information related to UL measurements (e.g., LMF Measurement ID and RAN Measurement ID). 2) The user consent check for data collection for AIML positioning was agreed during SA#108. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedures of data collection for NG RAN-sided models. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Positioning Information collection for UEs under positioning required by RAN 3 is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x(new), 6.x.1(new), 6.x.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

6.x	Procedures of data collection for NG RAN-sided models for AI/ML positioning

6.x.1	General

This clause describes the procedure for data collection by the NG RAN for input data to train or monitor the performance of the ML model for NG RAN-sided models.

6.x.2	Data Collection for NG RAN-sided models for UEs under positioning

Figure 6.x.2-1 shows a procedure for data collection for NG RAN-sided models for UEs under positioning.

Figure 6.x.2-1: Data Collection for NG RAN-sided models for UEs under positioning

Precondition:	The UEs are under positioning, and the network assisted and network based positioning is used.

1.	This procedure in Figure 6.11.2-1 can be used for data collection by the NG RAN for input data to train or monitor the performance of the ML model for NG RAN-sided models, where the NG-RAN requests for positioning information collection for a UE being positioned.

The steps 1-6 of network assisted positioning procedure are used as defined in clause 6.11.2, and the information to indicate that data collection is needed for the UE being positioned is included in the network positioning message in steps 5-6 as defined in NRPPa signalling in TS 38.455 [15].

2.	To provide the data collection for the UE being positioned, the LMF performs the user consent check by interaction with UDM as step 4 of data collection to train models for LMF-based AI/ML positioning based on NG RAN measurements as defined in clause 6.22.3. If user consent is not granted, then a failure indication with appropriate cause value is to be sent to NG-RAN, as defined in TS 38.455 [15].

3-4.	The LMF determines the data requested by NG-RAN based on the ground truth data of the UE. The LMF transfers a Network Positioning message to the NG-RAN node that requested the data collection for the UE as defined in TS 38.455 [15]. If the LMF is unable to provide the positioning information, a failure indication to be sent to NG-RAN with appropriate cause value as defined in TS 38.455 [15].

* * * End of Change * * *