3GPP TSG-WG SA2 Meeting #171	S2-2509320

# 13-17 October, 2025, Wuhan, China										(revision of S2-2508617)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0319 | rev | 2 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EAS Discovery Procedure with L-DNS for Local Offloading Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | EAS discovery may be performed with either EASDF or local DNS server, for both non-roaming and HR-SBO roaming. EAS discovery with L-DNS should also be supported for Local Offloading Management. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new clause: EAS Discovery Procedure with L-DNS for Local Offloading Management. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EAS discovery is not supported for Local Offloading Management. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change  * * * *

### 6.10.1	General

In edge computing deployment, to reduce the impact on a centrally deployed SMF on management of the edge computing related information, 5GS may support the Local Offloading Management.

In Distributed Anchor Point and Multiple PDU Sessions connectivity models (see clause 4.3), the existing means such as URSP and SSC mode are used to support Local Offloading Management.

In Session Breakout connectivity model, the Local Offloading Management only applies to the non-roaming PDU session. For Session Breakout connectivity model, if the AMF determines that the UE is within the Local Offloading Management service area and Local Offloading Management is allowed in the SMF selection subscription data, AMF selects/re-selects an I-SMF to support Local Offloading Management. This also includes the case when the UE is within the SMF service area. For the Local Offloading Management, the I-SMF is in charge of edge computing handling and supports following functionalities:

-	EASDF configuration and DNS message handling, as defined in clauses 6.2.1 and 6.2.3, instead of SMF.

-	EASDF discovery and selection is performed by the I-SMF instead of SMF as described in clause 6.3.23 of TS 23.501 [1].

-	Management and retrieval of EDI from NEF as described in 6.2.3.4.

NOTE 1:	The SMF can be also preconfigured with the EDI.

-	Receiving of the Local Offloading Management Information from PCF via SMF, which indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN.

NOTE 2:	When the I-SMF is inserted to support Local Offloading Management, the SM NAS message is still terminated to the SMF.

When the I-SMF is inserted for the Local Offloading Management, i.e. the inserted I-SMF providing Local Offloading allowed indication to SMF, the SMF does not discover, select and configure EASDF, as well as DNS message handling for the locally offloaded traffic towards the local part of DN.

The UE is unaware of whether the Local Offloading Management is applied or not for a PDU Session.

NOTE 3:	If local DNS server is deployed locally, the Local Offloading Management is also applied to Option C and Option D in section 6.2.3.2.3.

# * * * * End of changes * * * *