3GPP TSG-SA2 Meeting #171	S2-2509377

Wuhan, China, October 13-17 2025	(revision of S2-2507677)

Source:	Nokia, Lenovo, NEC

Title:	KI#3: proposed agreement of principles

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract: This document proposes to agree principles for KI#3.

# 1.	Introduction

This paper proposes a n initial set of principles for agreement and also outstanding topics that deserve further consideration. (yellow text is new text from last meeting version in S2-2507677)

In addition, Nokia has as a pen holder generated from these papers this version text including commented text in the P-CR and also an issue list is provided in clause 1.a.

| S2-2508377 | Interim principles for KI3. | Vivo |
| --- | --- | --- |
| S2-2508391 | KI#3 interim agreements. | CATT |
| S2-2508616 | Agreed principles for key issue #3. | China Mobile |
| S2-2508695 | KI #3, New Agreed Principles: Agreed Principle for Key Issue #3. | InterDigital Inc |
| S2-2508760 | WT#3: conclusion and agreement. | Huawei, HiSilicon |
| S2-2508765 | [KI#3] proposed agreement of principles. | [Nokia] NTT DOCOMO |
| S2-2508824 | Key issue#3, the analysis and interim agreement. | ZTE |
| S2-2508893 | KI#3: proposed agreement of principles. | Ericsson |
| S2-2509003 | Interim agreements for KI#3. | MediaTek Inc. |
| S2-2509091 | KI#3: Proposal for agreements and conclusion. | Apple |

# 1.a	Issues list

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-67.

# * * * * First change (all new text) * * * *

### 7.1.3	Agreed Principles for KI#3

The following principles are agreed for KI#3:

-	For NF selection, the NF profile is enhanced to include the parameters in the table below. This is a feature requiring the software to be upgraded in NRF and its consumers.

| Information | Description |
| --- | --- |
| Schedule of EnergySaving/not EnergySaving | The schedule of the EnergySaving/not EnergySaving states (NOTE 1), includes the EnergySaving or not EnergySaving state which applies at each time interval in the schedule. |
| Energy Priority Information | Value of Energy priority Set by the operator (e.g. taking into account various aspects such as usage of renewable energy or other energy related information outside the scope of this documents). It indicates the priority relative to other NFs of the same type.from an energy standpoint. For instance, if the Priority attribute is also used in the NF profile, the Energy Priority indicates a further priority based on energy among NFs with the same Priority attribute value in the NF profile. Whether the operator chooses the Energy Priority or the Priority attribute as primary evaluation criterion is up to the operator. |
| NOTE 1: EnergySaving/not EnergySaving refers to states of a network function which are defined in TS 28.310 [9]. |  |

-	For UP path selection and reselection, the UPFs are selected/reselected by using NF discovery and selection procedures which may be Energy-aware if NF profile is updated as per bullets above. Changes in NF profile of a UPF may trigger UPF reselection subject to SSC mode of the PDU sessions.

-	The NF Discovery procedure, based on operator policy, takes into account any Energy-related NF profile attributes query parameters. Both the NRF and the NF service consumer can take into account the Energy-related NF profile attributes if available, as part of the NF Discovery procedure.

-	A NF service consumer takes into account any Energy-related NF profile attributes from the NF profiles discovered from NRF using the NF Discovery procedure (and subsequent updates if the NF subscribed to NF profile updates), for the selection of a target NF.

-  The schedule information may be taken into account in the process of the selection of the NF by the consumer of the information in order to select the appropriate NF considering the future behaviour, if needed. The criteria for taking into account the schedule information is implementation specific.

### 7.2.3	Topics for further consideration for KI#3

The following principles are for further discussions for KI#3:

-	UP path(s) adjustment for PDU Session(s) may be triggered via DNAI (re)selection, taking into account energy related information, e.g. the overall energy consumption and/or energy efficiency of the UP path(s) for transmitting the service data flow

-	In case of multiple configured energy policies in network, the Energyenergy Priority is an array of priorities. Each entry of the array is the priority of the NF in the corresponding energy policy configured by the operator.

The NRF may store NF Profiles for N3IWFs and TNGFs. N3IWF and TNGF profiles may include Energy Information (e.g. Energy Priority Information or Schedule of Energy Priority Information). The AMF may query the NRF for N3IWF and TNGF profiles so that the information can be considered for N3IWF or TNGF reselection. N3IWF and TNGF profiles are be provided to the NRF via OAM.

# * * * * End of change * * * *