3GPP TSG-SA2 Meeting #171	S2-2509623

# Wuhan, China, October 13th – 17th, 2025		(revision of S2-2506339)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5521 | rev | 4 | Current version: | 19.5.0 |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1: Cancel procedure for subscriber specific IMS Events |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | After successful creation of a subscription for a subscriber specific IMS Event Exposure, the HSS may need to cancel the subscription due to the following reasons: - UE subscription (e.g. IMS Public Identity) associated to the created Event Exposure Subscription is withdrawn by the operator. - AF Authorization for the particular UE Subscription (IMPU) is revoked. This situations in HSS, require that HSS informs the NEF that the subscription for the subscriber specific IMS Event is cancelled. This is not yet covered in TS 23.228. These notifications are needed to inform the AF about the situation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support the scenario where HSS cancels a subscrition and notifies the NEF to that effect. The NEF then notifies the AF accordingly. When the subscribe request included a list of IMPUs, the Notify request includes the affected IMPU as notified by the HSS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AF will not be informed about the subscription was cancelled consuming resources waiting to be notified about events which will never occur for a particular UE subscription. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6.39.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.228 CR1655 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

5.2.6.39.2.3	Nnef_ImsEE_Notify service operation

Service operation name: Nnef_ImsEE_Notify

Description: This service operation is used by the NEF to report to the NF consumer received information by the NEF in a Notify Request from the IMS AS that detected the event previously subscribed.

Inputs, Required: Event ID, time stamp, Notification Correlation ID.

Inputs, Optional: Event Removal Indication, Notification Correlation Information, Event information (defined on a per Event ID basis), affected IMS Public Subscriber identity for subscriberspecific subscription.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# *** End of Changes ***