3GPP TSG-SA3 Meeting #123	S3-252619

# Goteborg, Sweden, 25 – 29 August 2025

Source:	ZTE

Title:	Pseudo-CR on KI on security and privacy aspects for local access

Document for:	Approval

Agenda item:	6.1.3

Spec:	TBD

Version:	v0.0.0

Work Item:	FS_NR_Femto_Sec_Ph2

Comments

Add a new key issue on security and privacy aspects for local access

* * * First Change * * * *

5.X	Key Issue #X: Security and privacy aspect for local access

5.X.1	Key issue details

According to RAN3 conclusions in TR 38.799[X], the NR Femto nodes reuse LADN and edge computing functionality to support access to local services. TS 23.548 [Y] elaborates following connectivity models to enable Edge Computing:

- Distributed Anchor Point: For a PDU Session, the PSA UPF is in a local site, i.e. close to the UE location. The PSA UPF may be changed e.g. due to UE mobility and using SSC mode 2 or 3.

- Session Breakout: A PDU Session has a PSA UPF in a central site (C-PSA UPF) and one or more PSA UPF in the local site (L-PSA UPF). The C-PSA UPF provides the IP Anchor Point when UL Classifier is used. The Edge Computing application traffic is selectively diverted to the L-PSA UPF using UL Classifier or multi-homing Branching Point mechanisms. The L-PSA UPF may be changed due to e.g. UE mobility.

- Multiple PDU Sessions: Edge Computing applications use PDU Session(s) with a PSA UPF(s) in local site(s). The rest of applications use PDU Session(s) with PSA UPF(s) in the central site(s). Any PSA UPF may be changed due to e.g. UE mobility and using SSC mode 3 with multiple PDU Sessions.

URSP rules, for steering the mapping between UE applications and PDU Sessions, can be used for any connectivity model and they are required for the Multiple PDU Sessions model.

For NR Femto scenario, unlike gNB, the NR Femto nodes are deployed outside operator domain and considered to be in un-trusted environments. This key issue is aimed to study the security and privacy aspects for local access in the presence of NR Femto node and study the security aspects for the local UPF which is tightly integrated with NR Femto.

5.X.2	Security threats

If the connectivity model(s) and connections for local access services in the presence of NR Femto node are not protected in NR Femto, local access can lead to loss of confidentiality, integrity and/or availability.

5.X.3	Potential security requirements

The 5GS shall provide mechanisms to secure the connection(s) for local access services for NR Femto, and ensure the connectivity model(s) used for local access is(are) secure and do not leak privacy details.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[X]	3GPP TR 38.799: "Study on Additional Topological Enhancements for NR".

[Y]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

* * * End of Changes * * * *