3GPP TSG-SA4 Meeting #132	S4-250839

# Fukuoka JP, 19th - 23th May 2025

Source:	Apple Inc.

Title:	Update on HEIF and JVET Support for Enhanced Colour Format Signalling

Agenda item:	9.10

Document for:	Information

# 1 Overview

This informative contribution follows up on our previous contribution S4-241989 [1] to inform 3GPP SA4 of the progress made on colour format enhancement technologies in both the HEIF file format and JVET bitstream signaling. The amendment to HEIF introducing "colour format enhancement derivation" has now reached the Final Draft Amendment (FDAM) stage [2]. In parallel, the "enhanced colour format information SEI message" has been further refined by JVET and is currently maintained in the Technologies under Consideration (TuC) document [3].

# 2 Details

Many existing HEVC and VVC decoder implementations support only 4:2:0 profiles. However, there is growing demand, particularly for screen content, gaming, and professional imaging, to deliver and render 4:4:4 content. The native delivery of 4:4:4 signals is often constrained by compatibility and deployment limitations in current infrastructure.

To address this, two complementary developments have been introduced:

- HEIF Amendment 1 (3rd edition) [2]: Introduces a method for colour format enhancement derivation, enabling a 4:4:4 signal to be reconstructed from a combination of multiple 4:2:0 image items using derived image mechanisms.

- JVET SEI Signaling [3]: JVET has proposed an enhanced colour format information SEI message that signals how layered bitstream based on 4:2:0 base layer can be interpreted or reconstructed as enhanced 4:4:4 content.

These tools enable backward-compatible delivery of enhanced fidelity content while supporting constrained decoding environments.

# 3 Proposal

It is proposed that SA4 note the progress of this work for future consideration in specifications related to file formats and bitstream signalling for media delivery and playback.

# References

- S4-241989, "On Derived chroma formats using layered HEVC", Apple Inc.

- MDS25061_WG03_N01525, "Text of ISO/IEC 23008-12 3rd edition FDAM 1 Support for tone mapping and other improvements", WG03 (MPEG Systems)

- JVET-AL2032, "Technologies under consideration for future extensions of VSEI (version 8)"