Source:	Nokia

Title:	On performance evaluation

Document for:	Discussion & Agreement

Agenda Item:	7.6 – DaCAS

# 1. Introduction

In the DaCAS work plan [1], agreement on the definition of performance requirements and performance evaluation methodologies for example solutions is anticipated in SA4-132. In this document a brief overview on possible approaches for subjective performance evaluation are discussed. In addition, recording scenario to cover objective tests defined in 3GPP TS 26.260 is presented and proposed to be agreed.

# 2. Subjective performance evaluation

Subjective performance evaluation of the DaCAS capture solutions is necessary to verify correct processing of the audio. One way to evaluate subjective performance is to listen to the DaCAS solution rendered audio created from the raw microphone audio recordings. If the raw microphone audio recording scenarios are clearly defined the subjective experience can be compared to the described recording scenario. This enables listeners to notice obvious bugs in the implementation, such as wrong channel orders, wrong configuration of the DaCAS solution or wrong directions of the resulting rendered audio. Care should be taken to make the raw-microphone recordings such a way that correct information is provided.

If multiple solutions or tunings are provided for the same device type, making subjective listening test comparing the solutions at least in terms of listener preference is easy. It is possible to simply process all the different solutions and run for example MUSHRA test without reference or P.800 ACR or P.800 CCR or a pair test to compare which solution listeners prefer.

# 3. Objective performance evaluation

In [2], a brief overview on available objective performance evaluation methodologies according to the 3GPP TS 26.260 [3] was presented. To obtain suitable recordings for the evaluation, following recording scenario is proposed to be included to the database, and further made by all available target devices. The same recordings can be used for assessing the loudness, frequency response, and directional information of the target device and example solution combinations. The proposed recording scenario covers the single talker scenarios. For assessing objectively performance with double-talk and more complex sound fields, the required recording scenarios and metrics are TBD. The proposed recording scenario is also attached as Recording_scenario_performance_evaluation.xlsx.

Table x: Recording scenario for single sound source scenario

| No | X (objective testing) |
| --- | --- |
| Sound source | Loudspeaker or HATS according to the clause 4.0.2 in TS 26.260 |
| Source signal | British English single talk sequence according to the ITU-T P.501 |
| Signal characteristics | Language: English Gender: Male, Female Range: 20Hz-20kHz Length:35.4 s Level: -27 dBRMS Sample Rate: 48 kHz Bit depth: 16 bit |
| Sound source calibration | According to the clauses 5.4.2 and 5.5.1 of 3GPP TS 26.260 for tested UE type. |
| Acoustic environment | According to the clause 4.0.3 of 3GPP TS 26.260 |
| Detailed Positioning | Source distance: according to the clause 5.4.2 of 3GPP TS 26.260 for tested capture mode and UE type Source direction: Loudspeaker diaphragm towards the UE Source angles: 0°, ±30°, ±60°, ±90° |
| Reference recording |  |
| Description/additional info | More information can be found from 3GPP TS 26.260 |

# 3. Summary

On subjective evaluation of DaCAS target devices and example solutions, a brief overview on possible approaches for evaluation are presented.

In addition, further details on the objective performance evaluation for DaCAS target devices and example solutions are provided. Additional recording scenario is proposed to be included in the recording scenarios database. Objective performance evaluation according to the 3GPP TS 26.260 can be then conducted based on the proposed recording scenario.

# References

[1]	S4-250668: “Work Plan for DaCAS v0.3”

[2]	S4aA250021: “On objective performance evaluation”

[3]	3GPP TS 26.260: “Objective test methodologies for the evaluation of immersive audio systems” version 18.1.0 Release 18