Agenda item: 	10.6

Source: 	Qualcomm Inc.

Title: 	[5G_RTP_Ph2] Scenarios and scope analysis for updating the data burst size indication

Document for	Discussion

# Introduction

At the SA4 #131-bis-e meeting, S4-250672 [1], which proposed to specify the update of the burst size indication, was agreed. But there were still some questions about the need and justification for such updates. This paper provides additional information on the need for the update, and analyzes whether such update is within the scope of the WID.

# Scenarios for burst size update

In WebRTC, the transmission of the RTP packets is handled by the pacer, which utilizes a prioritized round robin scheduling policy over a number of queues per SSRC stream, as quoted below from [2]:

## Life of a Packet

The typical path for media packets when using the paced sender looks something like this:

- RTPSenderVideo or RTPSenderAudio packetizes media into RTP packets.

- The packets are sent to the RTPSender class for transmission.

- The pacer is called via RtpPacketSender interface to enqueue the packet batch.

- The packets are put into a queue within the pacer awaiting opportune moments to send them.

- At a calculated time, the pacer calls the PacingController::PacketSender() callback method, normally implemented by the PacketRouter class.

- The router forwards the packet to the correct RTP module based on the packet's SSRC, and in which the RTPSenderEgress class makes final time stamping, potentially records it for retransmissions etc.

- The packet is sent to the low-level Transport interface, after which it is now out of scope.

Asynchronously to this, the estimated available send bandwidth is determined - and the target send rate is set on the RtpPacketPacer via the void SetPacingRates(DataRate pacing_rate, DataRate padding_rate) method.

## Packet Prioritization

The pacer prioritized packets based on two criteria:

- Packet type, with most to least prioritized:

  - Audio

  - Retransmissions

  - Video and FEC

  - Padding

- Enqueue order

The enqueue order is enforced on a per stream (SSRC) basis. Given equal priority, the RoundRobinPacketQueue alternates between media streams to ensure no stream needlessly blocks others.

The prioritized round robin queue is implemented in [3] and illustrated in Figure 1. It was shown in [4] through source code analysis that the pacing rate (the data rate of paced sending) is regulated by congestion control, and in general there is a time gap between adjacent packets departing the pacer and the duration of transmitting a data burst can be significant, e.g., the duration is 40% of the inter video frame duration (the packets of a video frame forms a data burst in this example) with the default value 2.5 for the scaling factor of the pacing rate. During the time of a data burst being paced out, the likelihood of new packets being added to the pacer may not be negligible.

As explained in [1], when the new packets have higher priority, to meet the low delay requirement, it is not desirable to delay the transmission of the new higher priority packets. The higher priority packets inserted into the pacer after the transmission of the first packet of the current data burst may be placed before the last packet of the data burst, therefore altering the data burst size. This requires an update of the already indicated data burst size.

Figure 1 Packet pacing in WebRTC [2, 3]

It is conceivable that other implementations of paced sending may result in scenarios which require an update of the indicated data burst size.

# WID scope analysis

Regarding whether updating the data burst size indication is within the scope of the 5G_RTP_Ph2 WID. TR 26.822 [5] states in the conclusion of Key issue #12 that “Do normative work for signaling burst size, when deterministically known.” For the scenario in clause 2, when the data burst size was indicated, it was indeed deterministically known. Therefore, updating the burst size indication is within the scope of the WID.

As new (higher priority) packets are added to the pacer while a data burst is being paced out, because it is undesirable to delay the new packets due to low-latency requirements, the new packets will be added to the data burst. The earlier data burst size indication needs to be updated.

# References

[1] S4-250672, On updating the data burst size indication, SA4 #131-bis-e meeting, April 2025.

[2] Paced Sending, https://webrtc.googlesource.com/src/+/refs/heads/main/modules/pacing/g3doc/index.md, retrieved May 13, 2025.

[3] source code for prioritized packet queue in WebRTC: https://source.chromium.org/chromium/chromium/src/+/main:third_party/webrtc/modules/pacing/prioritized_packet_queue.h

[4] S4-250219, Impact of WebRTC paced sending on burst duration, 3GPP SA4 #131 meeting, Geneva, Feb 2025.

[5] 3GPP TR26.822, Study on 5G Real-time Transport Protocol Configurations.