Agenda item: 	10.6

Source: 	Qualcomm Inc.

Title: 	[5G_RTP_Ph2] The need for redefining Time to the Next Data Burst based on experimental results

Document for	Discussion

# Introduction

The notation of time to next data burst (TTNB) originated from SA4. Since then, it has been defined in SA2 in TS23.501 as follows:

The time to next data burst, … is the interval between the transmission of the last PDU in the current data burst and the first PDU of the next data burst

In the 5G_RTP_Ph2 WID, SA4 agreed to indicate TTNB in the RTP header extension for dynamically changing traffic characteristics on the timeliness and accuracy of the indication:

- Conduct normative work on burst size, time to next burst, data boosting indication and the definition of data burst.

NOTE: RAN2 has indicated that TTNB may be useful if provided in time and is reliable. SA4 needs further evaluation before proceeding with normative work.

The timeliness requirement obliges the RTP sender to indicate the TTNB before the next data burst is generated, and this calls for prediction.

In the rest of the paper, we look at the consequence (in terms of prediction errors) if the SA2 definition of TTNB were adopted using the experimental data from S4-241835 [1].

# Prediction results

We present prediction results for the packet traces provided in S4-241835 [1] if the SA2 definition of TTNB were adopted. The prediction uses exponential weighted moving algorithm (EWMA) with the optimal weight found through exhaustive search. The prediction for the kth TTNB is calculated as

pk=αpk-1+(1-α)tk-1

where pk-1 is the previous predicted TTNB, and tk-1is the previous measured TTNB, and α is the weight in the range from 0 to 1.

The Python script in [1] produces two TTNB sequences, saved as a_b_bursts_windows_duplex.csv and b_a_bursts_windows_duplex.csv. In what follows, we present the predictions for them separately.

2.1 The first TTNB sequence (a_b_bursts_windows_duplex.csv)

The optimal weight of EWMA predictor is found by searching for the weight that minimizes the mean absolute prediction error over the entire first TTN sequence. The optimal EWMA is 0.82, as shown in Figure 1.

Figure 1 Mean absolute prediction error vs the WEMA weight for the first TTNB sequence

The measured TTNB (i.e., actual TTNB) vs the predicted TTNB for the first TTNB sequence is shown in Figure 2. It is seen that although the predicted TTNB well tracks the trends of the measured TTNB, it often over-estimates or under-estimates the measured TTNB with a significant prediction error.

Figure 2 The measured TTNBs and the predicted TTNBs for the first TTNB sequence

The cumulative distribution function (CDF) of the prediction error, i.e., the predicted TTNB minus the measured TTNB, is shown in Figure 3. It is seen that the prediction over-estimates by 14.7ms 10% of the time, and under-estimates by 19ms 30% of the time.

Figure 3 CDF of the prediction error for the first TTNB sequence

2.2 The second TTN sequence (b_a_bursts_windows_duplex.csv)

For the second TTN sequence, the optimal EWMA weight is 0.99, as shown in Figure 4. The measured TTNB and the predicted TTNB is shown in Figure 5. Figures show that the prediction over-estimates by 12.4ms 20% of the time, and under-estimates by 13.4ms 20% of the time.

Figure 4 Mean absolute prediction error vs the EWMA weight for the second TTNB sequence

Figure 5 The measured TTNBs and the predicted TTNBs for the second TTNB sequence

Figure 6 CDF of the prediction error for the second TTNB sequence

# Conclusion

Based on the prediction results in clause 2, we have:

Observation: Adopting the definition TTNB given in TS23.501 could result in significant error in the TTNB indication.

Proposal: SA4 needs to redefine TTNB to make the TTNB indication more useful.

# References

[1] S4-241835, Additional statistics for RTP Senders and Receivers in Solution #13, KI#12 for dynamic traffic, SA WG4#130, November 2024.