Source:	SA4 Audio SWG Chair, Vice-Chair (Ericsson LM, Orange)

Title:	Report from Audio SWG during 3GPP SA4#132

Document for:	Approval

Agenda Item:	12.1

## Executive Summary

The Audio SWG (47 registered delegates - 17 with confirmed attendance) met in 12 sessions (including extended sessions, excluding offline sessions). The SWG handled 96 documents, and the outcome is summarized below:

- Liaison

- The LS in S4-250773 from ETSI TC STQ is noted; it was felt premature to answer given that ambient noise testing is until definition in ATIAS_Ph2

- Maintenance (Rel-18 and earlier, Other Rel-19)

- CR to TS 26.253 in S4-250928 on IVAS algorithmic description was agreed.

- CR to TS 26.253 Annex A in S4-251135 was endorsed as basis for further work. It was initially intended to be presented to plenary, but to save time its status is proposed to be transferred to SA4 level.

- CRs to TS 26.114 in S4-250929 (Rel-18), in S4-250930 (Rel-19) on corrections for IVAS were endorsed. They were initially intended to be presented to plenary, but to save time their status is proposed to be transferred to SA4 level.

- IVAS_Codec_Ph2

- Update to IVAS-1 was agreed in S4-251073 with addition of IVAS-11 (non-BE conformance)

- Update to IVAS-2b (time plan) was agreed in S4-251074

- Update to IVAS-7b was agreed in S4-251100

- Update to IVAS-8b was agreed in S4-251138

- Update to IVAS-10 was agreed in S4-251072 with inclusion of the encoder verification report agreed in S4-251069, which is to be sent to TSG SA for approval of the fixed-point encoder delivery fulfilling 90% of the requirements.

- New Pdoc IVAS-11 (non-BE conformance) was agreed in S4-251129

- FS_ACAPI

- The TR was updated with identified gaps for WebRTC and common APIs in S4-250920, which was agreed.

- An updated time plan was agreed in S4-251047.

- FS_ULBC

- There was a discussion of the scope of the study and whether application scenarios beyond GEO should be part of the study. As a result, an update to the SID is proposed in S4-251148, which has not been seen in the SWG. A related proposal on the process was discussed with a revision in S4-251149 to be discussed in plenary (not seen in SWG).

- An initial version of the Pdoc was agreed in S4-251151. A revision implementing further agreed proposals in S4-251154 is to be presented directly to plenary (not seen in SWG). An additional proposed update for the Pdoc in S4-251162 was requested to be presented directly to plenary.

- An update of the TR in S4-251152 implementing agreed pCRs is to be presented directly to plenary (not seen in SWG). Two proposals for the TR on mouth-to-ear delay in S4-251125 and test methodologies in S4-251160 were requested to be presented directly to plenary (not seen in SWG).

- An update to the time plan in S4-251155 is to be presented directly to plenary (not seen in SWG).

- S4-251092 was noted without presentation then revised (directly by the Source, with no indication from the Chair) to S4-251108 to fix a company name. In this report, S4-251108 (not handled by the SWG) is noted without presentation.

- DaCAS

- 16 input Tdocs covering topics such as initiating TS 26.533, defining Pdocs in DaCAS, updating templates, defining microphone signal requirements, target devices, recording databases and other topics (license, performance evaluation, deliverables) have been processed.

- It was agreed to assign the following new roles under DaCAS:

- Editor of TS 26.533: Lasse Laaksonen (Nokia)

- Editor of DaCAS-1 on target devices and recording databases.: Nien Wu (Xiaomi)

- Editor of DaCAS-2 on test methodologies and requirements: Arvi Lintervo (Nokia)

- Editor of DaCAS-3 on deliverables: Mike Ye (ByteDance)

- An initial version of TS 26.533 (v0.1.0) in S4-251027 has been agreed.

- Three new Pdocs (DaCAS-1, DaCAS-2,DaCAS-3) v0.1 in S4-251057, S4-251058, S4-251059 (respectively) have been initiated and agreed.

- The time plan is revised in S4-251039 and agreed, including one telco

- ATIAS_Ph2

- Five input Tdocs on echo testing, ambiance transmission testing, diffuseness reproduction, nominal transmission levels and updates to the ATIAS-2 Pdoc on complex acoustic sound field transmission have been reviewed. In particular, the input Tdoc on echo testing in S4-250960 is agreed, others are noted.

- The ATIAS-2 Pdoc revised in S4-251020 (v0.4) is agreed including the output of offline editing reflecting input Tdocs.

- The time plan is revised in S4-251021 and agreed, including one telco

- Agreed telcos

- Telco on DaCAS: 16 June 2025, 16:00-18:00 CEST, submission deadline: 13 June, 16:00 CEST, host: Xiaomi

- Telco on IVAS_Codec_Ph2, 17 June 2025, 14:00-16:00 CEST, submission deadline: 16 June 2025, 14:00 CEST, host: Ericsson

- Telco on ATIAS_Ph2: 23 June 2025, 16:00-18:00 CEST, submission deadline: 20 June, 16:00 CEST, host: Xiaomi

7.1. Opening of the session, registration of documents

Stéphane opens the Audio SWG session on Monday 19 May 2025 at 14:00 JST and welcomes delegates.

Stéphane shows the initial Tdoc allocation available in the Drafts/Audio folder. This initial version of the Tdoc allocation is agreed.

7.2. IPR and antitrust reminder

The attention of the delegates to the meeting was drawn to the fact that 3GPP activities were subject to all applicable antitrust and competition laws and that compliance with said laws was therefore required by any participant of the meeting, including the Chair and Vice-Chairs and were invited to seek any clarification needed with their legal counsel. The leadership would conduct the present meeting with impartiality and in the interests of 3GPP. Delegates were reminded that timely submission of work items in advance of TSG/WG/SWG meetings was important to allow for full and fair consideration of such matters.

7.3. Reports/Liaisons from other groups/meetings

S4-250773

Presenter: Jan Reimes

Discussion:

- Stéphane: any comments? None

- Stéphane: any volunteer to draft reply LS

- Jan: can volunteer if needed

(later as part of the discussion of S4-251020)

- Stéphane: need to reply to STQ?

- Jan: depends on need to phrase officially what is important for us

- Andre: we already sent an LS and they replied, not needed

- Stéphane: let’s see what we have on ambient noise simulation, we can summarize that it may be premature then to send an LS if the ambience noise simulation is just work in progress

Decision: S4-250773 is parked then noted

7.4. Release 18 and earlier matters

S4-250924

Presenter: Stephane Ragot

Discussion:

- Stephane: Some overlap with 928, so a clean-up was made.

- Stefan D.: Two ways, E-byte and SDP, how do they interact?

- Stephane: In the init-mode the initial mode is selected, then coded format request can update this later in the call. Similar to bandwidth and bitrate.

- Sujeet: cf-sub-info is declarative, so it sets the range for sub-format request.

- Stefan D.: On cf-sub-info, this imposes a limitation on what a receiver can do, this would be in conflict that the receiver shall support all formats.

- Stephane: We assume all formats are supported on receive, now we specify the send direction only.

- Stefan D.: init-mode can be confused with mono-init.

- Sujeet: Sub-format in cf-sub-init and not modifiable.

- Stefan D.: Echo or reject, should never specify what it can receive.

- Stephane: Working offline to resolve comments. On name, e.g. icm.

- Stefan D.: What is the syntax of the init-mode, is it a string, what is it?

- Stephane: Good point, we should add some examples.

- Markus M.: This now introduces switching of sub-formats, which is not supported.

- Stephane: The comment applies also to basic coded format request, support of switching can be implemented, it is not rocket science.

Decision: S4-250924 is revised to S4-251025

S4-251025 is noted

S4-250928

Presenter: Lasse Laaksonen

Discussion:

- Lasse: Some additional corrections to be made, but can be put into Annex A CR.

Decision: S4-250928 is agreed

S4-250929

Presenter: Tomas Toftgård

Discussion:

- Stefan D.: LC3plus parameters are not spelled out, need to check offline.

- Document is parked

- Later discussion

- Tomas: Since corresponding CR to 26.253 is now endorsed, propose to endorse this one also.

Decision: S4-250929 is endorsed

S4-250933

Presenter: Srikanth Nagisetty

Discussion:

- Stefan D.: First observation, this proposal is application specific, not applicable for IVAS spec. UE types are not of relevance here.

- Srikanth: These can also be mapped to the codec format, but we concluded this parameter could be sent such that the

- Stefan D.: HDM is not lister, where are AR glasses, where is my watch?

- Srikanth: There is a possibility to enhance.

- Stephane: Would this apply also to mono. We already have these definitions.

- Srikanth: This proposal is related to IVAS, it is applicable to IVAS mono.

- Stephane: We can switch from hand-held to handsfree, this is now a device implementation we are not handling.

- Lasse: Some interesting points, impacting the choice of coded format, based on what the receiver is doing. Should we try to connect it to the rendering of the receiver rather than the mode. What is clear is the output.

- Srikanth: There is some flexibility.

- Rishabh: Agree with Lasse, what is not clear, we have coded format negotiation. Both devices are aware of their capabilities.

- Srikanth: There is no info from the receiver on the context.

- Stefan D.: The CMR seems to be misunderstanding. What do you add on top?

- Sujeet: The list of devices is not, more agnostic way to specify it.

- Stefan B.: Would like to express concern, do not see the issue. Codec that is flexible on receive side. The concepts we have right now seem sufficient, additional concepts might just confuse the implementers.

- Lasse: I disagree in sense that this input brings the idea that the audio experience can be improved. Capture side should make the decision on how the capture can be improved based on the feedback from the receiver on playback.

- Stephane: Would you favor sender or received based adaptation. Then we need to specify how both these should be handled, maybe both should not be used at the same time then.

- Review of revision

- Stefan D.: What does this add to the functionality using CMR to switch format? Should I send CMR in addition, or hope for the best?

- Srikanth: Highlights what the receiver is playing out.

- Lasse: The playout configuration tells the send side that there is a change. There will be smart implementations which can use this for optimizations. The coded format is independent.

- Sujeet: An example, binaural rendering to binaural room, the sender can select whether to send PI data or not.

- Stephane: Should clarify how it is used in practice. If you go from stereo to binaural, any recommendation, maybe sometimes it is used by the terminal and sometimes not. You may receive something you can only decode to mono (due to level), how to handle this?

- Srikanth: Playout config is info to the sender, it is PI data which is optional information.

- Stefan B.: What is meant by desired playout configuration? Why signal this? What is desired. We have a certain playout configuration, can we not just select codec format.

- Lasse: We can update the wording. It is however different than requesting a different coded format. This follows the principle that sender side knows best. Relying on the CMR says that the receiver side knows best, which conflicts with the understanding of immersive experience and how levels are defined.

- Stefan D.: What happen when this reverse PI data is used? Would the sender take action and change the format by themselves.

- Stefan D.: If I receive this, which entity will take the action?

- Sujeet: Media sender application.

- Stefan D.: In the example of AE, you should have reverse PI data which specifies this.

- Markus M.: Also think there is a problem if the action to be taken is not specified. In addition, I do not understand the application scenarios. If playout is changed, it is counterproductive to change the encoding.

- Lasse: If the change by the sender side is not required, do not send this PI data.

- Tomas: How would receiver know what happens?

- Markus: Send it to different devices giving different results is a nightmare.

- Stefan B.: For implementers, very difficult to optimize performance. At start of session, clear exchange of capabilities, the receiver can select from these capabilities based on playback. Just leads to confusion for implementers.

- Stephane: How do you handle multi-party, what if two receivers send differnt information.

- Sujeet: If MCU, might not allow this PCS.

- Lasse: We can provide some examples.

- Stefan D.: On general level, not only related to this contribution, additions for payload handling should be testable. There would be test specifications.

- Lasse: Agree on some level, but would not like to limit differentiation in the market.

- Lasse: Is this blocking PI data at this meeting?

- Stefan D: PI data and response should be clearly understood.

- Stephane: 26.130 does not yet include IVAS, but it could be good to add PI data testing. Otherwise, there is likely very different implementations.

| E-mail Discussion | Srikanth Nagisetty on Tue, 20 May 2025 07:35:42 +0000 Srikanth Nagisetty on Thu, 22 May 2025 03:58:30 +0000 |
| --- | --- |

Decision: S4-250933 is revised to S4-251133

S4-251133 is noted

S4-250954

Presenter: Lauros Pajunen

Discussion:

- Stefan D.: How to proceed, is this the basis for further work?

- Lauros: That is the idea.

- Stephane: If there are interop issues, it would be justified with Rel-18 correction.

- Lasse: I agree, and another point is that we need to see what we call a feature. A lot of corrections are minor, and largely related to the optional PI data feature. This could be seen as a feature.

- Stefan D.: Agree we should have for Rel-18 all interop related stuff, e.g. E-byte. However, other features should go to Rel-19.

- Sujeet: We have to see feature by feature. E.g. early initialization is also improvement needed for Rel-18.

- Stephane: Makes sense to park this Tdoc. Some dependency with GSMA, and if Rel-18 is used, we need to be careful what is in releases.

- Stefan D.: Are we at all aiming for two CRs? Regarding GSMA, we should not push too many things. Maybe everything is not needed for GSMA.

- Stefan B.: A bit dangerous discussion, need to make sure to not create interop issues. This Tdoc is properly allocated to Rel-18 and other Tdocs can be considered on case-by-case basis.

- Lasse: Such approach makes sense.

- Revision reviewed

- Stephane: Please add Orange to sources.

- Stefan D.: Is this a mandatory parameter, there is a shall.

- Lauros: No, it is not mandatory.

- Stephane: Maybe you want to separate what you support and what is used as initial state. If not present, what is the default value?

- Lauros: There is no default value.

- Sujeet: If not present in SDP, nominal would make a lot of sense.

- Rishabh: If not present, we should not mandate any default value, it is up to the implementer.

- Stephane: For testing, then you do not know what you are testing, you would need to test all 16 levels and match which one is the default.

- Sujeet: It is not mandated in the answer, so the parameter may be rejected.

| E-mail Discussion | Lauros Pajunen (Nokia) on Thu, 22 May 2025 02:05:13 +0000 Lauros Pajunen (Nokia) on Thu, 22 May 2025 03:35:32 +0000 |
| --- | --- |

Decision: S4-250954 is revised to S4-251135

S4-251135 is endorsed

S4-250956

Presenter: Lauros Pajunen

Discussion:

- Stefan D.: On the onset, since we have DTX we have talk spurts, when goes from inactive to active. The marker bit is not only used by the receiver but also by the network.

- Sujeet: DTX is not always on, we are not proposing to remove the use of the marker bit.

- Lauros: Onset is more generic term.

- Stefan B.: Would it just be a single frame where the flag is raised.

- Sujeet: Setting of marker bit would be as now for mono, but for IVAS it is not the same meaning. Marker bit not giving enough information.

- Stephane: If we have PI data for mono, this would not apply. We could have a note that PI data should not be used for mono.

- Stefan D.: DTX off is also possible in mono. Not sure we should change the ruleset used in IETF for audio.

- Sujeet: Do not under what the problem is. Would be fine to call it talk spurt. Receiver has no idea of which channel there is an onset. No need to change anything on the sender side.

- Stefan D.: If we have this feature, the IVAS encoder should also expose this information.

- Sujeet: I can have my own VAD, it does not need to be part of the IVAS codec.

- Lauros: Also in EVS, if the are multiple channels, the marker bit is not enough.

- Stefan D.: One encoder might be inactive the other active, that is the reason for the text in EVS.

- Stephane: What is default value of capture-type? Here we offer only three values, but what if you want to test all possible value. You would need to use PI data. From testing point of view, it would be better to just use capture-type. Would be good to clarify how it would be used in real calls and for testing.

- Lauros: You would be allowed to use all suppression levels within the range (on sender side).

- Arvi: On the granularity, we wanted to represent the prominent use cases.

- Rishabh: On the naming, capture-type seems too broad. The parameter list can have all values, but why have all three, would not a range be enough.

- Lauros: You would start with the first value.

- Stefan D: Agree the name is not optimal. Sets the default values.

- Lauros: Even is PI data is not supported the levels used will be negotiated.

- Stefan D.: New features, less relevant for interoperability.

- Lasse: On the limiting of the capture side noise suppression. If sender wants full control, it will not offer this parameter.

- Sujeet: On change 1, similar to what is used in VR stream, but not focusing on HMDs.

| E-mail Discussion | Lauros Pajunen (Nokia) on Tue, 20 May 2025 08:42:20 +0000 Tyagi, Rishabh on Wed, 21 May 2025 02:45:26 +0000 Lauros Pajunen (Nokia) on Wed, 21 May 2025 06:09:57 +0000 |
| --- | --- |

Decision: S4-250956 is revised to S4-251037

| E-mail Discussion | Lauros Pajunen (Nokia) on Tue, 20 May 2025 08:42:20 +0000 Tyagi, Rishabh on Wed, 21 May 2025 02:45:26 +0000 Lauros Pajunen (Nokia) on Wed, 21 May 2025 06:09:57 +0000 |
| --- | --- |

S4-251037 is noted

7.5. IVAS_Codec_Ph2 (EVS Codec Extension for Immersive Voice and Audio Services, Phase 2)

S4-250761 is revised to S4-251001

S4-250778

Presenter: Liangping Ma

Discussion:

- Lauros: There are PI data with variable sizes.

- Liangping: That is correct, this scheme will not handle that

- Stefan D.: This concept has some limitations, e.g. PI type can only come once. Can easily lead to 2-byte HE only.

- Sujeet: Bitrate allocation for PI data would be part of SDP, so that would not be problematic. To have a single header extension is to me very useful.

- Stefan D.: There can be 14 different types and 16 bytes.

- Sujeet: Would like these smaller representations to be verified.

- Stephane: How consistent is this with the video part of HE?

Decision: S4-250778 is noted

S4-250784

Presenter: Anssi Rämö

Discussion:

- Stefan B.: P.800 ACR with spatial audio is very tricky. We have seen in this group that naive listeners did not understand the benefit of spatial audio. Here we get again a proposal for such test. Would strongly recommend not doing such test, and also separating formats just on categories.

- Anssi: Here there is no comparison between mono and spatial. Regarding categories, there are different formats and completely different material, so it is impossible to compare the categories. This is especially interesting for marketing purposes.

- Eleni: People tend to prefer stereo because of wider spatial image. Why bitrate switching.

- Anssi: On bitrate switching, would stress the codec, but if it is too early, we can leave these out. I have prepared a draft.

- Milan: Share the concern by Stefan B. It might work with some material, but can give confusing results for other material. This is somewhat dangerous for the characterization. Also share the view that bitrate switching should not be part given there were no requirements on seamless switching. Inconsistency in table 1, 3 categories using model-based.

- Anssi: Should be correct.

- Erik: Removing mono made it a bit more acceptable, but would be interesting with test results. Would we now compare formats, e.g. on 24.4 kbps. This would be unfair comparison.

- Anssi: The whole point is not to compare the categories.

- Takehiro: Would recommend some trial test before the characterization.

- Anssi: We have good experiences with ACR for spatial audio.

- Stephane: Better if you can bring evidence of benefits. The purpose for the characterization is to guide the users of the codec.

- Revision reviewed

- Rishabh: For ACR more fundamental question if the spatial aspects can be tested. It could give good idea on signal aspects.

- Anssi: In this test all tests are spatial, do not see the issue. Test is constructed so that you cannot compare formats. Normal ACR scale, naive listeners.

- Stefan B.: I can soften my view, we are not any longer comparing to mono. We will get rate distortion curves. Maybe we will see that SWB is better than WB, good to communicate.

- Erik: Looks good with the adaptations.

- Stephane: For music and naive listeners, there could be issue with taste. WOuld MC be involved?

- Anssi: Yes would go through MC.

- Stefan B.: Would be good if we could get some more time to potentially also carry out such test. E.g. check saturations.

- Anssi: Time to agree now.

- Lasse: What would be the date when everything is available?

- Milan: This is limited by adhoc call, and the deadline for the collection of the material.

- Milan: Would it be the same statistical analysis as DCR? Can integrate this contribution for now.

Decision: S4-250784 is revised to S4-251043

| E-mail Discussion | Anssi Ramo (Nokia) on Wed, 21 May 2025 01:31:12 +0000 |
| --- | --- |

S4-251043 is agreed. Proposal is integrated into IVAS-8b, currently within brackets. Conditions may potentially be revised in a telco.

S4-250854

Presenter: Rishabh Tyagi

Discussion:

- Markus M.: Document also contains some with HiFi DSP which are still missing, are there now numbers? Would other DSP platforms be included?

- Rishabh: Not yet. Would be good to include other platforms.

- Markus M.: Would be good what type of optimizations may be used, we know some of them have issues.

- Rishabh: Would appreciate input from other companies.

- Stefan D.: For EVS conformance different optimizations were evaluated, and the conclusion was that fast math should not used, and the tolerances were then much stricter than allowing such optimizations.

- Stefan D.: Maybe the goal of the conformance should not just be that it sounds good.

- Rishabh: Thresholds for fixed vs float should not be the same, we should use a threshold that makes sense for float.

- Erik: Here encoder and decoder are changed at the same time, which is not something we would see here.

- Rishabh: Yes, we have the three combinations.

- Stefan D.: Results should go to 26.252. Would also be good to store results in a TR.

| E-mail Discussion | Tyagi, Rishabh on Wed, 21 May 2025 23:47:51 +0000 Tyagi, Rishabh on Wed, 21 May 2025 23:50:45 +0000 |
| --- | --- |

Decision: S4-250854 is agreed. Contribution is integrated into a new Pdoc IVAS-11 with Rishabh as editor.

S4-250862

Presenter: Tomas Toftgård

Discussion:

- Milan: WMC tool also needs an option –s for BASOP code to skip instrumentation. Could add that BASOP operators should also not be instrumented.

- Markus M.: G.191 filter tools are not yet update in STL. Who will feed that back to ITU, joint SA4 activity or contributing party (Orange)?

- Stefan D.: There have already been LS to ITU in the past. Now Github is used, so maybe best if individual companies can make the contributions.

- Stephane: Orange can do such updates.

- Erik: Update to bs1770demo is likely done.

- Markus M.: 2 LS were send but there was no action.

- Anssi: Update for MASA C Reference can be provided.

- Markus M: Can anybody use the new version? Is the license the same?

- Tapani: It should still be possible to use for this work.

- Erik: bs1770demo, the rms is in the main STL.

- Eleni: Brackets in Annex C.

- Stephane: Would be fine to remove them.

- Revision was reviewed

- Erik: Additional reference for bs1770 demo can be removed.

Decision: S4-250862 is revised to S4-251100

S4-251100 is agreed

S4-250874

Presenter: Marek Szczerba

Discussion:

- Anssi: Feel that this testing is very beneficial since it runs with live headtracking.

- Stefan B.: Two alternatives, both are using the HDM?

- Marek: Yes, only the options differ. Demo application is available on request.

- Rishabh: Is the option for room reverb available for all bitrates, how was the bitrate selected for the different formats.

- Marek: Rendering mode differ, aim is to cover all cases.

- Rishabh: SBA, highest bitrate.

- Marek: To align bitrates, not having all high, all low.

- Stefan B.: In the test carried out, a clear preference using room acoustics compared to no room acoustics. Should there not be a scene where no room acoustics would be the right.

- Marek: This would be beyond characterization even though it is interesting to have a condition like that.

- Stephane: Can we skip the anechoic reference at all. It compresses the results.

- Marek: The aim of the test is to evaluate that IVAS can renderer the room acoustics correctly. The issue, there is no golden reference.

- Stephane: We could consider various comparisons in CCR

- Andre: Here we address the comments on the previous contribution. There are many options, but we need something for the characterization, and we should converge.

- Markus M.: Second the comment on the bitrate, low and medium rates, unclear what comes from the codec and the renderer. Ambisonics contains already part of the room, would it not be interesting to test objects. Who will run these tests?

- Marek: Could be considered, but we are now limited in time. Philips would be ready to run tests, but we invite other parties to run these as well.

- Stephane: Would be acceptable to run it with option A.

- Milan: Would also support.

Decision: S4-250874 is agreed

S4-250914 is revised to S4-251013

S4-250915 is revised to S4-251012

S4-250916

Presenter: Stefan Döhla

Discussion:

- Stephane: Should we have a review by RTC? They have been using header extensions e.g. for split rendering.

- Stefan D.: Not only header extensions, but SDP should also be reviewed by RTC

- Sujeet: Not sure we also need header extensions for the PI types. Would be specified if used in other specs. There are some things that need clarification. You can’t associate PI with speech frame.

- Stefan D.: Many of them are just a marking, but it is possible to use RTP timestamp. There is base overhead, but if you have header extension in any case.

- Sujeet: Other proposal has more unified definitions.

- Stefan D.: The bitrate of several PI data is not neglectable, and this would be one way to put one PI data.

- Sujeet: Would like to have something that has less overhead.

- Stefan B.: Mainly on RTP header extensions, we need to take decision whether to use header extensions, then which parameters it would be. Should also be consistent.

- Stefan B.: On mono-init, we need to think about it, there are other ways also. In any case it should be separate.

- Stefan D.: No issue reusing the PI data structure, but we would then also need 2-byte header extensions. We can discuss this.

- Rishabh: On quaternion quantization, have not had time to verify the impact.

Decision: S4-250916 is noted

S4-250926 is revised to S4-251011

S4-250934

Presenter: Lasse Laaksonen

Discussion:

- Stephane: What is meant by noise suppression recommended?

- Lasse: If the receiver has the control, this could be a way to recommend it to the other side.

- Stephane: Is it not more clear to have a clear statement, either applied of not.

- Lasse: This would be fine, to have 11 as reserved.

- Rishabh: What would noise suppression applied or not applied add on top of capture-type?

- Lasse: These are different parameters and PI types.

- Lauros: Are you referring to the SDP? This would be the indication of what is applied right now.

- Rishabh: Not clear what is the additional value.

- Stefan B: Would like to be cautious not agree things that are not fully understood.

- Stefan D.: Introduction states that PI data is of relevance for rendering, but whether this is something we need to support in IVAS is not clear.

- Stefan D.: On the noise suppression control, there would be some privacy concerns controlling e.g. the audio focus.

- Lasse: Noise reduction and audio focus are key elements for immersive audio. Maybe there could be a privacy mode where the receiver control would be limited. Implementation-wise then you do not follow the PI data anymore, or do not negotiate it to be used. These features depend on many factors, and we need to enable the control of these parameters. Would be fine to leave out the audio property part.

- Stefan D.: The new thing is that the remote side can control my capture.

- Lasse: This capability has been discussed for some time in SA4.

- Rishabh: Agree that control for noise suppression has been there earlier. It would be appreciated if the new fields are left out for now. Please remember these are optional features.

- Stefan B.: Controlling the noise suppression should not imply any privacy issue. Audio focus is maybe something different.

Decision: S4-250934 is noted

S4-251001

Presenter: Milan Jelinek

Discussion:

- Document was reviewed and edited during editing session.

- Milan proposes to go through the highlighted text, to see what can be agreed.

- Discussion on lab allocation and need for material in native languages for pre-produced content categories.

- Mixed content and music or generic audio, we agree the language is not of relevance.

- Revision presented, includes further updates

- Stephane: Would propose to specify randomizations in the test plan.

- Tomas: This can be addressed later, currently it is not specified who would provide the randomizations.

| E-mail Discussion | Milan Jelinek on Wed, 21 May 2025 02:58:13 +0000 Fotopoulou, Eleni on Wed, 21 May 2025 08:08:32 +0000 Milan Jelinek on Thu, 22 May 2025 00:44:07 +0000 Multrus, Markus on Thu, 22 May 2025 01:34:47 +0000 Milan Jelinek on Thu, 22 May 2025 02:21:30 +0000 |
| --- | --- |

Decision: S4-251001 is revised to 1138. Content of 1001 is agreed as basis for further work.

S4-251005

Presenter: Tomas Toftgård

Discussion:

- Stefan D.: We need to check that it is clear which version, without reading the IVAS bitstream. Spherical and Cartesian

- Lauros: Resolutions would be good. Non-ISM versions of e.g. orientations, but for ISM there could be different PI names.

- Tomas: Yes, that sounds reasonable.

- Rishabh: Would like to verify representations

- Stefan D.: Other alternative is to use 8-bit quaternions

Decision: S4-251005 is agreed

S4-251011

Presenter: Erik Norvell

Discussion:

- Erik: Not sure the document is needed, would be fine to note.

Decision: S4-251011 is noted

S4-251012

Presenter: Erik Norvell

Decision: S4-251012 is revised to S4-251072 including the updated encoder verification report.

S4-251072 is agreed

S4-251013

Presenter: Erik Norvell

Discussion:

- Stephane: Which float version is used?

- Erik: It is always the version that corresponds to the tested BASOP version.

- Stephane: At least for the stereo-downmix, I think was used.

- Takehiro: No, there was no mismatch between the versions. Some small (but statistically significant) difference for 13.2 kbps.

- Stefan B.: Would like to add a sentence

- Tapani: Are we pointing out the specific critical issues in the report?

- Tomas presented a proposed cover page for the SA approval

- Stephane proposed some editorial changes which were included.

- Erik: Some figures are not consistent; I will update those.

Decision: S4-251013 is revised to S4-251069 including the cover page, and updated conclusion and some further corrections to the figures.

S4-251069 is agreed

S4-251073

IVAS-1 is updated with the new Pdoc IVAS-11.

Decision: S4-251073 is agreed.

S4-251074

IVAS-2b is updated to delay the finalization of IVAS-7b and IVAS-8b until July and include proposed telco and further details on the non-BE conformance.

Decision: S4-251074 is agreed.

S4-251129

Discussion:

- Markus M.: How would new results be added?

- Rishabh: Would propose to not remove earlier results by default.

- Stefan D.: In the end we like to have it in the TS.

Decision: S4-251129 is agreed.

7.6. DaCAS (Diverse audio CApturing System for UEs)

S4-250820 is withdrawn

S4-250821

Presenter: June Sun

Discussion:

- Matti: see measurement, closer to real life, have commercial, measure from harware with no dim

- June: no anechoic chamber, but considered all influence

- Matti: see fig 1 parameter

- June: no info on mic

- Rishabh: on SNR in tab 1, claeify how SNR was obtained?

- Mike: before quantization

- Rishabh: is there an agreed method to compute SNR?

- Mike: see nokia recording, a weightrd vfilter

- Andre: simulzytion of mic, assumes sensitivity of all micrs are same

- Mike: can simulate any kind of paramter in simulation, here yes, sippose capsule are tje same

- Andre: similarly, right now noise mix may be the same, but can be decorrelated

- Mike: yes, noise is decorrelated

- Andre: distortion or assume linear?

- Mike: assume linearity, capsule working in linear range, no distortion

- Srtefan B: on BEM simulation, and non ideality of mics, you are simulating what? How verify that simulation is correct? Any kind of possibility to give experimental proof that simulation corresponds to physical reality?

- Mike: BEM simulates to IR to center point, then use propose original signal, for verification target device OS has some permission such that we cannot access raw microphone data, can only verify by geometry of internal cavity and port dimension, if crossvalidate, no, but should be fine

Decision: S4-250821 is agreed

S4-250822 is revised to S4-251009

S4-250823

Presenter: Nien Wu

Discussion:

- Stefan B: several contributions provide scenario 1 and scenario 2, same scenarios and hamronize descriptions? Or scenario per device?

- Nien Wu: scenario updates to give more freedom, included range, most recordings needed

- Arvi: would be good to one harmonized method for all devices used, we are quired to have some flexibility, I am ok with changes, on length of signals we have a further proposal to allow bit short sweep, good if similar manner

- Stefan B: one base description with all common things and then variation per device, relative to base description, otherwise confusing and will be more effort

- Stéphane: acceptable database

- Arvi: elevated sources shoudl be included or only zero elevation

- Nien Wu: will update later

- Stefan B: before agreeing on this, same for other scenario, spend time to harmonize scenarios, look at corresponding sound files

- Marek: in both cases, sweep starting at 20 Hz, high end loudspeaker can expose to non lineariries and also capture devices only have direction characteistics for these low frequencyies? Consioder that low ? What kind of additional preprocessing to be applied?

- Nien Wu: using 30 Hz for lower frequency, most UE cannot get lown for measurement can have large range, for UE there is data, useful to collect information

- Arvi: in initial proposal thjere is suggesiotn reference rcording from reference mike, so know loudspeaker capability, defined signals siuch that same for everyone, based on used loudspeaker it could be chedked

- Stéphane: note?

- Nien Wiu: update later based on feedback

| E-mail Discussion | Nien Wu 吴宁航 on Wed, 21 May 2025 00:51:09 +0000 |
| --- | --- |

Decision: S4-250823 is noted then revised (by mistake) to S4-251040

S4-250824 is withdrawn

S4-250851

Presenter: Wang Bin

Discussion:

- Lasse: question on Pdoc on target devices, difference to clause in draft TS? Collect all of target devices in this Pdoc?

- Wang bin;: content for spec, may have coming target devcices in Pdoc then extract

- Srikanth: difference between prototype and smarpthone?

- Wang Bin: device on market vs. Device on TR for FS_DaCED

Decision: S4-250851 is agreed

S4-250852

Presenter: Wang Bin

Discussion:

- Wang Bin: received comment, Xiaomi legal is ok with this update

Decision: S4-250852 is noted

S4-250853

Presenter: Wang Bin

Discussion:

- Wang Bin: cane merge description, invite corresponding proponent to provide updates for specification

- Stéphane: brackets?

- Wang Bin: not needed

- Stefan B: other document on requirements? Or requirements also in this document? On raw microphone signals like SNR, frequency characteristics? Expected in this TS?

- Wang Bin: proposal is to catch that in Pdoc, when Pdoc is defined can get information

- Stefan B: eventually wil lbe in this document?

- Wang Bin: yes

- Stefan B: placeholders for sections, nice structure containing necessary structures to have requirements

- Wang Bin: two TSs?

- Stefan B: various options, requirements could be in a normative annex, can be fine in one spec

- Stéphane: who is editor? By default it would be the Rapporteur?

- Andre: in this document what is a technical specification? All this content is suitable for a TR, nothing normative or requirement for implementation or interop? Wonder if this portion is TR, what is TS?

- Wang bin: in clause 4 target devices, then example solutions that can be implemented, now structure of TS, clause is needed to make it appropriate for a TS

- Andre: is this an example and sufficient to know how it works for a device, trying to understand what is implication of material produced by DaCAS, just an example, means this can be implemented, here not saying mandatory for device but give solution, we can it example but this can be implemented, two points, one is we do not mandate, second we define example solutions

- Wang Bin: will make a Pdoc then see

- Stefan B: not extend too long, but see we get normative things, does not mean mandatory, they could be a description of upmix mechansmis that could be normative spedification text and requirements also normative then possibility to remind more of TR, then put such things in an Annex, not see problems

- Stefan D: what will be normative and justifiy a TS, if just examples, then TR, examples are more a TR work, distringuish what is normative aspects of this work

- Lasse: we have requirements defined and examples need to meet these requirements, in that sense they are normative but not mandatory

- Stefan D: if requirements, what is right home, could be related to ATIAS, ideally all capturing could be in ATIAS

- Qualcomm: fine to have a working assumption but may revisit if content is in TS or TR

- June: volunteer

- Lasse: also volunteer

- Stéphane: check offline for TS Editorship

Decision: S4-250853 is revised to S4-251027

S4-250856

Presenter:  Wang Bin

Discussion:

- Wang Bin: includes update from Nokia and XR device

- Stefan B: for coordinate system two competing version geometric center of smarpthone then I saw still in lower left corner, confusing if conflicts

- Wang Bin: right, two version, Nokia is supplement, give flexibility to describe device

- Matti: provided alternative, because our devices do not have display in same way, can support, center of gravity may depend on device

- Stefan B: not allow too many options, just single definition, on smartphone agree on single definition

- Markus; naming template is confusing, uodate excel sheets

- Wang bin: change name, includes template and device

- Srtikanth: clarify spatial capture algortihm, any agreed definition

- Wang Bin: no clear definition

- Stefan B; purpose is clear, see contriubtion from Nokia, signals from various positions, signals recorded, triggger and geometry information (where sound was played), to develop upmix method

- Srikanth :

- Eleni: under DaCAS

- Hiroyuki: differebnce between dev and evluation

- Wang Bin: not much difference,

- Hiruoyuki: material may be the same

Decision: S4-250856 is agreed

Text edited in brackets in online discussion

Development recordings

- [Recordings targeted to facilitate IVAS-basedalgorithm development and potential tuning.] (offline editing)

is not agreed but the rest of the document is agreed, offline editing is invited for this text left in brackets

S4-250935

Presenter: Lasse Laaksonen

Discussion:

- See joint discussion with S4-250853

- Stéphane: can merge this proposal to prepare revision in S4-251027

Decision: S4-250935 is merged

S4-250936

Presenter: Matti Hämäläinen

Discussion:

- Stefan B: talked before about things delivered, is expectation that provider will also provide impulse reponses that would belong to this device? Then signals are raw or compensated by this measurement impulse response?

- Matti: provided impulse respeonses, if combine with raw recordings, get good idea, but open for discussion

- Stefan B: you provide also raw microphone signals, not compensated?

- Matti: not compensated, this is to reporesent hardware of virtual prototype

- Andre: tryinhg to iunderstand, use of probe, a pressure frequency response at output of mic, is probe not to distrub sound field, live with no using probe?

- Matti: holes in commerical device are in same order, same crosssection area at high frequency, this is accurate, order of device

- Andre: other question is substitute for free field frequency response, if leakage and multipath problem, captured? If directional dependency due to leakage of mic is this capture?

- Matti: no directional element, difficult to do, when doing devices A and B, easier to notice leakage, measuring just port

- Stefan B: what it means if we agree this Tdoc? Method mandary but anybody can use other technique in free field?

- Matti: if agree frequency mask when compensated, no alternative ways to do this, up to us to decide whether such requirements are documented

- Stefan B: quite narrow, do not know if anybody can do this

- Huan-Yu: also think important to record perofmrance of output of microphone subsystem but exact method should be left to manufacturer, some not willing to share but serving purpose, so define requirement at output of software, any hardware special chraxcteristics

- Matti: raw mircorphone is more about hardware, if compensate tells less about engineering that happened, structure spec in two ways, one is informative, other normative just require output signals, can accept that, new input, appreciate feedback

- Rishabh: mention feequency response meausrmeenti in anechoic, share in silent room and see how method works

- Matti: see reference 2

- Lasse: this method provides ways to make target devices more approachable for different proponent, commercial devices are out of scope of this WI, there are simulation based approaches, one way to make it possible for more proponent

- Matti: propose as preferred method for measureme

Decision: S4-250936 is parked then noted

S4-250947

Presenter: Arvi Lintervo

Discussion:

- Rishabh: is it correct understanding that for all target devices, need at least two types of recordings?

- Arvi: assuymption is that it woudl help develop example solutions, need hamronization

- Rioshabh on SNR values, looking at recordings (file shared), was filtered out?

- Arvi: A weighted

- Stefan B: contributoin from last meeting, speciried how SNR is calculated, take noise floor during silence, take signal when pplaying sine wave, do not capture, non linearities, harmonics from non linearirites, how wrong is SNR from these effects?

- Arvi: no answer for that

- Stefan B: in principle filter out anyhing not sine wave, then real SNR, no idea of effects from non linearities, other question 46F mic or other one?

- Arvi: only for referecne recordings

- Stefan B: which mic for equalization and measurement of impulse response

- Matti: see publication, Grass microphone, can download 3D printed holder

Decision: S4-250947 is revised to S4-251026

S4-250948

Presenter: Arvi Lintervo

Discussion:

- Huan-Yu: same comment, very unacceptable to use this type of microphone, rather should require output of analog front end or digital processing, if low cost mic and good signal processing, why not aceptalbe

- Arvi: for DaCAS help other proponents if common ageement; signals are within rab

- Huan-Yu: mic cost is increased, not accept

- Markus M: TR or TS? Example solutions work not device requirements

- Rishabh: square brackets so open? Bit depth of ADC? Better snr, 20 or 24 bit ADC?

- Arvi: most porposed have 24 bits in ADC

- Rishabh: recommed

- Stefan B: 94 dB SPL is loud if headroom, 20 dB, overload high, needed?

- 1k sine, raw

- Laqqz: objecting to what? Should, fine for dompensated,  leave should?

- Hy: ih numers relaxed, no pb, if not matched wityh porodctys then pb, proc part of mic

- Stefan d: realiustic freq resp? Fo evs different

- Arvi: 261 for handset

- Stefan b: chzllernging, how to verifu this id needed, further test? Otherwise believe

- Arvi:

- Stefan d: hats in 132 now spatial caputer

- See table 2

- Matti: no assumption on compenent selection

- Andre: just mag or phase ggroup delay also to be considered?

- Arvi: now mag, phse could be ocnsidered

- Andre:1/3 octave?

- Arvi: Not defined

Decision: S4-250948 is parked then noted

S4-250957

Presenter: Arvi Lintervo

Discussion:

- Rishabh: on subjective evaluatiion, DaCAS includes upmix or complete chain (noise suppresion and other blocks)?

- Arvi: think should include everything after target device

- Lasse: at least one IVAS input format for evaluation data, which should be raw or compensated signals, you get IVAS input format, this is open

- Rishabh: requires more discussion, keep it open; on recording scenario, how to define that? Which information provided?

- Arvi: Keep in midn that subjective is important, not just ofcus on objective evaluation, comparison could be done, no proposal yet for discussion

- Rishbah: on objective performance, some values in clause 4.0.2 are in square brackets

- Arvi: in DaCAS context may have some agreement or allow some flexbility

- Rishbah: on acoustic envrioment, nneds to be anechoic? Relax a bit?

- Arvi: as correct as possible but no strong opnion on relaxing room requirements, discuss further

- Stéphane: pospone ?

- Risbah;: not ready

Decision: S4-250957 is noted

S4-250962

Presenter: Markus Multrus

Discussion:

- Arvi: on previously proposed recording scenrio, intention is max is 10 deg, could be higher, question: excluding all elevations, only 3 orientations, consider only for device that produce planar signals?

- Stefan D: check with colleagues, turn table, main issue, 3D dimensional

- Arvi: lacks elevated point, like 40 deg, interpolated?

- Stefan D: can check

- Arvi: can merge similar recording scenario or individual ones?

- Markus M: in general open to merge iwth current scenario 2

- Stefan B: what i gain with higher spatial resolution? More effort to do measurement, understand what is gain?

- Stefan D: feedback from lab expert, just more data, necessary

- Stefan B: like to see in algoirym dev, rather than evaluation; there are limitations in what we do , but face up is a scenario on table, see contributions, also assume put on a table

- Stefan D: in real conferenceing can have table, also display downward., for conference capturing, here idea, free field

- Stéphane: may have more than

- Markus M; is such setup available

- Nien Wu: can all devices record 24 bit resolution, what happens if not 24?

- Markus M: preferred

- Rishabh: on display orientation, cannot use camera mike? If only 3 mics including camera mike, operating with 2 microphones, expected performance?

- Stefan D: even there camera not on same planar level, little 3 dimensionality, not purely planar

- Stéphane: not related to Nokia input

- Markus: more to Nokia method 2

- Arvi: can proponent have freedom to choose spatial resolution

- Markus M: whaty is expectation for target devices? If can loan device, can do measurement ourselves, for others, to guarantee participation from other companies, agree on a certain set of measurements to allow develoent of example solutions

- Arvi: concerned with labour and length of method

Decision: S4-250962 is parked then noted

S4-250963

Presenter: Lasse Laaksonen

Discussion:

- Andre: on complexity analysis, example solutions, complexity analysis needs to be mandatory to make sure this is implemented

- Lasse: Mike had the same comment agreeing, lack consistent method

- Eleni: missing dependencies wiht ATIAS for objectives

- Stefan B: look at time plan, various dependencies on things to agree, can do everythign or discuss on potentially extension

Decision: S4-250963 is parked then noted

S4-250967

Presenter: Matti Hämäläinen

Discussion:

- Stefan D: Similar offers for other device types?

- Wang Bin: database capture, purpose not limited

- Matti: for DaCAS, collect own database

- Stefan B: we can make information avalaible

- Stefan D: if commercial device, would be good to see where to buy, otherwise defined templates and recordings with sufficient details, see other input

- Wang Bin: yes more info needed if no device is loaned

Decision: S4-250967 is noted

S4-251009

Presenter:  Andre Schevciw

Discussion:

- Rishabh: why choose the orignal coordinate system depending on head? Better choose original dependong on getometry of device

- June: form IVAS, required for headset, origin center of head, for recording, user wearing it, more popular

- Mic po why choose system depending on geometry of head?

- June: reconsider , if different head, chaznge coord

- Mike: head and hmd enitre syst, not random avg of head geom data, represet large amunt iof humazn head,

- Wang bin: xr device, now comination, perfect name

- June: database: had to find person , to hzb e acoustic properties

Decision: S4-251009 is agreed

Coordinate system open, name to be improved

Stéphane: offline discussions were invited to check if there would be volunteers to take on the tasks of TS 26.533 Editor and Pdoc Editor(s). Any progress?

Wang Bin: yes there is a proposal discussed offline as follows:

- Editor of TS 26.533: Lasse Laaksonen (Nokia)

- Editor of DaCAS-1 on target devices, recordings, etc.: Nien Wu (Xiaomi)

- Editor of DaCAS-2 on test methods and requirements: Arvi Lintervo (Nokia)

- Editor of DaCAS-3 on deliverables: Mike Ye (ByteDance)

Stéphane: can we agree on this assingment of Editorship? Any objection? Answer: no

S4-251026 is agreed without presentation

S4-251027

Presenter:  Lasse Laaksonen

| E-mail Discussion | Re: [DaCAS] Draft TS 26.533 Ver0.1.0 |
| --- | --- |

Discussion:

- Stéphane: sending the specifiction for information as 1.0?

- Wang Bin: review structure

- Stefan B: microphone compensation to be move in another section

- Lasse: need to clarify role of compensation, there are raw microphone signals, compensated or combination as input to solution

- Andre: prerequisite for DaCAS or device implementation?

- Stefan B: for DaCAS, we have algorithms

- Rishbah: will hvave test methods?

- Stefan B: do not undestand test methods and requirements

- Lasse: for example solutions we need requirements from DaCAS, to see that example solutiuons provide reasonably good qualiry for IVAS encoder inputs, depending on clarification of role of microphone compensation, may have text there, includes requirements for raw microphone signals, open,

- Stefan B: relmated to perfprmance of raw microphone, under prerequisites

- Stefan B: move performance of examples olutions to informative annex

- Lasse: agree

- Markus: in Nokia input, source code is annex or other spec?

- Lasse: not mentioned, electronic attachment

- Update and legal

Decision: S4-251027 is agreed

S4-251057 (DaCAS-1)

Presenter:  Nien Wu

See draft version at

| E-mail Discussion | Revision of draft DaCAS Pdoc-1 Re: Revision of draft DaCAS Pdoc-1 |
| --- | --- |

Discussion:

(Nien Wu presents a draft version)

- Stefan D: offline coordination; source signals shall be provided

Decision: S4-251057 is agreed

S4-251058 (DaCAS-2)

Presenter:  Arvi Lintervo

See draft version at

| E-mail Discussion | Draft S4-251058 PDoc DaCAS-2 |
| --- | --- |

Discussion:

- Wang Bin: fix editorial aspects (source)

- Rishabh: test methods have depency on anechoic chamber? Put TBD for acoustic environment; also discussesd recommended 24 bit recordings

- Arvi: here 16 bits is for source signal, up to target device to provide 24 bits

Decision: S4-251058 is agreed

S4-251059 (DaCAS-3)

Presenter:  Mike Ye

See draft version at

| E-mail Discussion | Revision of DaCAS pdoc on deliverables |
| --- | --- |

Discussion:

- Markus: documents for description of deliverables?

- Stéphane: might expect a list of deliverables similar to EVS or IVAS deliverables Pdoc, how does it align with the Nokia input in 963?

- Lasse: good alignment with Nokia proposal, good to group some timing of deliverables, not based on calendar but milestones

- Mike: added comments to address comments from Nokia, FhG wants to remove all content in red

- Markus M: more confortable if removed

- (Mike removign text in red)

Decision: S4-251059 is agreed

7.7. ATIAS_Ph2 (Terminal Audio quality performance and Test methods for Immersive Audio Services, Phase 2)

Stéphane: 1003 is a late Tdoc, any objection to handle this document? Answer: none

Jan: can even take it later after this meeting if there is no time

S4-250958

Presenter:

Discussion:

- Andre: this text is actual specification text proposed?

- Arvi: just summary of discussions for the Pdoc

- Andre: this depends on the other discussion on IVAS, with multiple noise suppression

- Arvi: not directly depending on those things, we are not agreeing on what we will do in future

- Jan: kick-off to introduce complex sound scene capturing, this was my task as Editor of the Pdoc

- Andre: prefer to wait the other discussion

- Stéphane: then park and revise later

- Arvi: with help from Editor, it is ok

Decision: S4-250958 is parked

S4-250959

Presenter: Arvi Lintervo

Discussion:

- Jan: good input, comment on naming of capture mode type, clear how to name it, could have option to have measurements with classify if two fail, left out intentionally

- Arvi: could be fallback

- Jan: loudness or frequency test may be suitable for running, then see results and classify, care if 1 or 2 apply, then notice different type

- Andre: not clear on definitions of noise vs ambience vs multiple talkers, how do we define what is noise in case of suppression? If testing ambience in SBA with multiple talkers, is this noise?

- Arvi: distinction not made yet, babble noise is considered noise in IVAS, but test method should guide UE performance, may be defined in test scenes

- Jan: asking for clear terms, check if noise reduction is triggered or not, not regarding quality or spatial aspects

- Andre: this term noise, don’t know if reword, backgound or foreground, how to define ambiance, background, may make definition

- Rishabh: when say all supported capture type shall be tested, here noise suppression max and min if only two supported, is it defining a range and testing all steps?

- Arvi: sufficient to test two modes, but not needed, only test one subset of three capture types

- Rishabh: also expect value to be like what in SNR?

- Arvi: I do not know whether information would be available, in predefined scenarios or setups performance should be similar

- Andre: on loudness, why pick SNR or P.700? developed for speech with mouth, sensitivityy of mouth simulator path, here for ambience external to mouth, translates?

- Arvi: one point was to have harmonized analysis for signal and ambience, considered P.700 discussed mapping, fundamentally could be something else than SLR

- Jan: make sense to use rating referencing input to output, if more noise types, different levels, reference could be mono

- Andre: broaband signal, could use LKFS to calculate loudness, calculating in terms LKFS

- Arvi: regarding analysis, good for puposes, not opposing

Decision: S4-250959 is parked

S4-250960

Presenter: Arvi Lintervo

Discussion:

- Jan: already discussed, it would be a good idea, this might trigger multiple loudspeakers, might show signal processing not working correctly, why linear movement? Sounds like x y z, could reword. Would support the change, can add a column in Pdoc and change source signal.

- Nien Wu: how to define movement?

- Jan: applied to test signal, input to reference client, far end that terminal is receiving, could be with virtual positioning, with medadata of ISM, MASA or SBA, would change metadata accordinly. There is no movement of UE, only test signal having this movement.

- Nien Wu: test signal is over speaker?

- Arvi; only test signal

- Andre: testing with multiple talkers?

- Arvi: just for one talker, this could reflect where two talkers and exchange their turn to talk

- Andre: M1 and M4 are not simultaneous

- Stéphane: clear in test method thjat non overlapping talkers?

- Arvi: yes

- Stéphane: agree on this Tdoc?

- Andre: if clear that single talker, may be have multiple talkers in future

- Jan: mean double talk? For send and receive?

- Andre: double talk

- Jan: tricky so excluded, if proposal to realize that, there overlapping talkers

- Andre: also think of multiple talkers in send side, in acoustic environment, variable acoustic path?

- Jan: to all devices with send/receive direction and corresponding acoustic environment

- Andre: here change soure direction but device stays static, acoustic path remains constant, include as well, rotate device

- Jan: think of device type, for HHHF in between, for handset dynamic element in measurement, good idea as triggers lots of issues in signal processing

- Andre: and common use case for device

- Jan: should at least such a test, can add something else on top

- St&phane: agree but capture discussion in Editor’s note

- Andre: be descriptive on naming of the test, there are some limitations

Decision: S4-250960 is agreed

The ATIAS-2 Pdoc Editor is tasked to integrate proposals, with clear naming for clauses and editor’s note to capture the discussions

S4-251003

Presenter: Jan Reimes

Discussion:

- Stéphane: expect update?

- Jan: in case note issue, will report to IVAS work item, looks good so far

Decision: S4-251003 is noted

S4-251004

Presenter:  Andre Schevciw

Discussion:

- Andre: TS 103 224 for bakground noise reproduction, not for ambience

- Alex: original and reconstruction, if reuse same eigenvectors, may reuse original eigenvectors, this would be for a subsequent input

- Jan: how are eigenvalues in dB interpreted? Taking first 4, plus always on top but red cross is not performing that bad for first 3 or 4 items, what does this diference mean? If 2-3 dB difference

- Alex: interpretability remains an open question, if 2-3 dB below bar markers, this does not have any direct interpretation, relative distribution describes how much sound field expands, not exact value but rather distribution and spread are different

- Jan: correlation between original and reproduced one are of interest, they are linearly related? Would think good measure considered during eualization procedure or for reference and information, in response to ETSI STQ?

- Alex: if adapt 103 224 for generalized sound reproduction, could be good idea, how this would be incorporated, this would be a good next step, just spectrum difference, can be EQ, to change diffusiveness of reproduced sound field, useful to pursue this in ETSI

- Andre: potentially with higher order, here used HOA2, even in HOA2 capture this is this much advantage from the 4th order array

- Arvi: few claryfing questions, in condition A 103 224 bur refering 202 396

- Alex: methodology in 103 224

- Arvi: which configuration for B?

- Alex: Fliege point array, see figure displaying online

- Arvi: input was pink noise?

- Alex: outdoor stadium scene in HOA, signal talker

- Arvi: general comment on eigenvalue distribution, not sure how reflects reproduction quality

- Alex: correlation value may be good for a future contribution

- Andre: question is how does it sound? Clear from the figure 2 that there is a difference, clear that 8ch is lower, in DaCAS could compare, and discuss

- Arvi: in general should be considerd if 8ch is enough, rather than taking best ever array

- Andre: should use proper time for setup definition, careful in moving forward

Decision: S4-251004 is noted

S4-251020

Presenter: Jan Reimes

| E-mail Discussion | Draft S4-251020 - PDoc ATIAS-2 v0.4 |
| --- | --- |

Discussion:

- Stéphane: “capture type” may not be stable

- Andre: this suppressed type and capture type, there was also a document from Nokia showing an example of a slider, will there be 3 doiscrete modes or a scale?

- Lasse: related PI data proposed has 16 steps, so more than 3, it’s a question of mapping

- Andre: nominal is default, transparent is no attentuation?

- Arvi: devices can offer two of these modes, with PI data you could have a slider, we are trying to make it simple for testing, if superset is supported nominal is in between

- Rishabh: if name (capture type) is 0 to 16 dB, each point is 1 dB

- Andre: in terms of test plan?

- Stéphane: similar to volume control for testing

- Stéphane: on setup for ambient sound simulation, what file format?

- Jan: could be ambisonics or just current files from 103 224, tests are not tested spatial cues, just testing if sound is suppressed or not

- Andre: on temporal characteristics, put this in brackets, need to understand why this is needed, what we are trying to accomplish, in 3GPP should have tests that are essential

- Stéphane: ok with Pdoc updates until end of ambient noise simlation? Answer:yes

- Jan: also proposal from Bytedance

- Mike: can delete example values

Decision: S4-251020 is agreed

S4-251021

Presenter: Jan Reimes

Discussion:

- Arvi: June 27 does not work,

- Stéphane: can we agree on having a telco on June 23?

- Jan: timing of STQ, dependency on parameter

Decision: S4-251021 is agreed

7.8. FS_ACAPI (Study on Audio Codec APIs)

S4-250868

Presenter: Rishabh Tyagi

Discussion:

- Stefan D.: Agree this is something to work on, what is proposed to be done? On one hand a workaround, but we also see a limitation. Should we communicate this to W3C?

- Rishabh: Yes, we should communicate this to W3C. This falls into the same category of gaps we also earlier identified on the metadata.

- Sujeet: Would we send LS once this report is finalized.

- Stefan D.: Part of the WI is to communicate.

- Stefan B.: We could finalize the WI and then send the TR to W3C.

- Sujeet: I agree, we should have a more clear discussion and possibly solutions

- Rishabh: I can share pCR with the group.

- Stephane: Is multi-channel really part of WebRTC?

- Rishabh: It is part of WebRTC in the browser, but agree for WebCodecs, yes mono and stereo, and default is mono,

- Stephane: Not sure LS is the best way to address the gaps.

- Stefan D.: To really get IVAS supported by WebRTC, we might need to check everything that is missing. In addition, there are many libraries.

- Stephane: We should clarify if this layout is applicable to real voice engine used. Propose to capture this in editor’s note in the TR.

- Rishabh: We can have this editor’s note.

- Sujeet: There are different implementations in different browser, mostly one and maximum two.

| E-mail Discussion | Tyagi, Rishabh on Wed, 21 May 2025 00:54:31 +0000 Tyagi, Rishabh on Wed, 21 May 2025 02:24:44 +0000 |
| --- | --- |

Decision: S4-250868 is revised to S4-251054

S4-251054 is agreed

S4-250918

Presenter: Stefan Döhla

Discussion:

- Stefan D.: Propose even to extend until November, now when seen the progress here.

- Rishabh: On complete to identify potential gaps. Would like to keep it open at least one more meeting.

- Stefan D.: Yes, would leave all points open until the next meeting. Common pre- and post-processing block, so far no inputs. So inputs for these objectives are requested.

Decision: S4-250918 is revised to S4-251047

S4-251047 is agreed

S4-250920

Presenter: Stefan Döhla

Discussion:

- Stefan D.: Integrated 1054 and 927.

Decision: S4-250920 is agreed

S4-250927

Presenter: Stefan Döhla

Discussion:

- Sujeet: Other than WebCodecs and Broswer API,which W3C has specified.

- Stefan D.: We can specify APIs in 3GPP as well.

- Stephane: The calling app is not really in scope, not sure SoC (there could also be software implementation), but not requesting any change to the figure.

- Stefan B.: On Browser, can use WebRTC or codec from OS, but cannot access the hardware codec?

- Stefan D.: Hardware codec is typically under different abstraction level. OS can use hardware codecs. There is an arrow from browser to OS and from OS to SoC.

- Stephane: Ok to agree, and add some extra clarifications.

Decision: S4-250927 is tbd

7.9. FS_ULBC (Study on Ultra Low Bitrate Speech Codec)

S4-250782 is withdrawn

S4-250785 is revised to S4-250979

S4-250786 is revised to S4-250980

S4-250818

Presenter: Huan-yu Su

Discussion:

- Ryan: Can you explain the CNR values?

- Huan-yu: The tables are not proposed for the TR.

- Ryan: Our analysis gives around 600 bps

- Liangping: Item 3, we cannot meet the timeplan with this. Better way is that SA4 defines all parameters and checks with RAN for confirmation. Bring RAN expertise into SA4 otherwise they will not have time.

- Huan-yu: We are open for offline coordination between parties.

- Waqar: Could you check power settings, what is allowed for Rel-19, we have some different understanding.

- Zhe: From real product.

- Thomas: Believe we need to do it here and ask RAN to validate.

- Nicolas: SA4 should have the capability to set the parameters and send them to RAN for confirmation. What was provided in RAN is just an indicative set of capabilities.

- Stefan B.: RAN should confirm

- Stefan D.: I am not sure what RAN will do.

- Stephane: Proposal from Huawei could be put into brackets.

- Stefan D.: We should not set the process, and start the work based on inputs.

Decision: S4-250818 is noted

S4-250819

Presenter: Huan-yu Su

Discussion:

- Eric: Table does not include propagation delay.

- Huan-yu: Right, that delay is significantly higher.

- Markus S.: Right, this comes on top of the propagation delay. Would be good if the simulations include this so we get realistic estimates, processing included in vedor specific delay

- Stephane: On the first point, similar to HARQ for VoLTE simulations. Would it be included in Linagpings model.

- Liangping: Yes, but this can be derived from PHY configuration.

- Stefan B.: How do you ensure BLER?

- Huan-yu: Based on ptime and estimated number of errors. The BLER is determining the retransmissions. This are for good scenarios. In first column repeat 8 times, and worst case would be repeat 32 time, and then you would not support voice call. Running out of realtime.

- Andrei: On the assumption that delay profile is fixed, but do you not have early stopping. You may be able to decode earlier, and then you would not need to wait. From receiver point the delay is not fixed.

- Liangping: Transmission time cannot be longer than frame size.

- Liangping: Which were the assumptions

- Conclusion: Air interface delay, which includes repeated transmission, and the processing delay will be considered in the mouth-to-ear delay estimation, to be confirmed at a later point.

Decision: S4-250819 is noted

S4-250826

Presenter: Stefan Bruhn

Discussion:

- A revision, highlighting changes relevant for GEO scenarios was presented.

- Andre: On disaster, not sure capacity is enough for such scenarios.

- Stefan B.: Disaster is described in TR 22...., and there can also be limitations in other

- Markus S.: Half duplex only on a low level.

- Stefan B.: Different ways to utilize the transmission capabilities.

- Dong: Push-to-talk is different than IMS voice service?

- Stefan B.: Yes, agree, but we can draw different conclusions, but on the

- Amy: This type of service is not included in 3GPP for satellite. This need more careful considerations.

- Amy: Push-to-talk not considered for GEO in SA1, would like to have more time.

- Stefan B.: Would like make sure the half duplex is made clear in the scenario.

- Adrian: On push-to-talk, concerned looking into use cases not discussed in SA1. Other groups are not considering this in their design work.

- Stefan B.: FIne to have push-to-talk in the next meeting. But on the radio layer, half-duplex should be mentioned.

- Thomas: Half-duplex is wrong message here. It is not relevant for the service description. You should contribute to e2e simulation.

- Stefan B.: Discussion on radio parameters, debate on bitrate stated 1-3 would be UL, DL.

| E-mail Discussion | Bruhn, Stefan on Wed, 21 May 2025 23:53:01 +0000 |
| --- | --- |

Decision: S4-250826 is noted

S4-250850

Presenter: Stefan Bruhn

Discussion:

- Markus M.: Surprised how conclusions are drawn, any data? Looking at Nokia test results, ACR seems suitable.

- Stefan B.: Based on experience in testing. What Nokia provided is not in conflict.

- Markus M.: Premature to agree on test methodologies at this point without test results.

- Anssi: What is the sampling frequency of the DCR reference.

- Stefan B.: This would be based insights in relevant technologies, currently assuming SWB.

- Jiayi: Should we consider a single method or several ones and compare.

- Stefan B.: Several categories, for clean speech we are proposing DCR, for other ACR or CCR.

- Jiayi: Would you prioritize intelligibility?

- Stefan B.. Would like to see a codec that is not only working for clean speech.

- Jiayi: Should we focus on speech perforamance or that message scan be understood?

- Stefan B.: You are right, intelligibility is the highest priority.

- Markus S.: Get the message through and identify the speaker, with DCR always a mixture of both things.If integellibility would be prioritized, this should also impact the testing.

- Stefan B.: Are you proposing tests on speaker identification

- Makrus S.: 806, 835, multi-dimensional tests

- Andre: Would not like a small difference in prosodity in giving a large degradation like DCR could give.

- Stephane: Like the discussion, would be good to document this. Choice of methodology could be decided at later point.

- Stefan B.: Can agree to this.

- Anssi: Hallucination is likely with packet loss, should be tested.

- Markus S.: Would like to start with defining what we want to measure.

- Revision

- Markus M.: What is missing is the priorities of the degradations, instead of going to the methodologies.

- Andre: Certain negative aspects for tests, tests are doing what they intend to do.

| E-mail Discussion | Bruhn, Stefan on Wed, 21 May 2025 23:40:52 +0000 |
| --- | --- |

Decision: S4-250850 is revised to S4-251160

S4-251160 is presented directly to SA4 plenary

S4-250857

Presenter: Anssi Rämö

Discussion:

- Erik: Fact that ranking was more or less maintained, not hallicination?

- Anssi: No such effects for these codecs.

- Stephane: Expert listenners?

- Anssi: Yes.

- Stephane: 13.2 kbps WB and SWB almost the same?

- Anssi: Material has not so much high-frequency content.

- Rishabh: More info on the TSAC codec?

- Anssi: Not so much info available.

- Rishabh: Would propose to put into Pdoc.

- Stefan B.: Would be fine to put into brackets into the TR.

- Thomas: What is meant by brackets. Would be good to have editor’s note indicating

- Stephane: Fair to have brackets, but need to digest the text for make it ready for the TR.

- Lasse: Would like to document the test description and test results.

Decision: S4-250857 is agreed. Clause 2, parts on size of models from clause 3, and clause 4 will be included into the TR within brackets.

S4-250858

Presenter: Jiayi Xu

Discussion:

- Stefan B.: Performance requirements and test methodologies should be decoupled.

- Jiayi: Will update this.

- Markus S.. Packet loss concealment, external dependency?

- Stephane: Is this just a template.

- Jiayi:

- Dong: Proposes to delete first editor’s note under clause 6.

- Jiayi: General agreement in group to study application scenarios, whether this impacts design constrains is TBD.

- Stefan B.: What is meant by major?

- Markus M.: Proably relates to dependency analysis.

Decision: S4-250858 is revised to S4-251151

S4-251151 is agreed

S4-251152 (TR 26.940) is left to be presented directly to plenary

S4-251154 (Pdoc) is left to be presented directly to plenary incorporating agreed updates on top of 1151.

S4-251155 is left to be presented directly to plenary, will include telcos which are discussed via email.

S4-251148 (updated SID) is left to be presented directly to plenary

S4-250892

Presenter: Jiayi Xu

Discussion:

- Tomas: My name appears twice with different affiliation, could you please remove the incorrect row.

- Several other errors are found in the list of participants.

- Stephane: Since this is offline, maybe not needed to list names.

- Jiayi: Would like to list companies.

Decision: S4-250892 is revised into S4-251092 which is noted without presentation, removing names from list of participants, but including companies still.

S4-251092 was then revised (directly by the Source, with no indication from the Chair) to S4-251108 to fix a company name. In this report, S4-251108 (not handled by the SWG) is noted without presentation.

S4-250901

Presenter: Amy Zhang

Discussion:

- Stefan B.: For non-GEO proposal might still impact this scenario. One think is the topic of transmission mode, half-duplex should be mentioned. Then the distinction between main and sub-scenario is still kept here. We still think the “sub scenario” is still relevant, e.g. if there are cyber attacs. We would therefore still not like to call it main and sub.

- Amy: Hear other requests to keep it as sub scenario, the propagation delay is much larger.

- Huan-yu: Huawei is unhappy to rule out glasses to be part of these IoT devices.

- Jiayi: Would IoT devices put additional design constraints.

- Huan-yu: It is currently not clear what is meant.

- Andre: On the main scenario discussion, what would be the name for the disaster scenario?

- Stefan B.: We should just be neutral, state it will occur in rare scenarios.

- Amy: Do you mean emergency scenario?

- Stefan B.: Not only, it may be the case where terrestrial network is not available.

- Amy: That scenario is completely different from what we have depicted here. Many things would need to be deployed for this to work and this is currently not the case.

- Amy: Handheld divice is coming from SA1, do not want to exclude other devices but want more evidence that such devices can handle this scenario.

- Zhe: Handheld devices is not really differentiating between smartphones and glasses. Difference is just a few dB in power. Wearables should be the same category.

- Amy: My understanding is that handhold device is smartphone.

- Zhe: SA1 does not distinguish between wearables and smartphone.

- Markus M.: Main reason for statement is to state that we address devices used by everybody.

- Amy: For handheld the power and antenna gain is clear, but for wearables do not think the same data rate can be achieved, so it means . IoT and handheld are two different categories which need different considerations.

- Zhe: There are clear requirements from the business needs.

- Amy: Understand there is a business motivation. Agree wearables may utilize this, but here we need to define which power class is to be addressed here. In 3GPP, 31 dBm is the maximum supported.

- Zhe: If we later find out we can not support such devices, fine to remove.

- Stefan B.: Since we are referring to TS 22.261, regarding the bitrate, half duplex would need to be covered here.

- Stefan D.: Half duplex does not mean half the bitrate.

- Stefan B.: Understood the contribution from Huawei like that.

- Rishabh: If TDM scenario, both uplink and downlink, the bitrate will be half.

- Amy: Bitrates from SA1 is from service perspective.

- Stefan B.: Half duplex has to be reflected here somewhere.

- Amy: Half or full does not matter.

- Liangping: Half duplex does not mean you half the uplink, downlink can still a significant amount of the time. E.g. 80 ms bunding send over 64 ms, might still generate 80 ms audio. Half duplex is not an issue.

- Stefan B.: Still you reserve resources for the other direction. Are bitrates in 22.261 (1-3 kbps), is it the total bitrate or the one I need to distribute.

- Liangping: UL and DL are very different. DL can be several Mbps. 1-3 kbps should be uplink. This is the link between UE and satellite

- Huan-yu: Based on our simulation results, downlink is the bottleneck.

- Andrei: One issue in Huawei 34 dBm which is not allowed in certain regions.

Decision: S4-250901 is revised to S4-251161 with updates made online to the note on devices to be considered, brackets on scenario names, and proposed text from Dolby on [sub] scenario introduction.

S4-251161 is agreed

S4-250902

Presenter: Amy Zhang

Discussion:

- Stephane: We are getting pCRs to real TR, just collect open issues would be a better practise.

- Amy: Then we only consider clause 4.2.4

- Stefan B.: Assumptions in the Pdoc fine, but no assumptions in the TR.

- Markus S.:

- Stefan B.: If we already now need to discuss

Decision: S4-250902 is revised to S4-251162

S4-251162 is left to be presented directly to plenary

S4-250903

Presenter: Dong Wang

Discussion:

- On proposal for delay-error profiles.

- Stefan B.: On the new GEO channel, it seem to be the same physical layer UE1 to eNB1, which is not really correct. If we do simulations we need to consider these aspects. Another thing is if we use transparent method.

- Stefan B.: It would be good with end-to-end, we done so before, but here there are several radio interfaces, and some are not shown.

- Dong: Legacy in right part has some legacy, we need to focus to new channel.

- Stefan B.: My focus is also on GEO channel, are we using transparent or regenerative mode. UE2-> core network might be covered and we can get figures, but we need to zoom in on the GEO channel.

- Dong: Most of deployments for GEO is transparent mode.

- Stefan B: Saw some rather recent paper listing some benefits for regenerative mode.

- Huan-yu: We agree using the simulation method, but what means by controlled environement, guidance from RAN?

- Liangping: SA4 define target packet loss rate, SA4 defines parameters for the simulation.

- Stephane: 26.132 Annex E, profiles used for delay testing, basic simulation model, a seed that controls the simulation. Understanding controlled means the results are reproduceable.

- Erik: 26.132 E, model for LTE access, with new parameters, then ask RAN groups. To get relevant outputs we also need good guesses for these parameters.

- Dong: Yes, we have initial guesses.

- Stephane: If you compare real traces and Annex E, you get very different results. It is a good start, but there are some tweaks to be done to make these realistic. Only for characterization of JBM.

- Andre: If we go outside parameter range, it is not guaranteed to be relevant.

- Liangping:

- Stephane: We need to double check simulation

- Liangping: Verification of channel model 38.811 on channel model, real world model was created.

- Huan-yu: We are asking for correct error-delay profiles. Problem is parameters that controls this. How to derive the parameters?

- Markus M.: Instead of discussing hypotetical details, we should ask for inputs.

- Andrei: Not as easy as picking the range of

Decision: S4-250903 is noted

S4-250904

Presenter: Amy Zhang

Discussion:

- Thierry: Fundemenatal problem, e.g. the bitrate for LEO, some of statements are totally fault.

- Peng: NR NTN is not of relevance

- Adrian: The fundamental part is the scope,

- Stefan B.: On the high level need to discuss documents together. Some statements on the right-hand side in the table are not precise and factual.

- Carlson: We support the document. There is not time to consider other scenarios, and SA1 only requests GEO to be addressed.

Decision: S4-250904 is noted

S4-250905 is withdrawn

S4-250919

Presenter: Liangping Ma

Discussion:

- Stefan B.: Is the parameters for UL or DL, or satellite to ground station? On channel realizations, what kind of elevation, which variabilities could there be. We should have a statement that all worst-case scenarios are covered.

- Liangping: Ground station has huge antenna, the link is very reliable.

- Stefan B.: Then please state this, and then it can be confirmed.

- Stephane: Will you reuse annex E for the right part of the figure, and then extend delay-loss for GEO channel and input to the model. min/max_net_delay is based on a strong assumptions, uniform delay distribution was found incorrect.

- Markus S.: On the tables, why start at 80 ms bunding? Could consider e.g. 40 ms.

- Liangping: 40 ms will not give any feasible data rate.

- Dong: Our input was based on several assumptions.

- Liangping: For 3.75 kHz, the time duration must be a multiple of 32 ms. That leaves not sufficient time for DL and control channel in the 40ms.

- Markus S.: Would be good to state that shorter bunding is not feasible.

- Ryan: Do we need values in 6 kbps range, it exceeds the

- Dong: We would be fine to constraint into reasonable range.

- Liangping: RAN ranges based on SA1 are conservative.

- Stefan D.: Is this the full set of TBS that are possible?

- Liangping: We picked certain values to give some examples.

- Stefan D.: It would be good to add a column with the resulting bitrates.

- Liangping: We cannot decide the bitrates right now.

- Erik: Is NTN-TDL-C fully defined by 38.811? Are bitrates for UL and DL?

- Liangping: Yes. 80 ms is divided for both links.

- Erik: Are the simulation parameters available or be obtained.

- Liangping: If not available, can be obtained.

- Zhe: In SA4 we just need to set the parameters.

- Liangping: Correct.

- Andrei: Could you please provide reference to 3GPP SET 1?

- Stefan B.: On the simulator, are there proprietary implementations? Would provide similar results, but not identical. Would there be some baseline that can be shared?

- Stephane: Would be good to get a script that can handle output from radio simulator.

- Stephane: When do we check if this is realistic?

- Liangping: We would check with RAN groups.

- Stephane: This would be GEO, how do we get end-to-end simulations?

- Jiayi: In vivo proposal, 4 options, would all those be covered?

- Liangping: Impact is on bitrate for the codec, no impact on RAN simulations.

- Revision

- Stefan B.: For which link is this now. Where can I see if it is UL or DL

- Liangping: This is for uplink.

- Stefan B.: Would like to see a note that downlink needs to be covered later.

- Erik: Can there be different bitrates UL DL?

- Liangping: Yes.

- Stephane: max_tx / max_rx, HARQ retransmission, not applicable for GEO.

- Liangping: The feeder link is the difference between regenerative and transparent.

- Huan-yu: We shown that with different parameters very different conclusions can be drawn.

- Liangping: There is no simulation, just assumptions taken from RAN1, these are just candidate TBS, bitrates.

- Huan-yu: We will be mislead from this text.

- Markus S.: Not fully happy with the lowest bitrate of 1.0, there might be some optimizations such that even lower bitrates can be considered.

- Stefan B:: Why is this discussed to be included into the TR. Could be included in [] into the TR.

- Waqar: In simulaitons for TBS, some parameters are used. What are those?

- Stefan D.: Would reserve that additional TBS are considered.

- Stefan B:: Nothing preventing companies from running simulations

- Huan-yu: Do we need 160 ms ptime.

- Zhe: Who would be interested in simulation work?

- Stefan B.: Would expect results before agreement.

Decision: S4-250919 is revised to address comments into S4-251123

S4-251123 is agreed. The proposal is put into the Tdoc with the three tables within brackets.

S4-250923

Presenter: Markus Schnell

Discussion:

- Rishabh: Provided some additional codec for the table, will check offline. On the performance results, our proposal would be to keep results in the Pdoc until test methods are clarified. Add note to conclusions that final conclusions cannot be drawn until design constraints are clear.

- Markus S.: There is nothing now in the conclusion depending on the design constraints. If someone is providing more results, we might have more conclusions.

- Markus M.: We would rather derive design constraints based on this information, not the other way around.

- Stefan B.: Test was done on clean speech, and some of the codecs this would not work on other material.

- Andre: Propose to go one by one

- Stefan B.: Could add, further evidence on the practicality

- Erik: If there is hallucination, could you run this on DCR instead.

- Stephane: Would be good if people do their own estimations of algorithmic delay. Codecs using LSTM may even

- Anssi: Propose to add size of the model to the table.

- Noboru: Have tandem conditions with EVS been studied?

- Markus S. Currently just these results.

- Eric: Do you think WB codecs are suitable to meet the requirements?

- Markus S.: Why not, do not think the audio bandwidth is the main factor.

- Thomas: Could we make an annex to document the existing codecs?

Decision: S4-250923 is agreed with the text put into brackets.

S4-250925

Presenter: Markus Schnell

Discussion:

- Andrei: On the ground station delay, large range, do you have some details.

- Markus S.: Got this from Skylo

- Rishabh: Would be good to keep in [] for now, to get this confirmed. Proposes to have separate heading for existing codecs

- Stephane: On UE delay components, how are these estimated? Air interface delay is typically included MRP to air interface to ERP. Did you take this into account when deriving the numbers.

- Markus S.: 2 ms for air interface was not included, would be

- Stefan B.: Are we having adaptive jitter buffers?

- Stephane: For tests we assume stationary jitter condiftions, and on top we have profiles with extra delay, and this is accounted for in the requirements spec.

- Stefan B.: Should be note that in practice we will see extra component depending on the delay-loss profile.

Decision: S4-250925 is revised S4-251125

S4-251125 is left to be presented directly to plenary

S4-250961

Presenter: Markus Multrus

Discussion:

- Hiroyuki: Possible to change week?

- Markus M.: Would one week later work?

- Proposed to keep the original proposal.

Decision: S4-250961 is agreed

S4-250976

Presenter: Markus Schnell

Discussion:

- Stephane: We need to define how measure/verify the delay.

Decision: S4-250976 is revised to S4-251116 to address comments

S4-251116 is agreed

S4-250979

Presenter: Thomas Stockhammer

Discussion:

- Discussion together with 904

- Jiayi: Update to SID is not necessary, could be seen as scope of the SID.

- Adrian: Four proposal a,b, plus vivo proposal. In our view, SID should be updated, scope and timeline, now note has be used to go outside of the scope. We prefer 1a.

- Stefan B.: Dolby preference would also be option A, but option B could also be an option.

- Huan-yu: Problem with point 2, 0024 document

- Thomas: Forget about the document number, it is the scenario with potential revisions.

- Jiayi: If we go with option A, no other scenarios should be discussed at this meeting.

- Thomas: In the agreed time plan there this is already allowed.

- Jiayi: Do not make updates to SID until we know what to include.

- Adrian: If we agree to update the SID the other scenarios should be on the agenda.

- Amy: If 3-5, the scope is clearly extended and the SID needs to be updated.

- Stefan B.: Do not understand the reasoning. The priorities are unchallenged, we focus on GEO but do still consider other scenarios. If the conclusion is that there are huge efforts needed, we will conclude, let’s not do this.

- Amy: We have the application scenarios, channel characteristics and design constraints. Other scenarios can utilize the codec, but why do we need to add other scenarios here?

- Thierry: Let’s be pragmatic, do not challenge the priority, primary focus is GEO because it is the most challenging scenario. Should spend more time on working than this discussion.

- Adrian: There is nothing else than GEO in the SID.

- Thomas: Misunderstanding, very supportive to have highest priority on GEO, but let’s doing also one step outside. If the codec has other application scenarios, the likelihood to get it sucessful is much higher if other scenarios are considered in design constraints.

- Wang Bin: Main goal to define codec.

- Elmira: Agree with Verizon, it is for GEO. Anything else should be another study.

- Ryan: GEO should be the main scenario, as long as we follow item 6, option A or option B would be fine.

- Thierry: From SA1, they will not understand if we develop a codec that is not applicable also for other scenarios.

- Carlson: Why address VoLTE or NR. We do not update the SID right now, but continue study proposals.

- Adrian: Prefer SID update, otherwise time allocation is not clear.

- Jiayi: We should wait with the update. We do not know if it is one or more codecs.

- Lasse: We should get going, and the way forward is to update the SID.

- Thomas: Concern, are we taking too much work on too short time. What is the reason to not consider other scenarios for design constraints.

- Amy: The timeline is very sensitive. Why not wait until next release. It is not clear what type of codec it would be to handle other scenarios.

- Thomas: Video codecs are developed and can be used for everything. This is also what we do here, because this will cause issues. Let’s move on and do the work.

- Stefan B.: This is a study, if we get indication of a bitrate for the normative phase, do not see any reason not to list bitrates for the scenarios.

- Adrian: Who is objecting to option A?

- Amy: Understand that EVS and IVAS took a long time. Propagation delay is now different. Think it can be applied to different scenarios.

- Thomas: Working agreement, the group agrees to deliver within the timeline for GEO (Rel-20).

- Markus S.: As codec developer, for GEO for sure, but would also like to have the freedom to address other use cases and then these other application scenarios are needed.

- Stephane: Current TR has a scope which is not agreed. It is important that we clarify the scope. If it is found there is a mismatch, we need to update the SID.

- Adrian: Mandate of the group is to work within SID objectives.

- Who would object to option 1a. CMCC, mediaTek

- Who would object  to 1b: vivo, MediateK, verizon,

- Stefan D.: On option #1, if there is no study it is just a matter of lack.

- Amy: Rel-20 is for GEO and further release for NGSO and other use cases.

- Stefan B.: If you are talking about another release, would we not then get another codec. Should we have one codec for GEO and in short time another codec?

- Amy: If codec design is similar, it could be the same, otherwise another.

- Amy: If LEO is included it is very different.

- Peng: Latency and access technology, GEO and LEO would be ok, but NR seems outside of scope.

- Jiayi: Agree to address other use cases but want evidence it can be addressed within one codec.

- Stefan B: On the evidence, you will get it in the end of the study.

- Markus M.: Cannot map note in highlight that we have mandate to create several codec. Do not understand the focus on application scenarios, we could set design contraints that allow broader scope.

- Thierry: Nb IoT is the limiting factor here. Should not have one codec for each use cases.

- Stephane R.: Would the consequence be that documents are not discussed.

- Amy: For objective 2, 3, there would be different simulations needed.

- Amy: Timeplan need to be updated if other scenarios should also impact.

- Peng: Agree let’s start with the technical part, let’s prioritize GEO when studying parameters.

- Revision

- Stephane: How can we commit, we work based on consensus.

- Adrian: Still prefer to update the SID. If we do not have agreement, how are we going to work in the future.

- Stefan B.: Why are we not discussing the scope.

- Thierry: I would also prefer to update the SID, but a pragmatic approach. Understand some parties want to finalize for GEO in the given timeline.

- Stephane: Need to find a pragmatic way. We currently have unagreed scope. We will phase

- Adrian: Will not agree to anything here.

- Amy: Only the application scenario allow for additional scenarios but not the other use cases. We may update the SID if we find that relevant.

- Stefan B.: One company always blocking the progress, saying we do not have time.

- Amy: Other scenarios are not in the objectives, how can you consider other scenarios.

- Stefan B.: We run into significant risk here. Would like to extend the SID to make it transparent.

- Thomas: Document uploaded two days ago. There is no spirit we move jointly.

- Stephane: Spent significant time on the scope issue. Go more formal, try the SID track.

- Lasse: We should not close doors at the very beginning, GEO is focus, but need to study, should update the SID.

- Adrian:

- Erik: Would support updating the SID, other scenarios are of relevance for the codec, although GEO is the priority.

- Jiayi: Fast is not right.

- Thomas: I can make a proposal of update SID.

- Amy: Agree, people have different understanding of the SID.

- Stephane: Might be that we need to just add a note.

Decision: S4-250979 is revised to S4-251149

S4-251149 is left to be presented directly to plenary

S4-250980

Presenter: Thomas Stockhammer

Discussion:

- Ryan: What would be the coordination? Would be reluctant to create an unofficial position.

- Thomas: Whatever moves this topic forward.

- Jiayi: This would facilitate the discussion.

- Ryan: Contribution-driven.

- Jiayi: Coordination work

- Stephane: Would it be good to map to Pdoc on this topic.

- Amy: Better to discuss Pdoc.

- Stefan B.: This is only the second best solution, we would have liked to liaise already with other groups. We need to take the communication very seriously. E.g. on the duplexing modes, this was forgotten or left out.

- Liangping: On half duplex FDD, this is known.

- Stefan B.: There are other things e.g. transparent/regenerative mode.

- Thomas: Agree we need to get agreement within the other working groups. Not sure we can send something at this meeting.

- Huan-yu: We are supportive to create Pdoc, but we need to define the purpose of the Pdoc. Simulator work should be carried out by RAN groups.

Decision: S4-250980 is agreed

S4-250982

Presenter: Stephane Ragot

Discussion:

- Sujeet: If ULBC very low bitrate, more computational complex, what would be use case?

- Stephane: Not the case of interworking. Maybe 1 kbps is not seen as needed, but would be good to understand if we can improve situation for improved robustness. If a codec is introduced in the media stack.

- Amy: Do you want to study codec larger bitrates?

- Stephane: Good questions, that’s why we bring this input. Do you think it is a good idea to limit the codec for 3 kbps. Maybe that would be the decision by the group, but are we already there at this point?

- Amy: We need some evidence to understand potential of market.

- Stephane: Do not see that we can postpone this question much longer.

Decision: S4-250982 is noted

7.10. Other Rel-19 matters including TEI

S4-250930

Presenter: Tomas Toftgård

Discussion:

- Stephane: Not sure we should list that this one affects 253, it is rather the opposite.

- Stefan D.: Not sure the CR number for dependency is correct.

- Tomas: Since Rel-18 CR is endorsed, propose to endorse this one also.

Decision: S4-250930 is endorsed

7.11. Any Other Business

None.

7.12. Close of the session

The chair and vice chair thanked the delegates for their contributions for this busy meeting. The meeting was closed on Thursday May 22 at 13:05 (JST).

Annex A: Agenda with Tdoc Allocation

| 7 | Audio SWG |  |
| --- | --- | --- |
| 7.1 | Opening of the session, registration of documents |  |
| 7.2 | IPR and antitrust reminder |  |
| 7.3 | Reports/Liaisons from other groups/meetings | 773n (ETSI TC STQ) |
| 7.4 | Release 18 and earlier matters | 928a (26.253, main part) A.I 13 954->1135e (26.253, Annex A) 924->1025n (26.253, subformats) 956->1037n (26.253, capture type, onset, diegetic) 933->1133n (26.253, terminal status) 929e (26.114) |
| 7.5 | IVAS_Codec_Ph2 (EVS Codec Extension for Immersive Voice and Audio Services, Phase 2) | 778n (26.253, header extensions) 916n (26.253, header extensions) 1005a (IVAS PI data representations) 934n (26.253, noise reduction, audio focus) 761->1001->1138a (IVAS-8b) A.I 14.1 784->1043a (ACR) 874a (Room acoustics test) 914->1013->1069a (Verification BASOP encoder) A.I 14.1 915->1012->1072a (IVAS-10) A.I 14.1 926->1011n (Proposed corrections BASOP) 862->1100a (IVAS-7b) A.I 14.1 854a (floating-point conformance) 1073a IVAS-1 (overview) A.I 14.1 1129a IVAS-11 (“IVAS non-BE conformance”) A.I 14.1 1074a IVAS-2b (TP) A.I 14.1 |
| 7.6 | DaCAS (Diverse audio CApturing System for UEs) | TS 26.533 853r->1027a (Xiaomi, v0.0.1->Editor v0.1.0) A.I. 14.3 935m (Nokia, DaCAS target device type descr.) Pdocs / templates 851a (Xiaomi, Pdoc proposal) 856a (Xiaomi, templates 2.1) Microphone signal requirements 936n (Nokia, IMPro method) 948n (Nokia, requirements) Target devices 822r->1009a (Bytedance, XR HMD) 967n (Nokia, prototype loan) Recording databases 821a (Bytedance, smartphone-like device) 823nn (Xiaomi, smartphone device) 947->1026a (Nokia, devices A and B) 962n (FhG, acoustic properties measurements) Misc. 852n (Xiaomi, license) 957n (Nokia, performance evaluation) 963n (Nokia, deliverables) 820, 824 1039a (Rapporteur, Time plan) A.I. 14.3 1057a (Editor : Ninghang Wu, Xiaomi, Pdoc DaCAS-1 Target devices&recording scenarios/databases, incl. templates) A.I. 14.3 1058a (Editor: Arvi Lintervo, Nokia, Pdoc DaCAS-2 Test methodologies and performance requirements) A.I. 14.3 1059a (Editor: Mike Ye, Bytedance, Pdoc DaCAS-3 Deliverables) A.I. 14.3 |
| 7.7 | ATIAS_Ph2 (Terminal Audio quality performance and Test methods for Immersive Audio Services, Phase 2) | 958n (Nokia, updates to Pdoc) 960a (Nokia, echo testing) 959n (Nokia, ambience transmission testing) 1004n (Qualcomm, diffuseness reproduction) 1003n (HEAD acoustics, nominal transmission levels) 1020a Pdoc update (v0.4) A.I. 14.2 1021a Time plan (v0.4) A.I. 14.2 |
| 7.8 | FS_ACAPI (Study on Audio Codec APIs) | 868->1054a (WebRTC gaps MC, SBA) 927a (common APIs) 918->1047a (TP) A.I 15.1 920a (TR) A.I 15.1 |
| 7.9 | FS_ULBC (Study on Ultra Low Bitrate Speech Codec) | 892->1092n (summary offline) 785->979->1149 (process application scenarios) A.I 15.7 1148 SID update A.I 15.7 904n (scope application scenarios) 786->980a (e2e service performance evaluation) 818n (channel characteristics evaluation) 903n (progress ULBC) 919->1123a (e2e channel characteristics) 901->1161a (GEO, TR) 902->1162 (GEO, Pdoc) A.I 15.7 819n (mouth-to-ear delay) 925->1125 (mouth-to-ear delay) A.I 15.7 923a (existing technologies) 857a (listening test results) 850->1160 (test methodologies) A.I 15.7 976->1116a (template design constraints) 961a (ad-hoc meeting) 826n (LEO/MEO) 982n (other than GEO) 858->1151a (Pdoc) A.I 15.7 1152 TR A.I 15.7 1154 Pdoc A.I 15.7 (to integrate further agreed text) 1155 TP - discuss telco dates over email A.I 15.7 782, 905 |
| 7.10 | Other Rel-19 matters including TEI | 930e (26.114) |
| 7.11 | Any Other Business |  |
| 7.12 | Close of the session |  |

n – noted

a – agreed

p – parked

pp – postponed

r – revised

rp – replied

m – merged

Annex B: List of Participants

| Family Name | Given Name | Organization Represented | PRESENCE |
| --- | --- | --- | --- |
| Brekalo | Andrijana | ETSI | Present |
| Buckley | Adrian | Verizon Spain | Present |
| Gabin | Frederic | Dolby France SAS | Present |
| Jansson Toftgård | Tomas | Ericsson LM | Present |
| Jelinek | Milan | VoiceAge Corporation | Present |
| Laaksonen | Lasse | Nokia Canada | Present |
| Lehtiniemi | Arto | Nokia Hungary | Present |
| Li | Lei | Huawei Tech. Japan, K.K. | Present |
| Mate | Sujeet | Nokia Solutions & Networks (I) | Present |
| Multrus | Markus | Fraunhofer IIS | Present |
| Norvell | Erik | L.M. Ericsson Limited | Present |
| Pajunen | Lauros | Nokia Korea | Present |
| Pihlajakuja | Tapani | Nokia Poland | Present |
| Rämö | Anssi | Nokia Americas | Present |
| Reimes | Jan | HEAD acoustics GmbH | Present |
| Schnell | Markus | Fraunhofer IIS | Present |
| Szczerba | Marek | Philips International B.V. | Present |
| chelibane | ouerdia | Orange Romania | Absent |
| Ding | Shilin | Qualcomm Incorporated | Absent |
| Dong | Wang | vivo Wisdom Technology | Absent |
| Ehara | Hiroyuki | Panasonic Holdings Corporation | Absent |
| Goel | Gaurav | Reliance Jio | Absent |
| Hämäläinen | Matti | Nokia Mexico | Absent |
| Han | Lufeng | Spreadtrum | Absent |
| Lee | Hakju Ryan | Samsung R&D Institute UK | Absent |
| Li | Jiali | ZTE Corporation | Absent |
| Liangping | Ma | Qualcomm Innovation Center Inc | Absent |
| Lintervo | Arvi | Nokia Japan | Absent |
| Liu | yt | BUPT | Absent |
| Liu | Zehan | BJTU | Absent |
| Ragot | Stephane | Orange UK | Absent |
| Rai | Reshma | Ittiam System Pvt Ltd | Absent |
| Schevciw | Andre | Qualcomm Incorporated | Absent |
| Stegenborg-Andersen | Tore | FORCE Technology | Absent |
| Stockhammer | Thomas | Qualcomm Germany | Absent |
| Stoica | Razvan-Andrei | Lenovo Future Communications | Absent |
| Sun | June | Bytedance | Absent |
| Teniou | Gilles | Tencent | Absent |
| Tyagi | Rishabh | Dolby Laboratories Inc. | Absent |
| Venkatesh | Sandesh | Ittiam System Pvt Ltd | Absent |
| Villette | Stephane | Qualcomm Technologies | Absent |
| Wang | Bin | Xiaomi Technology Italy srl | Present |
| Wu | Nien | Xiaomi Technology Spain S.L | Present |
| Xu | Jiayi | China Mobile Com. Corporation | Absent |
| Yang | Ruonan | vivo Mobile Communication Co., | Absent |
| Zhang | Amy | vivo Wisdom Technology | Absent |
| Zia | Waqar | Apple Switzerland AG | Absent |

Annex C: Document Status

C.1 Agreed documents (to be presented to SA4 plenary)

| TDoc | Title | Source | Agenda item | TDoc Status |
| --- | --- | --- | --- | --- |
| S4-250920 | [FS_ACAPI] draft TR 26.858 v0.5.0 | Fraunhofer IIS (Rapporteur) | 7.8 | agreed |
| S4-250928 | Corrections to the IVAS algorithmic description | Dolby Laboratories Inc., Ericsson LM, Fraunhofer IIS, Huawei Technologies Co Ltd., Nokia, NTT, Orange, Panasonic Holdings Corporation, Philips International B.V., Qualcomm Incorporated, VoiceAge Corporation | 7.4 | agreed |
| S4-251020 | Permanent Document ATIAS-2 v0.4 | Rapporteur ATIAS_Ph2 | 7.7 | agreed |
| S4-251021 | Time and Work Plan for ATIAS_Ph2 v0.4 | HEAD acoustics GmbH | 7.7 | agreed |
| S4-251027 | [DaCAS]TS 26.533-Ver0.1.0 | Xiaomi Technology Italy srl | 7.6 | agreed |
| S4-251039 | [DaCAS]Work Plan for DaCAS v0.4 | Beijing Xiaomi Mobile Software Co., Ltd | 7.6 | agreed |
| S4-251047 | [FS_ACAPI] Updated Time Plan | Fraunhofer IIS (Rapporteur) | 7.8 | agreed |
| S4-251057 | DaCAS-1: Target devices and databases v0.1 | Xiaomi Technology Spain S.L | 7.6 | agreed |
| S4-251058 | DaCAS-2: Test methodologies and requirements v0.1 | Nokia | 7.6 | agreed |
| S4-251059 | DaCAS-3 on deliverables | Bytedance | 7.6 | agreed |
| S4-251069 | Verification of IVAS BASOP Encoder | Dolby Sweden AB, Ericsson LM, Fraunhofer IIS, NTT, Nokia, Orange, Philips International B.V., VoiceAge Corporation | 7.5 | agreed |
| S4-251072 | Update of IVAS-10 | Ericsson LM | 7.5 | agreed |
| S4-251073 | IVAS codec development overview (IVAS-1) | Huawei Technologies France | 7.5 | agreed |
| S4-251074 | IVAS Permanent document IVAS-2b: IVAS_Codec_Ph2 Project Plan, v1.3.0 | Huawei Technologies Co., Ltd | 7.5 | agreed |
| S4-251100 | IVAS-7b: Processing plan for characterization phase, v.0.6.0 | Editor (Ericsson LM) | 7.5 | agreed |
| S4-251129 | Permanent document IVAS Non-BE Conformance (IVAS-11) | Dolby Laboratories Inc. | 7.5 | agreed |
| S4-251138 | IVAS Permanent Document IVAS-8b: Test Plan for Characterization Phase, Version v.0.7.0 | VoiceAge Corporation | 7.5 | agreed |
| S4-251151 | [FS_ULBC]Permanent Document FS_ULBC v0.1.0 | China Mobile Com. Corporation | 7.9 | agreed |

C.2 Agreed documents (not to be presented to SA4 plenary)

| TDoc | Title | Source | Agenda item | TDoc Status |
| --- | --- | --- | --- | --- |
| S4-250821 | [DaCAS] Recordings of smartphone-like target device | Bytedance | 7.6 | agreed |
| S4-250851 | Permanent documents proposal for DaCAS | Xiaomi Technology Italy srl | 7.6 | agreed |
| S4-250854 | IVAS floating point non-be conformance | Dolby Laboratories Inc. | 7.5 | agreed |
| S4-250856 | Update DaCAS templates to v2.1 | Xiaomi Technology Spain S.L | 7.6 | agreed |
| S4-250857 | Very low bitrate listening test results | Nokia | 7.9 | agreed |
| S4-250874 | IVAS Characterization - Room Acoustics Test Plan | Philips International B.V., Qualcomm Incorporated | 7.5 | agreed |
| S4-250923 | Analysis of existing technologies for ULBC | Fraunhofer IIS, vivo, Qualcomm Incorporated, China Mobile, Skylo | 7.9 | agreed |
| S4-250927 | [FS_ACAPI] Common APIs | Fraunhofer IIS | 7.8 | agreed |
| S4-250960 | On applied test signal for AEC assessment | Nokia | 7.7 | agreed |
| S4-250961 | FS_ULBC: Proposed Ad Hoc Meeting | Fraunhofer IIS | 7.9 | agreed |
| S4-250980 | [FS_ULBC] On Bitrates and Delay-Loss Information for Ultra Low Bitrate Speech Codec over GEO | Qualcomm Incorporated, vivo, Nokia, China Mobile, Fraunhofer II, Dolby Laboratories, Inc., Viasat, Novamint, Thales, Orange | 7.9 | agreed |
| S4-251005 | IVAS PI data representations | Ericsson LM | 7.5 | agreed |
| S4-251009 | Propose a XR HMD target device | Bytedance | 7.6 | agreed |
| S4-251026 | Database for Four-microphone prototype devices A and B | Nokia | 7.6 | agreed |
| S4-251043 | ACR characterization of IVAS codec | Nokia | 7.5 | agreed |
| S4-251054 | [FS_ACAPI] Multichannel Audio and Scene Based Audio Format Support in WebRTC | Dolby Laboratories Inc. | 7.8 | agreed |
| S4-251116 | Template for ULBC design constraints | Fraunhofer IIS | 7.9 | agreed |
| S4-251123 | End to end simulation and channel characteristics | vivo, Qualcomm | 7.9 | agreed |
| S4-251161 | Application scenario for IMS voice call using GEO satellite | vivo, Fraunhofer IIS, Qualcomm Incorporated, China Mobile Com. Corporation, [Huawei Technologies Co., Ltd.], Viasat, Inmarsat, EchoStar,[Orange], Spreadtrum, Nokia | 7.9 | agreed |

C.3 Other status than agreed documents (not to be presented to SA4 plenary)

| TDoc | Title | Source | Agenda item | TDoc Status |
| --- | --- | --- | --- | --- |
| S4-250761 | IVAS Permanent Document IVAS-8b: Test Plan for Characterization Phase, Version v.0.6.1 | VoiceAge Corporation | 7.5 | revised |
| S4-250773 | Reply LS on extending ETSI TS 103 224 for testing immersive UEs | ETSI TC STQ | 7.3 | noted |
| S4-250778 | RTP Header Extension for Processing Information for IVAS | Qualcomm Innovation Center Inc | 7.5 | noted |
| S4-250784 | ACR characterization of IVAS codec | Nokia | 7.5 | revised |
| S4-250785 | [FS_ULBC] On Application Scenarios | Qualcomm Incorporated, Dolby Laboratories, Viasat, Novamint, Thales, Nokia, Fraunhofer IIS | 7.9 | revised |
| S4-250786 | [FS_ULBC] On Bitrates and Delay-Loss Information for Ultra Low Bitrate Speech Codec over GEO | Qualcomm Incorporated, vivo, Nokia, China Mobile, Fraunhofer II, Dolby Laboratories, Inc., Viasat, Novamint, Thales | 7.9 | revised |
| S4-250818 | [FS_ULBC] On channel characteristics evaluation for ULBC | Huawei Technologies France | 7.9 | noted |
| S4-250819 | [FS_ULBC] On mouth-to-ear delay estimation for ULBC | Huawei Technologies France | 7.9 | noted |
| S4-250822 | Propose a XR HMD target device | Bytedance | 7.6 | revised |
| S4-250826 | Application scenario for IMS voice call using LEO and MEO satellites | Dolby Laboratories Inc., Novamint, Sateliot, EDF, Iridium, EchoStar | 7.9 | noted |
| S4-250850 | [FS_ULBC] On test methodologies for ULBC performance evaluation | Dolby Laboratories Inc., Novamint | 7.9 | revised |
| S4-250852 | License text updates for DaCAS | Xiaomi Technology Italy srl | 7.6 | noted |
| S4-250853 | [DaCAS]Proposal of initial version of TS 26.533 | Xiaomi Technology Italy srl | 7.6 | revised |
| S4-250858 | [FS_ULBC]Permanent Document FS_ULBC v0.0.1 | China Mobile Com. Corporation | 7.9 | revised |
| S4-250862 | IVAS-7b: Processing plan for characterization phase, v.0.5.1 | Editor (Ericsson LM) | 7.5 | revised |
| S4-250868 | [FS_ACAPI] Multichannel Audio and Scene Based Audio Format Support in WebRTC | Dolby Laboratories Inc. | 7.8 | revised |
| S4-250892 | [FS_ULBC] Summary of Offline Discussion | China Mobile Com. Corporation | 7.9 | revised |
| S4-250901 | Application scenario for IMS voice call using GEO satellite | vivo, Fraunhofer IIS, Qualcomm Incorporated, China Mobile Com. Corporation, [Huawei Technologies Co., Ltd.], Viasat, Inmarsat, EchoStar,[Orange], Spreadtrum, Nokia | 7.9 | revised |
| S4-250902 | Application scenario for IMS voice call using GEO satellite | vivo, Fraunhofer IIS, Qualcomm Incorporated, China Mobile Com. Corporation, [Huawei Technologies Co., Ltd.], Viasat, Inmarsat, EchoStar,[Orange], Spreadtrum, Nokia | 7.9 | revised |
| S4-250903 | Considerations on how to progress ULBC | vivo | 7.9 | noted |
| S4-250904 | Discussion on scope extension on New Application Scenarios | vivo, Bytedance | 7.9 | noted |
| S4-250914 | Verification of IVAS BASOP Encoder | Dolby Sweden AB, Ericsson LM, Fraunhofer IIS, NTT, Nokia, Orange, Philips International B.V., VoiceAge Corporation | 7.5 | revised |
| S4-250915 | Update of IVAS-10 | L.M. Ericsson Limited | 7.5 | revised |
| S4-250916 | RTP Header Extensions and other improvements for IVAS Transport | Fraunhofer IIS | 7.5 | noted |
| S4-250918 | [FS_ACAPI] Updated Time Plan | Fraunhofer IIS (Rapporteur) | 7.8 | revised |
| S4-250919 | End to end simulation and channel characteristics | vivo, Qualcomm | 7.9 | revised |
| S4-250924 | Support of subformats in IVAS RTP payload format | Orange, Nokia | 7.4 | revised |
| S4-250925 | Estimation mouth to ear delay for GEO NTN voice | Fraunhofer IIS, vivo, Qualcomm Incorporated, Skylo | 7.9 | revised |
| S4-250926 | Proposed corrections to the IVAS BASOP Codec | L.M. Ericsson Limited | 7.5 | revised |
| S4-250929 | Update to IVAS SDP parameters | Ericsson LM | 7.4 | endorsed |
| S4-250930 | Update to IVAS SDP parameters | Ericsson LM | 7.10 | endorsed |
| S4-250933 | Signaling Terminal Status | Panasonic Holdings Corporation | 7.4 | revised |
| S4-250934 | Enabling noise reduction and audio focus in IVAS sessions | Nokia | 7.5 | noted |
| S4-250935 | Initial description of DaCAS target device type for draft TS 26.533 | Nokia | 7.6 | merged |
| S4-250936 | On compensated microphone pressure frequency response measurement | Nokia | 7.6 | noted |
| S4-250947 | Database for Four-microphone prototype devices A and B | Nokia | 7.6 | revised |
| S4-250948 | On microphone signal requirements | Nokia | 7.6 | noted |
| S4-250954 | Further corrections to Annex A | Nokia, Fraunhofer IIS, Ericsson LM, Dolby Laboratories Inc. | 7.4 | revised |
| S4-250956 | Capture type parameter, audio onset indication, and diegetic term for IVAS | Nokia | 7.4 | revised |
| S4-250957 | On performance evaluation | Nokia | 7.6 | noted |
| S4-250958 | Updates to ATIAS-2 permanent document | Nokia | 7.7 | noted |
| S4-250959 | Proposal on ambience transmission testing | Nokia | 7.7 | noted |
| S4-250962 | Proposal on DaCAS Acoustic Properties Measurements | Fraunhofer IIS | 7.6 | noted |
| S4-250963 | Proposal for DaCAS deliverables | Nokia | 7.6 | noted |
| S4-250967 | On availability of Nokia device prototypes for short term loan | Nokia | 7.6 | noted |
| S4-250976 | Template for ULBC design constraints | Fraunhofer IIS | 7.9 | revised |
| S4-250979 | [FS_ULBC] On Application Scenarios | Qualcomm Incorporated, Dolby Laboratories, Viasat, Novamint, Thales, Nokia, Fraunhofer IIS, Orange | 7.9 | revised |
| S4-250982 | On FS_ULBC application scenarios | Orange | 7.9 | noted |
| S4-251001 | IVAS Permanent Document IVAS-8b: Test Plan for Characterization Phase, Version v.0.6.2 | VoiceAge Corporation | 7.5 | revised |
| S4-251003 | Analysis of nominal send loudness rating | HEAD acoustics GmbH | 7.7 | noted |
| S4-251004 | Assessing Diffuseness Reproduction of Loudspeaker Layouts for ATIAS | Qualcomm Incorporated | 7.7 | noted |
| S4-251011 | Proposed corrections to the IVAS BASOP Codec | Ericsson LM | 7.5 | noted |
| S4-251012 | Update of IVAS-10 | Ericsson LM | 7.5 | revised |
| S4-251013 | Verification of IVAS BASOP Encoder | Dolby Sweden AB, Ericsson LM, Fraunhofer IIS, NTT, Nokia, Orange, Philips International B.V., VoiceAge Corporation | 7.5 | revised |
| S4-251025 | Support of subformats in IVAS RTP payload format | Orange, Nokia | 7.4 | noted |
| S4-251037 | Capture type parameter, audio onset indication, and diegetic term for IVAS | Nokia | 7.4 | noted |
| S4-251092 | [FS_ULBC] Summary of Offline Discussion | China Mobile Com. Corporation | 7.9 | noted |
| S4-251133 | Signaling Terminal Status | Panasonic Holdings Corporation, Nokia | 7.4 | noted |
| S4-251135 | Further corrections to Annex A | Nokia, Fraunhofer IIS, Ericsson LM, Dolby Laboratories Inc., Orange | 7.4 | endorsed |

C.4 Other status than agreed documents (to be presented to SA4 plenary)

None.

Tomas Toftgård, tomas.toftgard@ericsson.com; Stéphane Ragot, stephane.ragot@orange.com