|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2025-2028 |  | SG2-TD262R2/PLEN |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 2 |  |  |
|  |  |  | Original: English |  |  |
| Question(s): |  | 5/2 |  | Virtual, 25-28 August 2025 Geneva, 1-5 September 2025 |  |
| TD |  |  |  |  |  |
| Source: |  | Editor |  |  |  |
| Title: |  | Revised baseline text for M.3393(M.rsmca): "Requirements for smart maintenance of cell antenna" (for consent) |  |  |  |
| Contact: |  | Zhiheng Gao China Telecom China |  |  | Tel: +8613316090139 E-mail: gaozhh@chinatelecom.cn |
| Contact: |  | Ping Zhao China Telecom China |  |  | Tel: +86 20 38639086 E-mail: zhaop11@chinatelecom.cn |
| Contact: |  | Hanqui Tan China Telecom China |  |  | Tel: +8618142821020 E-mail: tanhq@chinatelecom.cn |
| Contact: |  | Qiusen Wang China Telecom China |  |  | Tel: +8618024084388 E-mail: wangqs1@chinatelecom.cn |

| Abstract: | This updated Recommendation "Requirements for smart maintenance of cell antenna" based on TD134/PLEN, Contribution C128 and the editing instructions received during the SG2 meeting held on 25 - 28 August 2025. It is proposed for consent. |
| --- | --- |

Summary

This new version of draft Recommendation M.rsmca "Requirements for smart maintenance of cell antenna", based upon TD134/P, Contribution C128 and the editing instructions received during the SG2 meeting held on 25 - 28 August 2025. This proposal for the revised text is agreed.

The main changes based on Contribution C128 and discussion on 25-28 August are as follows:

- Removed some unnecessary term definitions; added some acronyms and resorted all entries.

- Added a general description in Clause 7.1, and removed the original Clause 7.2.

- Adjusted the Technical Framework Figure and revised the related functional requirements.

- In the Annex A, added explanations for certain variables in some formula; renumbered A.2 as Annex B; streamlined the descriptions of A.2.3 and A.2.4; changed "Annex" to "Appendix".

- Supplemented the preconditions and postconditions in the use case.

- Standardized the content and format of the Bibliography.

- Make some editorial revisions.

Draft Recommendation ITU-T M.3393(M.rsmca)

Requirements for smart maintenance of cell antenna

Summary

This Recommendation describes the requirements for smart maintenance of cell antenna. The requirements include framework, functional requirements and related interfaces, which are addressed to use AI technology to process the big data from network side and calculate the predicted antenna position and azimuth angle, and then provide correction proposals.

Keywords

smart maintenance, cell antenna

Table of Contents

1. Scope	4

2. References	4

3. Definitions	4

3.1 Terms defined elsewhere	4

3.2 Terms defined in this Recommendation	5

4. Acronyms and abbreviation	5

5. Conventions	5

6. Overview of cell antenna smart maintenance	5

7. Framework for smart maintenance of cell antenna	6

8. Functional requirements for smart maintenance of cell antenna	7

8.1 Functional requirements for data source layer	7

8.2 Functional requirements for data pre-processing layer	8

8.3 Functional requirements for model management layer	8

8.4 Functional requirements for service provision layer	9

9. Requirements for related interfaces	9

9.1 E1 interface	9

9.2 E2 interface	9

9.3 E3 interface	9

9.4 E4 interface	9

Appendix I	10

I.1 Use case template	10

I.2 Typical technical use cases of smart maintenance of cell antenna	10

I.2.1 Prediction of antenna position based on minimization of drive test (MDT)	10

I.2.2 Prediction of antenna position based on trajectory filtering	11

I.2.3 Prediction of antenna position based on neighboring cells	13

I.2.4 Prediction of antenna azimuth angle	14

Appendix II	18

Bibliography	19

Draft Recommendation ITU-T M.3393(M.rsmca)

Requirements for smart maintenance of cell antenna

- Scope

This Recommendation specifies the requirements for smart maintenance of cell antenna, including the framework, functional requirements and related interfaces, which are addressed to use artificial intelligence (AI) technology to process big data from network side and calculate the predicted antenna position and azimuth angle, and then provide correction proposals.

- References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T M.3080]   Recommendation ITU-T M.3080 (2021), Framework of artificial intelligence

enhanced telecom operation and management (AITOM).

[ITU-T M.3381]   Recommendation ITU-T M.3381 (2022), Requirements for energy saving

management of 5G radio access network (RAN) systems with artificial intelligence (AI).

- Definitions

## 3.1 Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	antenna [b-ITU-T K.70]: Device that serves as a transducer between a guided wave (e.g., coaxial cable) and a free space wave, or vice versa. It can be used to emit or receive a radio signal. .

3.1.2	antenna downtilt angle [b-ITU-R M.1797]: The angle between the direction of the main-lobe and the horizontal plane. NOTE 1 - It is a positive number for downtilt and a negative number for uptilt.

3.1.3	base station (BS) [b-ITU-T Q.1741.3]: A macrocell, microcell or picocell site and consists of transmitters generating radio frequency electromagnetic energy and receivers in a cabin or cabinet. A base station is connected to antennas by feeder cables.

3.1.4	cell site [b-ITU-T Q.1742.11]: The physical location of a cell's radio equipment and supporting systems. This term is also used to refer to the equipment located at the cell site.

3.1.5	degrees of latitude [b-ITU-T Q.762]: Information that indicates the geodetic location of the calling party in terms of degrees of latitude north or south of the equator.

3.1.6	degrees of longitude [b-ITU-T Q.762]: Information that indicates the geodetic location of the calling party in terms of degrees of longitude east or west of the Greenwich Meridian.

3.1.7	WGS-84 [b-ITU-T L.94]: The World Geodetic System - 1984 (WGS-84) coordinate system is a conventional terrestrial system (CTS), realized by modifying the Navy navigation satellite system (NNSS), Doppler Reference Frame (NSWC 9Z-2) in origin and scale, and rotating it to bring its reference meridian into coincidence with the International Time Bureau - defined zero meridian.

## 3.2 Terms defined in this Recommendation

3.2.1	antenna azimuth angle: The angle experienced when the vertical plane in true north direction rotates clockwise to coincide with the vertical plane where the antenna is located.

3.2.2	antenna position: The installation position of the base station antenna, which is usually represented using World Geodetic System 1984(WGS-84) latitude and longitude coordinates.

- Acronyms and abbreviation

AI		Artificial Intelligence

CTS		Conventional Terrestrial System

eNodeB		Evolved Node B

GNSS		Global Navigation Satellite System

GNSS-MR	Global Navigation Satellite System-Measurement Report

IMSI		International Mobile Subscriber Identity

MDT		Minimization of Drive Test

MR		Measurement Report

NMS		Network Management System

NNSS		Navy Navigation Satellite System

NSWC		Naval Surface Warfare Center

OAM		Operation, Administration and Maintenance

RSRP		Reference Signal Received Power

TA		Timing Advance

S1-MME	S1 Interface of Mobility Management Entity

WGS-84	World Geodetic System 1984

- Conventions

In this Recommendation:

The keywords "is required to" indicate a requirement which must be strictly followed and from which no deviation is permitted, if conformance to this Recommendation is to be claimed.

The keywords "is recommended to" indicate a requirement which is recommended but which is not absolutely required. Thus, this requirement need not be present to claim conformance.

- Overview of cell antenna smart maintenance

The base station registered account data includes location(longitude and latitude), azimuth, downtilt angle, power and other core working parameters. It is the core resource of the base stations, basic analysis data for network optimization, planning and construction, and the essential data for the application of user positioning algorithm. Therefore, it is crucial to ensure the accuracy of the base station registered account data. However, due to the adjustment and optimization of network equipment by engineering construction, network optimization, network planning and other scenarios, there may be inconsistencies between the registered account data and the actual status of current network equipment. According to some statistics, the proportion of antenna position deviation greater than 500 meters in some province is up to 10%. These imprecise account data will seriously affect the effectiveness of daily network optimization.

The traditional methods require on-site patrol to check the accurate account data, which requires a lot of manpower and resources and is inefficient. This Recommendation describes the requirements of using big data technology to predict the antenna position and azimuth angle.

As shown in Figure 1, the cell antenna smart maintenance system needs to interact with supporting systems such as resource system, work order system, network maintenance system and Internet map system.

Figure 1 Interfaces between cell antenna smart maintenance system and supporting systems

- Framework for smart maintenance of cell antenna

The smart maintenance of cell antennas is designed to significantly reduce the workload of on-site patrol by incorporating big data technologies and machine learning algorithms. It consists of several functional layers working together to enable automated prediction and intelligent decision-making regarding antenna parameters such as position and azimuth angle.

As shown in Figure 2, the smart maintenance of cell antenna is required to include the following function layers:

1) Data Source Layer: This layer is required to be responsible for collecting the required network signaling data such as measurement report (MR), S1 interface of mobility management entity (S1-MME) data and basic cell information such as cell ID, name, registered position, registered azimuth angle.

2) Data Pre-Processing Layer: This layer is required to be responsible for pre-processing the collected data before modeling, such as extraction of the required data, transformation by the need of data cleaning and data loading.

3) Model Management Layer: This layer is required to be responsible for training models, executing models and optimizing models.

4)Service Provision Layer: This layer is required to be responsible for providing data prediction service, such as sending out the prediction results and receiving and the status feedback.

Figure 2 Technical framework of cell antenna smart maintenance

Calculate the predicted antenna position

Get the registered antenna position and calculate the offset distance

Mark as abnormal and dispatch orders

Mark as normal

On-site inspection

Revise the registered antenna position

Verified as abnormal?

Verify the prediction algorithm and try to optimize it

The algorithm can be optimized？

Update the algorithm

Mark as whitelist and no longer predict this antenna

Start

End

End

offset distance larger than threshold？

N

Y

N

Y

Y

N

End

Smart maintenance system of cell antenna

Operations personnel

Algorithm engineers

Calculate the predicted antenna position

Get the registered antenna position and calculate the offset distance

Mark as abnormal and dispatch orders

Mark as normal

On-site inspection

Revise the registered antenna position

Verified as abnormal?

Verify the prediction algorithm and try to optimize it

The algorithm can be optimized？

Update the algorithm

Mark as whitelist and no longer predict this antenna

Start

End

End

offset distance larger than threshold？

N

Y

N

Y

Y

N

End

Smart maintenance system of cell antenna

Operations personnel

Algorithm engineers

- Functional requirements for smart maintenance of cell antenna

## 8.1 Functional requirements for data source layer

Data source layer includes MR, S1-MME data and cell information. Some functional requirements for the data source layer that need to be considered for smart maintenance of cell include:

- Data source naming convention: In order to facilitate management and query, the naming of data sources is recommended to follow certain norms, such as using standardized naming conventions and abbreviations.

- Data source structure specification: The structure of the data source is recommended to follow certain specifications, including conventions on data type, length, format, encoding, etc., to ensure data consistency and readability.

- Data collection standards: When collecting data, certain standards are recommended to be followed, including standardization of collection time, frequency, and method, to ensure the integrity and accuracy of the data.

- Data storage standards: When storing data, certain standards are recommended to be followed, including standardization in data backup, recovery, encryption, etc., to ensure the security and reliability of the data.

- Data usage standards: When using data, certain standards are recommended to be followed, including standardization of data access permissions, data processing processes, data sharing and exchange, to ensure data interoperability and legality.

- Measurement report (MR) data: MR source data is required to include MR latitude and longitude, direction angle, timing advance (TA), reference signal received power (RSRP).

- S1-MME: S1-MME source data is required to include user identifiers such as international mobile subscriber identity (IMSI), cell information such as evolved node B (eNodeB) ID, Cell ID, and timestamps of signaling messages.

- Cell info: Information of the cells is required to include base station registered antenna data and location data in network management system (NMS), such as latitude and longitude, azimuth angle.

## 8.2 Functional requirements for data pre-processing layer

Data pre-processing refers to a series of processing work such as extracting the relevant source data, necessary data transformation(cleaning, integration, conversion, dispersion and reduction of the original data) and loading the transformed data into the target database, which has reached the minimum specifications and standards required by the knowledge acquisition research. Some functional requirements for the pre-processing layer that need to be considered for smart maintenance of cell include:

- Data extraction: It is required to retrieve relevant data from various heterogeneous sources such as databases, files, logs, or network interfaces. It ensures that the raw data needed for analysis is collected in a timely and accurate manner, and prepares it for subsequent processing steps.

- Data integration: It is required to combine and uniformly store data from multiple data sources, and establish a data warehouse to achieve data integration.

- Data cleaning: It is required to remove noise data and irrelevant data in the data set, process missing data and clean dirty data, missing values, identify and delete outliers, etc

- Data conversion: It is required to the convert data with smooth aggregation, data generalization, normalization, and other methods.

- Data reduction: It is required to reduce representation of the obtained dataset, which requires the maximum simplification of the data while maintaining the integrity of the original data, and the results are the same or almost the same as before the reduction.

- Data loading: It is required to import the transformed and cleaned data into the target data warehouse or data repository. It ensures data consistency and supports incremental or batch loading.

Note - For the requirements for pre-processing of feature data, see clause 8.4 in [ITU-T M.3381].

## 8.3 Functional requirements for model management layer

Model management layer includes modules for model training, executing and optimizating. Some functional requirements for the model management layer that need to be considered for smart maintenance of cell include:

- Predictive Analytics: It is required to use machine learning algorithms to predict potential issues before they occur and recommend preventive maintenance actions.

- Model Performance Monitoring: It is required to monitor the performance of algorithms, identify performance bottlenecks, and provide recommendations for optimization.

- Integration: It is required to integrate with other systems and tools used for cell maintenance, such as asset management systems, work order management systems, and performance monitoring tools.

- Security: It is required to ensure the security and privacy of the data stored and processed within it, including access control, data encryption, and compliance with data protection regulations.

- Model Optimization: It is required to ensure continuous improvement of model accuracy, efficiency, and adaptability to the evolving network conditions and data patterns. The model is required to be updated and refined based on the feedback from on-site patrol results of problematic cells, enabling a closed-loop optimization mechanism.

For other functional requirements, see clause 8.2.4 of [ITU-T M.3080], including feature data selection, common AI model selection, offline training and model decision.

## 8.4 Functional requirements for service provision layer

### Service provision layer includes modules for providing registered and predicted data and receiving the status feedback.

- Cell list service: This service provides a list of cell antennas with predicted parameters in the specified area and time. The data is required to at least include the area of the cell, the predicting time, cell ID, antenna ID, registered parameter values, and predicted parameter values.

- Feedback service: This service receives and stores a list of cell antennas that have been manually verified and confirmed to have no parameter errors in the specified area and predicting time. The data is required to include at least the area of the cell, the predicting time, cell ID, antenna ID, current parameter values, and predicted parameter values. The stored data can be used to analyze and optimize algorithms. If the algorithm cannot be optimized for that cell, it can also be set as a whitelist and no further analysis and processing will be conducted on the cell.

Note - The security and privacy of the data stored and processed within each layer is required, including access control, data encryption, and compliance with data protection regulations. For more security requirements, see clause 10 of [ITU-T M.3080].

- Requirements for related interfaces

The related interface is required to include the following external interface.

## 9.1 E1 interface

E1 interface is the interface between the cell antenna smart maintenance system and the resource system. This interface is required to support the information collection with the resource system. The interaction data includes cell data such as registered antenna position and azimuth angle.

## 9.2 E2 interface

E2 interface is the interface between the cell antenna smart maintenance system and the work order system. This interface is required to support information collection with the work order system, and mainly interact with the on-site patrol order.

## 9.3 E3 interface

E3 interface is the interface between the cell antenna smart maintenance system and the network maintenance system. This interface is required to support the collection of MR data and upload the cell maintenance result analysis data and report.

## 9.4 E4 interface

E4 interface is the interface between the cell antenna smart maintenance system and Internet map system. This interface is required to support object presentation on map and address searching.

# Appendix I

Use cases for smart maintenance of cell antenna

# (This appendix does not form an integral part of this Recommendation.)

## I.1 Use case template

The use cases should adopt the format of template that is defined in ITU-T M.3381 for consistent readability and convenient material organization.

|  | Prediction and Distribution Check and Feedback Review and Optimization Prediction and Distribution Check and Feedback Review and Optimization |
| --- | --- |

## I.2 Typical technical use cases of smart maintenance of cell antenna

### I.2.1 Prediction of antenna position based on minimization of drive test (MDT)

| Title | Prediction of antenna position based on MDT |
| --- | --- |
| Description |  | During the communication between the mobile equipment and the cell base station, MR data will be generated every few seconds. Some of these data contain the location information obtained by global navigation satellite system (GNSS) positioning. At the same time, there is a timing ahead parameter "n" in the data (unit: TA, each TA represents the distance between the mobile equipment and the base station D is fixed under the same wireless communication system, such as 4G, D = 78m/TA). Therefore, theoretically, the base station is located on the circle with a radius of n*D meters centered on the location of the mobile equipment. After collecting a large amount of global navigation satellite system measurement report (GNSS-MR) data, the base station location can be predicted by analyzing the distribution of these circular intersections. The overall process of location prediction of the base station is shown in the Graphic Section. - MR data processing: MR (Measurement Report) data is the measurement information reported to the network by the mobile equipment during service execution. The process follows: read the MR data, extract the MR containing GNSS position with the fields such as cell ID, longitude, latitude, and TA, extract MR data with TA equals to 0 or 1, and group the data by the cell ID: drop duplicates, aggregate data points with longitude and latitude difference within the verified threshold, and obtain cluster with the maximum number of points. - Longitude and latitude prediction model: The algorithm to use is the big data clustering algorithm. Take each cell as a unit, find the points of TA=0 and TA=1 from GNSS-MR data, then construct the prediction model to predict the longitude and latitude of each cell: - Make a circle with the latitude and longitude of each MR point as the center and TA * D and (TA + 1)*D as the radius; - Grid the ring with 10x10 meters as size. Generally, the cell only appears at the intersection grids of the all rings; - With the maximum likelihood algorithm, the point with the most grid intersections is taken as the location of the predicted cell, and if there are multiple points, the average is taken. - Since the model uses TA for judgment and analysis, the theoretical error is about D meters in one TA. For cells with a significant distance deviation between account data and predicted location, the predicted longitude and latitude can be used directly instead of the account data. In this way, there is no need to go to the site to measure the actual location and make corrections. However, since the 800M coverage is too far and the indoor base station has few GNSS-MR data, it is difficult to predict the 800M and the indoor base station. |  |
| Roles | Communication service provider, Infrastructure vendor, customer, Operation, Administration and Maintenance (OAM) engineer. |
| Graphic (optional) |  |
| Pre-conditions (optional) | GNSS-MR data of the target cell is available. |
| Post- conditions (optional) | Prediction results are ready for cell list service described in clauses 8.4. |
| Derived requirements | All requirements are based on clauses 8.2 and 8.3. |

### I.2.2 Prediction of antenna position based on trajectory filtering

| Title | Prediction of antenna position based on trajectory filtering |
| --- | --- |
| Description | For the scenario of lacking MDT data, the method described in I.2.1 cannot be used to predict the antenna position. If the adjacent cell has sufficient MDT data, the user trajectory fitting can be used to predict the antenna position. First, obtain the user ID of one or more user terminals accessing the target base station within a preset time period. Then, according to the user ID of each user terminal, obtain the mobile track information of each user terminal in the preset time period, and then determine the adjacent cell set of the target cell according to the mobile track information of each user terminal in the preset time period. Among them, the target cell is the cell covered by the target base station, and the adjacent cell set includes the previous adjacent cell and the next adjacent cell in the path of each user terminal when passing through the target cell. Finally, according to the location information reported by each user terminal in the previous adjacent cell and the next adjacent cell, the interpolation method is used to estimate the approximate location of each user terminal in the target cell. Then, the location information of the target base station is predicted according to the approximate location of each user terminal in the target cell. The longitude and latitude interpolation method mainly uses the interpolation method to estimate the approximate location of each user terminal in the target cell. The calculation method is as follows: determine the location point of each user terminal in the previous adjacent cell and the location point of the next adjacent cell as the start point and end point of the interpolation operation. First, calculate the first time difference between the reporting time of the starting point position information () reported by each user terminal and the reporting time of the ending point position information (). Secondly, the second time difference between the reporting time of each user terminal's missing location information () in the target cell and the reporting time () of each user terminal's reporting start point location is calculated. Then, take the first time difference and the second time difference corresponding to each user terminal as the denominator and numerator respectively, and calculate the interpolation ratio corresponding to the missing position of each user terminal in the target cell. Finally, the distance between the start point position and the end point position of each user terminal is multiplied by the interpolation ratio corresponding to the missing position of each user terminal in the target cell, and the position information of each user terminal in the start point position is added to obtain the approximate position corresponding to the missing position of each user terminal in the target cell. Where, is the longitude of the user track point within the coverage of the predicted cell, is the longitude of the user track point within the coverage of the next adjacent cell, is the longitude of the user track point within the coverage of the previous adjacent cell, is the latitude of the user track point within the coverage of the predicted cell, is the latitude of the user track point within the coverage of the next adjacent cell, and is the latitude of the user track point within the coverage of the previous adjacent cell. The approximate position of the user terminal can be calculated through the above formula. And then, the position record set obtained by interpolation is applied to the algorithm in 9.1 for position prediction. |
| Roles | Communication service provider, Infrastructure vendor, customer, OAM engineer. |
| Graphic (optional) |  |
| Pre-conditions (optional) | GNSS-MR data of the neighboring cells is available. |
| Post- conditions (optional) | Prediction results are ready for cell list service described in clauses 8.4. |
| Derived requirements | All requirements are based on clauses 8.2 and 8.3. |

### I.2.3 Prediction of antenna position based on neighboring cells

| Title | Prediction of antenna position based on neighboring cells |
| --- | --- |
| Description |  | This section presents a methodology for predicting antenna positions using neighboring cell data. The process involves: - Acquiring coarse user positioning data to calculate access time differences. - Identifying neighboring cells with access time differences below a first threshold. - Deriving inter-station distances for these cells using account data and calculating mean distances between target cells and their neighbors. - Applying density clustering to determine valid cell combinations where inter-station distances meet a second threshold. - Calculating overlapping areas by modeling neighboring cell positions as circle centers with radii based on inter-station distances. - Determining the densest overlapping region's geometric center as the target base station location. This approach corrects base station latitude/longitude deviations through predictive positioning. |  |
| Roles |  | Communication service provider, Infrastructure vendor, customer, OAM engineer. |  |
| Graphic (optional) |  |  |  |
| Pre-conditions (optional) | S1-MME data of the target cell and neighboring cells is available. |
| Post- conditions (optional) | Prediction results are ready for cell list service described in clauses 8.4. |
| Derived requirements | All requirements are based on clauses 8.2 and 8.3. |

### I.2.4 Prediction of antenna azimuth angle

| Title | Prediction of antenna azimuth angle |
| --- | --- |
| Description | 1. This model first filters MR data where RSRP exceeds a predefined threshold. From the filtered data, it selects subsets with accurate TA values. The predicted antenna azimuth is determined based on these accurate TA values. 2. The predefined RSRP threshold is iteratively adjusted by comparing the predicted azimuth with the registered antenna azimuth. Using the revised RSRP threshold, the process repeats: filtering MR data, selecting accurate TA values, and recalculating azimuths. 3. Based on the predicted azimuth, cells with antenna azimuth discrepancies are identified and sent for user verification/correction. 4. GNSS position data is extracted from base station MR records. MR fields (longitude/latitude, RSRP, base station ID, cell ID, TA) are joined with base station longitude/latitude using cell ID as the key. Data is grouped by cell ID . Calculate MR latitude and longitude mean using the following equation: μ=i=1nxin （1） Variance () equation follows: std=sqrt(i=1n(xi-μ)2n-1) （2） In the joined MR data, filter out those that meet the filter criteria. For each TA value in a single base station, determine the collection range of the corresponding location point as a circular ring area; For multiple TA values in a single base station, determine the area range of multiple concentric circular rings, which is the effective TA value area. First, the longitude and latitude mean are calculated respectively, and then filter out points are over 10 km from the mean. Recalculate the mean value of longitude and latitude, and filter out the points which are above (+ 3*) or below (- 3*). Azimuth prediction model : The predicted azimuth is determined according to the value of GNSS longitude and latitude and the accurate TA value. Gaussian kernel density estimation is a nonparametric method for estimating probability density function. Because the cell's azimuth is the angle from the antenna tilting towards the area with relatively concentrated density, the Gaussian kernel density estimation algorithm is used to calculate the actual azimuth. After data cleaning, for each point, its coordinate is set by longitude and latitude from MR, and the density of each point is calculated by using the kernel density estimation algorithm. The coordinate of the point with the highest density is the density center coordinate. Kernel density estimation formula is as follows: f(x)=1hNi=1NK((¦x-xi¦)h) （3） Where N is the number of total points, h is a fixed bandwidth, which is set to be 0.002 from experiments Gaussian kernel density formula is as follows: K(x)=12πexp(-12x2) （4） The predicted azimuth is the angle between the line from the base station location to the density center point and the north direction. The pseudo code is as follows: /*Calculate the angle (the input order is the sector longitude and latitude and the density center point longitude and latitude) Define the angle calculation function calculate_angle Input: two point coordinates x1: Double, y1: Double, x2: Double, y2: Double Output: Int*/ /* Calculate the two points’ azimuth. The north direction is zero degrees, and the angle increases by clockwise rotation */ initialize alpha = 0 // In the first quadrant if x2 > x1 && y2 > y1: theta = math.atan((x2 - x1) / (y2 - y1)) / math.Pi * 180 alpha = theta.toInt // In the second quadrant if x2 < x1 && y2 > y1: val theta = math.atan((y2 - y1) / (x1 - x2)) / math.Pi * 180 alpha = theta.toInt + 270 // In the third quadrant if x2 < x1 && y2 < y1: val theta = math.atan((x1 - x2) / (y1 - y2)) / math.Pi * 180 alpha = theta.toInt + 180 // In the fourth quadrant if x2 > x1 & y2 < y1: val theta = math.atan((x1 - x2) / (y1 - y2)) / math.Pi * 180 alpha = theta.toInt + 90 return alpha Calculation for Angle difference: When the angle difference is over 180, the angle needs to be converted (the subtraction of 350 degrees and 0 degrees is 350, and the actual angle deviation is 10 degrees). The formula is as follows: γ=¦α-β¦ , ¦α-β¦ <180° 360°-¦α-β¦ , ¦α-β¦ ≥180° The pseudo code is as follows: /*Calculate angle difference Define function calculate_error Input：x: Int, y: Int Output: Int*/ /* Calculate the difference of angles */ if math.abs(x - y) >= 180 if x >= y return 360 - x + y else return 360 - y + x else return math.abs(x - y) Detection of cells azimuth problems : Antenna feed reversal means that the antenna feeders of different cells in the same frequency band on the same base station are switched. In this case, the azimuth angles of the two cells are also exactly switched.When a cell meets one of the following conditions, it is considered as a cell with antenna azimuth problems: - For the first cell and the second cell, where the errors between the predicted antenna azimuth angle and the registered antenna azimuth angle are both within the predetermined range, if the difference between the predicted antenna azimuth angle of the first cell and the registered antenna azimuth angle of the second cell is less than the first predetermined angle, and the difference between the predicted antenna azimuth angle of the second cell and the registered antenna azimuth of the first cell is less than the second predetermined angle, it is determined that the base station antenna feeds of the first cell and the second cell are reversed. 2) For a cell where the error between the predicted antenna azimuth angle and the registered antenna azimuth angle is within the predetermined range, if the ratio of the number of MR data within the third predetermined angle range on both sides of the registered antenna azimuth angle of the cell to the total number of MR data is less than the predetermined ratio value, it is determined that the deviation of the antenna azimuth angle of the cell is greater than the predetermined threshold. |
| Roles | Communication service provider, Infrastructure vendor, customer, OAM engineer. |
| Graphic (optional) |  |
| Pre-conditions (optional) | GNSS-MR data of the target cell is available. |
| Post- conditions (optional) | Prediction results are ready for cell list service described in clauses 8.4. |
| Derived requirements | All requirements are based on clauses 8.2 and 8.3. |

# Appendix II

Typical business process for smart maintenance of cell antenna

(This appendix does not form an integral part of this Recommendation.)

From a business process point of view, smart maintenance of cell antenna includes the following closed-loop processes:

1) Prediction and distribution: the management department periodically performs the prediction and generates the results; Distributes the identified abnormal data to the relevant operation and maintenance department, via the list service described in clause 8.4.1.

2) Check and feedback: the operation and maintenance department checks the abnormal data, modify it if it is confirmed to be abnormal, and mark and feed back to the management department if not, via the feedback service described in clause 8.4.2.

3) Review and optimization: the management department reviews and analyzes the feedback data, and optimizes the prediction algorithm based on the specific situation.

Figure II-1 Typical business process for smart maintenance of cell antenna

# Bibliography

[b-ITU-R M.1797]    Recommendation ITU-R M.1797(2007), “Vocabulary of terms for the land mobile service”

[b-ITU-T K.70]      Recommendation ITU-T K.70(2020), “Mitigation techniques to limit human exposure to EMFs in the vicinity of radiocommunication stations”

[b-ITU-T L.94]      Recommendation ITU-T L.94(2015), “Use of global navigation satellite systems to create a referenced network map”

[b-ITU-T Q.1741.3]   Recommendation ITU-T Q.1741.3(2003), “IMT-2000 references to release 5 of GSM evolved UMTS core network”

[b-ITU-T Q.1742.11]  Recommendation Q.1742.11(2014), “IMT 2000 references (approved as of 31st December 2012) to ANSI-41-evolved core network with cdma2000 access network”

[b-ITU-T Q.762]      Recommendation Q.762 (1999), “Signalling System No. 7 – ISDN User Part general functions of messages and signals”

_______________________