3GPP TSG-SA5 Meeting #163	S5-254499

Wu Han, CHINA 13-17 October 2025

# Source:	Huawei

# Title:	Discussion Paper on the network slice related spending limit control

# Document for:	Endorsement

Agenda Item:	7.1

1	Decision/action requested

Discussion how to use the spending limit control for UE network slice charging.

2	References

[1]	3GPP TS 32.255: "Charging management; 5G data connectivity domain charging; Stage 2"

[2]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2"

[3]	3GPP TS 29.594: "5G System; Spending Limit Control Service; Stage 3"

[4]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3"

3	Rationale

According to the general requirements of policy and charging control specified in the clause 4.1 TS 23.503 [2]. The network slice and network slice replacement related policy and charging control are required.

4.1	General requirements

It shall be possible to apply policy and charging control to any kind of 3GPP and non-3GPP accesses defined in TS 23.501 [2].

The policy and charging control framework shall support the roaming scenarios defined in TS 23.501 [2].

The policy and charging control shall be enabled on a per slice instance, per DNN, or per both slice instance and DNN basis.

NOTE:	In single PCF deployment, the PCF will provide all mobility, UE access selection and PDU Session related policies that it is responsible for. In deployments where different PCFs support N15 and N7 respectively, the standardized interface between them supports policy alignment and exchange of relevant information.

The policy and charging control framework shall fulfil non-session management related requirements as defined in clause 4.2, session management related requirements as defined in clause 4.3 and network slice related requirements as defined in clause 4.4.

According to the 5G data connectivity charging requirements specified in the clause 5.1.2 TS 32.255 [1]. The network slice charging and network slice replacement charging are required.

5.2.1	Basic principles

5.2.1.1	General

…

The charging information collected per PDU session includes the network slice instance the PDU session belongs to.

…

5.2.1.17	Network Slice Replacement for PDU session

Per Network Slice Replacement feature specified in TS 23.501 [200] clause 5.15.19, the S-NSSAI serving a PDU session may be temporarily replaced with an Alternative S-NSSAI. When Network Slice Replacement is supported for PDU session charging:

…

Figure 1: The related reference points between PCF, SMF and CHF for Network slice

For the N7 interface, the network slice related policy from SMF for PDU session is supported and Network slice usage control is provide by the PCF via PCC rules. When the Network Slice Replacement feature is supported, the NF service consumer (SMF) notifies the PCF about network slice replacement as per the provisions of TS 29.512 [4].

For the N40 interface, the network slice charging from SMF for PDU session per UE and per S-NSSAI are supported which is specified in the TS 32.255 [1] clause  5.2.1.9 and 5.2.1.1. When the Network Slice Replacement feature is supported, NF service consumer (SMF) reports the original S-NSSAI and Alternative S-NSSAI to the CHF about the change between the initial S-NSSAI of the PDU Session and the Alternative S-NSSAI. The CHF can aggregate the counters per UE for the initial S-NSSAI and the alternative S-NSSAI.

For the N28 interface, The Nchf_SpendingLimitControl service enables the NF service consumer (e.g., PCF) to retrieve policy counter status information and spending limit reporting per UE from the CHF. For the scenarios of network slice charging and network slice replacement charging, the following issues should be considered.

-	Based on the current spending limit control mechanism specified in the TS 29.594 [3], the spending limit counter subscribed by PCF cannot carry network slice information, and CHF does not know how to accumulate the counter for the spending limit per network slices per UE. Especially, for the case that the network slice is not static configurated.

-	When the Network Slice Replacement feature is supported, If CHF is aware that the change from the initial S-NSSAI of the PDU Session to the Alternative S-NSSAI happened, CHF does not know whether the usage of switching network slices need be accumulated into the corresponding per UE spending limit counter or not. The alternative S-NSSAI is determined dynamically by AMF which cannot be pre-configured in the CHF and PCF, the spending limit control between the CHF and the PCF for policy decision for network slice replacement thus cannot be associated.

4	Detailed proposal

According to the above description, propose to send the LSout the CT3 about the addition of subscribers related network slice information for Spending Limit control