3GPP TSG-SA5 Meeting #163	S5-254501

Wu Han, CHINA 13-17 October 2025

# Source:	Huawei

# Title:	Discussion paper on the CHF Group ID

# Document for:	Endorsement

Agenda Item:	7.1

1	Decision/action requested

Discussion how to use the CHF group id in charging mechanism.

2	References

[1]	3GPP TS 32.290: "5G system; Services, operations and procedures of charging using Service Based Interface (SBI) "

[2]	LSin S5-253301/S2-2506095: "Reply LS on Extending Charging Support in 5GC"

[3]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3"

[4]	3GPP TS 23.501: "System architecture for the 5G System (5GS) "

[5]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2"

[6]	3GPP TS 29.503:	"5G System; Unified Data Management Services; Stage 3"

3	Rationale

The LSin (S5-253301 [2]) about extending charging support in 5GC from SA2 provides the clarification that the CHF group ID is a parameter independent from CHF address and when available is used for CHF selection via NRF.

3.1	Background

The corresponding changes of additional of CHF group id are implemented in the SA2/CT4 specifications:

3.1.1	Definition and CHF Discovery

The Definition of CHF group id is specified in the TS 23.501 [4].

CHF Group ID: This refers to one or more CHF instances managing a specific set of SUPIs.

6.2.6.2	  NF profile

NF profile of NF instance maintained in an NRF includes the following information:

…

-	UDM Group ID, range(s) of SUPIs, range(s) of GPSIs, range(s) of internal group identifiers, range(s) of external group identifiers for UDM.

-	UDR Group ID, range(s) of SUPIs, range(s) of GPSIs, range(s) of external group identifiers for UDR.

-	AUSF Group ID, range(s) of SUPIs for AUSF.

-	PCF Group ID, range(s) of SUPIs for PCF.

-	HSS Group ID, set(s) of IMPIs, set(s) of IMPU, set(s) of IMSIs, set(s) of PSIs, set(s) of MSISDN for HSS.

-	CHF Group ID(s), range(s) of SUPIs, range(s) of GPSI(s), list of PLMN(s) for CHF, as defined in clause 6.1 of TS 32.290 [67] and TS 29.510 [58].

…

6.3.11	CHF discovery and selection

The CHF selection functionality in NF consumer or in SCP should consider one of the following factors:

1.	CHF Group ID of the UE's SUPI.

NOTE 3:	The NF Consumer can infer the CHF Group ID the UE's SUPI belongs to, based on the results of CHF discovery procedures with NRF.

2.	SUPI; the NF consumer selects a CHF instance based on the SUPI range the UE's SUPI belongs to or based on the results of a discovery procedure with NRF using the UE's SUPI as input for CHF discovery.

3.1.2	CHF Group ID used in PCF Services

Based on the SA2 TS 23.503 [5], the "CHF group ID" was introduced in the policy information from PCF to SMF/AMF.

-	The CHF group id is different with the CHF address and without the association with charging address.

Figure 3.1.2-1: CHF Group ID in UE context policy control subscription information

Figure 3.1.2-2: CHF Group ID in PDU Session policy control subscription information

-	The CHF group id is different and no association with the Charging Information with containing CHF address.

Figure 3.1.2-3:PDU Session related policy information

Figure 3.1.2-4: Access and mobility related policy information

The CHF group ID contains the CHF Group ID of the CHFs that manages charging for the PDU Session and when received, the AMF/SMF shall apply it as defined in clause 6.3.11 of TS 23.501 [2].

3.1.3	CHF Group ID used in UDM Services

Based on the CT4 TS 29.503 [6], the "chfGroupid" was introduced in the subscriber data from UDM to SMF/AMF/SMSF. The corresponding data type "Nfgroupid" is the string which can be associated with the subscription to identify the CHF group.

Figure 3.1.3-1: chfGroupId in SessionManagementSubscriptionData

3.1.4	CHF Group ID used in CHF Discovery via NRF

Based on the TS CT4 29.510 [3], the "groupId" was used in the "ChfInfo" to identify CHF group that is served by the CHF instance.

NOTE 1:	If none of these parameters are provided, the CHF can serve any SUPI or GPSI managed by the PLMN of the CHF instance. If "supiRangeList" and "gpsiRangeList" attributes are absent, and "groupId" is present, the SUPIs / GPSIs served by this CHF instance is determined by the NRF (see 3GPP TS 23.501 [2], clause 6.2.6.2).

Figure 3.1.4-1: groupId in chfinfo

The "groupid" in NRF is the common attribute which can be used for the UDM, PCF, CHF and others (identified by supiRanges and gpsiRanges).

Based on the SA5 TS 32.290 [1], the CHF GroupID was described in the SA2 TS 23.501, which refers to one or more CHF instances managing a specific set of SUPIs.

The Nnrf_NFManagement service invoked by CHF for CHF instance(s) and CHF service(s) instance(s) registration described in the TS 29.510 [300] may include in particular:

-	The attributes in ChfInfo in NFProfile, as defined in clause 6.1.6.2.32, for example, Range(s) of SUPIs, Range(s) of GPSIs, Range(s) of PLMNs, CHF Group ID.

3.2	Use Case

The subscriber segmentation obtained from the subscription data (e.g. policy control subscription information in UDR or UE's subscription data in the UDM) will benefit the NF consumers to perform CHF discovery, i.e., one CHF Set will be used for post-paid subscribers only, and another dedicated CHF Set for prepaid subscribers.

According to the current "NFprofile" and "chfinfo" in TS 32.290 [1], the NF consumer can discover the CHF instances based on CHF group id, range of supi(s), range of gpsi(s), preferred-locality/ext-preferred-locality, PlmnRange, target-nf-service-set-id, etc..

Question and Suggestion:

-	Q1: Whether the "groupid" in TS 29.510 [3] is the same with the "chfgroupid" in the TS 29.503 [6], which can be used for the CHF discovery associated with subscription data?

S1: Send the LSout from SA5 to CT4 to clarify the description of "group id" in the "chfinfo" with the special information for the CHF selection.

-	Q2: Whether the "chfgroupid" used for scaling aspects (i.e., one or more CHF instances manage a specific set of SUPIs) can also support the Pre-paid and Post-paid subscriber segmentation information and operator’s charging properties. In other words, the set of SUPIs can indicated the group of Pre-paid/Post-paid subscribers.

S2: Research the subscription information specified in the TS 29.503 [6], the "UEC_SMF" and "supiRanges" may support the subscriber segmentation.

-	"SubscriptionData" without the subscriber segmentation to indicate Pre-paid/Post-paid.

-	"UEC_SMF" is the string attribute in the Enumeration DataSetName, describing UE Context in SMF Data, the UE Context may be used to indicate the subscriber segmentaion.

-	"UeContextInSmfData" includes the "pdusessions", "pgwInfro" and "emergencyInfo"

-	"supiRanges" may be used to indicate the subscriber segmentation.

4	Detailed proposal

According to the above description, the following proposals are present:

-	Clarify the "CHF group id", "Charging information" and "CHF address" of the SMF, SMSF and AMF charging specifications in the corresponding CR. (S5-254502, S5-254503, S5-254504).

-	Sent the LS out to CT4 to clarify the CHF group id.