3GPP TSG-SA5 Meeting #163	S5-254516

Wuhan, China, 13 – 17 October 2025

Source:	Nokia

Title:	Pseudo-CR on Add new use case and requirements on Minimum acceptable performance for AI/ML Model Training

Document for:	Approval

Agenda item:	 6.20.2

Spec:	3GPP TR 28.882

Version:	0.0.0

Work Item:	FS_AIML_MGT_Ph3

Comments

This pCR proposes to add use case and requirements for Minimum acceptable performance for AI/ML Model Training to TR 28.882.

Proposed Changes

* * * First Change * * * *

5 AI/ML Lifecycle Management Enhancements

## 5.X 	AI/ML Model Training

### 5.X.1 Minimum acceptable performance for AI/ML Model Training

#### 5.X.1.1 Description

The AI/ML model needs to be trained before being deployed for inference. The training may involve initial model development or the re-training of an existing AI/ML model. The AI/ML model is trained by the ML training MnS producer, either in response to requests from one or more ML training MnS consumers or initiated directly by the MnS producer (e.g., following a model performance evaluation).

#### 5.X.1.2 Use case

##### 5.X.1.2.1  Minimum acceptable performance for AI/ML model Training

In some cases, the AI/ML training MnS consumer may define both a target performance and a minimum acceptable performance of the AI/ML model. Whereas the target performance has been already specified in TS 28.105, the minimum acceptable performance requirements is missing. The minimum acceptable performance requirements may allow the MnS producer to stop training earlier, e.g. in order to allow the MnS Producer to reach other requirements, e.g. resource consumption, as long as the model still meets operational needs.

This approach balances consumer performance assurance with producer efficiency, enabling practical trade-offs between performance and other requirements, e.g. resource consumption requirements.

#### 5.X.1.3 Potential requirements

REQ-FS_AIML_MGT_Ph3-MAP-01: AI/ML Training MnS producer should have a capability allowing an authorized consumer to indicate the minimum acceptable performance of an AI/ML model to be achieved by the MnS producer during training.

* * * End of Change * * * *