3GPP TSG-SA5 Meeting #163	S5-254571

Wuhan, China, 13 - 17 October 2025

Source:	Nokia, Ericsson

Title:	Pseudo-CR on Rel-19 pCR TR 28.869 Add Evaluation to the use of VNF generic OAM functions

Document for:	Approval

Agenda item:	          6.19.6

Spec:	3GPP TR 28.869

Version:	1.6.0

Work Item:	FS_Cloud_OAM

Comments

The pCR proposes to add evaluation of solutions for the use cases in clause 5.1 of TR 28.869

Proposed Changes

* * * First Change * * * *

#### 5.1.1.4	Evaluation of solutions

The solutions presented in clauses 5.1.1.3.1, 5.1.1.3.2 and 5.1.1.3.3 describe how the VNF generic OAM functions can be used to convey/apply configuration of NF Deployments.  For these solutions, the interactions between the MnS Producers and the VNF generic OAM functions is through the interfaces specified in ETSI GS NFV-IFA 049 [2] and out of scope of 3GPP.

The solutions presented in clause 5.1.1.3.4 and 5.1.1.3.6 relies on the use of existing SA5 specifications to support configuration of NF Deployments. These solutions do not require any further enhancements to support the use case requirement REQ-CVNF_CM-1.

* * * Next Change * * * *

#### 5.1.2.4	Evaluation of solutions

The solution presented in clause 5.1.2.3.1 describes how the VNF generic OAM functions can be used to convey/apply policies related to the NF Deployments.  For this solution, the interactions between MnS Producers and the VNF generic OAM functions is through the interfaces specified in ETSI GS NFV-IFA 049 [2] and out of scope of 3GPP.

The solution presented in clause 5.1.2.3.2 relies on the use  of existing 3GPP SA5 specifications to support policy management for the NF Deployments. The solution does not require any further enhancements in the 3GPP management system to satisfy the use case requirement REQ-policy-1.

* * * Next Change * * * *

#### 5.1.3.4	Evaluation of solutions

The role of the 3GPP management system in supporting traffic management (as per the use case requirement REQ-CVNF_TM-1) is ambiguous. The use case has not defined what “NF Deployment traffic” means and the relation if any with 3GPP signalling traffic.

Subsequently, if NF Deployment traffic is not the 3GPP signalling traffic, then the traffic management use case is out of scope for 3GPP SA5 in this release.

* * * Next Change * * * *

#### 5.1.4.4	Evaluation of solutions

The solution presented in clauses 5.1.4.3.1 describes how the VNF generic OAM functions can be used to support the upgrade of the NF Deployments.  For this solution, the interactions between MnS Producers and the VNF generic OAM functions is through the interfaces specified in ETSI GS NFV-IFA 049 [2] and out of scope of 3GPP.

The solution presented in clause 5.1.4.3.2 relies on the use of existing SA5 specifications to support the upgrade of NF Deployments. The solution does not require any further enhancements in the 3GPP management system to satisfy the use case requirement REQ-CVNF_UP-1.

* * * End of Changes * * * *