3GPP TSG-SA5 Meeting #163	S5-254602

Wuhan, China, 13. - 17. October 2025

# Source:	Orange, AT&T

# Title:	R19 pCR TR 28.869 recommendation on observability requirements (Use case 7)

# Document for:	Approval

Agenda Item:	6.19.6

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP TR 28.869 Study on cloud aspects of management and orchestration

[46]	https://opentelemetry.io/

# 3	Rationale

- The objective of this pCR is to adapt the management functions to the cloud native evolution.

- This pCR proposes to enhance with recommendations for the use case “Observability for Cloud Native Network functions” in TR 28.869 (Study on cloud aspects of management and orchestration).

- It seems relevant for 5GA and 6G evolution.

# 4	Detailed proposal

### 5.2.7	Use case #7: Observability for Network Functions in supporting cloud native

#### 5.2.7.1	Description

Current network management solutions have disaggregated means of transferring data associated with metrics, events, logs and traces in term of protocols and data format.

The management of NFs from multiple vendors should be simplified through a standardized and unified 3GPP management system with open interfaces and enabling a simplified operational model for the operators. As we move to a native cloud architecture, it is essential that standards related to network management ensure efficiency and increased service resiliency.

The 3GPP management system needs to consider the collection of metrics, events, logs and traces management data for network functions in supporting cloud native.

The following elements represent principles behind a well-defined observability solution:

-	Observability:The 3GPP management system needs to support robust observability mechanisms for proactive telemetry data analysis, failure detection and rapid response.

-	Interoperability:The 3GPP management system needs to ensure interoperability between different vendors, providers and operators for harmonized network functions management in a multi-cloud environment.

-	Energy Efficiency:The 3GPP management system needs to consider Energy Efficiency by design from a network management perspective.

-	Security:The 3GPP management system needs to implement security by design for open observability protecting confidentiality and integrity of data transfer.

-	AI driven:The 3GPP management system needs to support the adoption of AI-driven approaches to enhance observability data in order to understand the root-cause analysis, data transaction monitoring, automated discovery, geo-hosted network functions topology mapping, predictive maintenance, capacity planning, security threats identification and utilization for further optimization (as examples and not necessarily limited to).

#### 5.2.7.2	Potential requirements

PREQ-OBS_CNF-Cloud-OAM-01: The 3GPP management system should have the capability to generate Metrics to the MnS consumer.

PREQ-OBS_CNF-Cloud-OAM-02: The 3GPP management system should have the capability to generate Logs to the MnS consumer.

PREQ-OBS_CNF-Cloud-OAM-03: The 3GPP management system should have the capability to generate Traces to the MnS consumer.

PREQ-OBS_CNF-Cloud-OAM-04: The 3GPP management system should have the capability to support standard data format.

PREQ-OBS_CNF-Cloud-OAM-05: The 3GPP management system should have the capability to support standard data collection and reporting mechanisms.

PREQ-OBS_CNF-Cloud-OAM-06: The 3GPP management system should have the capability to provide telemetry data correlation capabilities between the network functions.

PREQ-OBS_CNF-Cloud-OAM-07: The 3GPP management system should have the capability to provide operator configurable filtering capabilities for the telemetry data exchanged between the MnS producer and the MnS consumer.

PREQ-OBS_CNF-Cloud-OAM-08: The 3GPP management system should have the capability to securely expose the filtered  observability data to MnS consumers outside of the trusted PLMN domain.

PREQ-OBS_CNF-Cloud-OAM-09: The 3GPP management system should have the capability to securely ingest observability data from MnS producers or consumers outside of the trusted PLMN domain.

PREQ-OBS_CNF-Cloud-OAM-10: The 3GPP management system should have the capability to perform telemetry data correlation between the network functions and data provided by the MnS producers or consumers outside of the trusted PLMN domain.

#### 5.2.7.3	Potential solutions

"OpenTelemetry" [46] could be considered as a potential option for transferring the telemetry data (metrics, events, logs and traces) from the network functions to the unified management system.

1st Modified Section

#### 5.2.7.4	Evaluation

The solution in clause 5.2.7.3 describes in high level use of “openTelemetry” for transferring the data associated with metrics, events, logs, and traces from network functions to the management system.

The proposed solution fulfilss the use case requirements expressed in clause 5.2.1.2. Further study is required to identify any potential changes required in the existing specifications for support of the proposed solution.

2nd Modified Section

#### 6.X Observability for Network Functions in supporting cloud native

It is recommended that any changes required for support of the proposed solution in clause 5.2.7.3 be introduced in the normative phase.

Furthermore, it is recommended to study solutions to address requirements PREQ-OBS_CNF-Cloud-OAM-01 to PREQ-OBS_CNF-Cloud-OAM-10 in 6G.

End of modification