3GPP TSG-SA5 Meeting #163	S5-254372

Wuhan, CHINA 13 - 17 October 2025

Source:	Huawei, China Mobile

Title:	Pseudo-CR on TR 28.886 Add new use case on domain information for MDAFunction

Document for:	Approval

Agenda item:	6.20.7

Spec:	3GPP TR 28.886

Version:	0.1.0

Work Item:	FS_eMDAS_Ph4

Comments

It is proposed to add a use case “Indicating supported domain information in MDAFunction” in TR 28.886.

Proposed Changes

* * * First Change * * * *

## 5.2	Improvements to MDA framework

### 5.2.x	Use case x: Indicating supported domain information in MDAFunction

#### 5.2.x.1	Description

The IOC MDAFunction represents the MDA function which supports one or more MDA capabilities as described in TS 28.104 [2]. It also describes that an MDA function can act as 3GPP domain-specific (e.g. RAN or CN) or as 3GPP cross-domain MDA MnS producer. From MnS consumer perspective, it is reasonable to know not only the support MDA capabilities but also to identify in which domains the MDA Function is supported for corresponding analysis. For example, an MnS consumer may wish to perform analytic for CN, while the MDA producer is only capable to provide report for RAN. There is no way to identify the capability of the MDA producer in advance, and no way to report why the producer cannot satisfy the request.

Currently, in clause 9.3.1.2 in 28.104 [2] the definition of MDAFunction IOC, only contains supportedMDACapabilities attribute without domain information. This makes a MnS consumer difficult to know the actual analysis scope of a MDAFunction.

#### 5.2.x.2	Potential requirements

REQ-MDA_Fun-01: The MDA MnS producer should have the capability to allow authorized MDA MnS consumer to request the supported analysis domain information.

#### 5.2.x.3	Potential solutions

It is proposed to make the following changes to TS 28.104 [2] clause 9.3.1.2:

- adding a new domain attribute into Table 9.3.1.2-1 to indicate the supported analysis domain of the MDAFunction.

#### 5.2.x.4	Evaluation of solutions

* * * End of Changes * * * *