3GPP TSG-SA5 Meeting #163	S5-254415

Wuhan, China, 13. – 17. October 2025

Source:	NTT DOCOMO

Title:	Pseudo-CR on Rel-20 TR 28.881 Add use-case description, requirement and solution for enhancing feasibility check

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	0.1.0

Work Item:	FS_IDMS_MN_Ph4

Comments

This pCR provides use-case description, requirements and solutions for WT-3.1	Intent feasibility check capability enhancement to support intent guarantee requirements (e.g. resource reservation).

Proposed Changes

* * * Next Change * * * *

# X	Use cases

## X.Y	Use case #Y: Enhancing intent feasibility check capability

### X.Y.1	Description

This use case describes proposed enhancements for feasibility check, enabling intent-driven MnS consumer to be able to check feasibility of expectation contexts, as well as to receive recommendations together with feasibility check in case the intent is deemed as infeasible. This will support MnS consumers to understand the requirements needed to make an intent feasible for fulfilment.

In 3GPP TS 28.312 [1], clause 5.3.3.1 states the following:

“In case the result of intent fulfillment feasibility check is infeasible, MnS producer notifies the MnS consumer the reason of infeasibility and corresponding recommendations, then the MnS consumer decides how to handle the issue that intent is infeasible, e.g. update the intent, suspend the intent, delete the intent, etc.”

However, the issue is the IntentFeasibilityCheckReport specified in 3GPP TS 28.312 [1], clause 6.2.1.3.10 does not include corresponding recommendations. The MnS consumer may request exploration as specified in 3GPP TS 28.312 [1] Clause 5.3.5, based on inFeasibleExpectationInfos which is an optional attribute indicating the infeasible expectations including infeasible targets.  However, if the inFeasibleExpectationInfos attribute is not supported, then it is very difficult for the intent-driven MnS consumer to update an infeasible intent as they do not have the recommendation on how to make it feasible.

Another issue is that the infeasibility of an intent can be due to the constraints specified by intent-driven MnS consumer as expectation contexts, however the InFeasibleExpectationInfo in 3GPP TS 28.312 [1], clause 6.2.1.3.15.2 does not include information regarding infeasible contexts. Therefore, the consumer may not be able to update an infeasible intent to make it feasible, if the infeasibility is due to the expectation contexts.

### X.Y.2	Potential requirements

REQ-Intent_FEAS1: The intent driven MnS producer should have the capability to enable the authorized MnS consumer to request feasibility check with corresponding recommendations, if the feasibility check result is infeasible.

REQ-Intent_FEAS2: The intent driven MnS producer should have the capability to inform the authorized MnS consumer regarding infeasible expectation context, if the feasibility check result is infeasible.

### X.Y.3	Potential solutions

#### X.Y.3.1	Potential solution #1 for enhancing feasibility check report with recommendations

This solution proposes to enhance the intent feasibility check report defined in 3GPP TS 28.312 [1], clause 6.2.1.3.10.

In order to enable an MnS consumer to receive a feasibility check report with corresponding recommendations, a new attribute for recommendations is proposed to be included in feasibility check report, which indicates recommended expectation target and/or context values to make an infeasible expectation feasible.

#### X.Y.3.2	Potential solution #2 for a new procedure of feasibility check with exploration

This solution is alternative to potential solution#1 in clause X.Y.3.1 and proposes to reuse the existing generic intent exploration report and intent feasibility check report defined in 3GPP TS 28.312 [1] and to enhance the Intent <<IOC>>.

In order to enable an MnS consumer to request a feasibility check with corresponding recommendations, a procedure is proposed with ensuring feasibility check with exploration is supported and enabled by a new allowed value “FEASIBILITYCHECK_WITH_EXPLORATION” that is to be included for intentMgmtPurpose attribute of Intent <<IOC>> which is specified in 3GPP TS 28.312, clause 6.2.1.4. When feasibility check with exploration is requested by the intent-driven MnS consumer, the intent driven MnS producer can report corresponding recommendations for infeasible expectations with existing IntentExplorationReport <<dataType>>, together with existing IntentFeasibilityCheckReport <<dataType>>.

#### X.Y.3.3	Potential solution #3 for infeasible contexts

In order to enable an MnS consumer to get informed regarding infeasible expectation contexts, a new attribute inFeasibleContexts is proposed to be added in existing InFeasibleExpectationInfo <<dataType>> specified in 3GPP TS 28.111, clause 6.2.1.3.15.2 in order to report the infeasible contexts.

* * * End of Changes * * * *