3GPP TSG-SA5 Meeting #163	S5-254445

Wuhan, China, 13. - 17. October 2025

Source:	China Mobile

Title:	Pseudo-CR on TR 28.882 Add concepts and new use cases Management of AI/ML monitoring

Document for:	Approval

Agenda item:	6.20.2

Spec:	3GPP TR 28.882

Version:	V0.0.0

Work Item:	FS_AIML_MGT_Ph3

Comments

This contribution proposes to address issues with respect to AIML lifecycle management features for TR 28.882.

Proposed Changes

* * * First Change * * * *

5	Management capabilities for AI/ML lifecycle

5.A	Management of ML E2E LCM capability

5.A.1	Description

With the advancement of artificial intelligence and machine learning technologies, employing end-to-end lifecycle management (E2E LCM) mechanisms—e.g., AIOps, MLOps, LLMOps—has become an established trend. For example, MLOps denotes an end-to-end management framework that supports ML models across model design, data management, model training, model testing, inference emulation, continuous delivery/deployment, and continuous monitoring and operations (including inference, update, rollback, decommission, and retirement). The objective of these E2E LCM mechanisms is to enable repeatable, traceable, and measurable execution, whereby the MnS Consumer orchestrates lifecycle functions via workflows/pipelines to achieve automation and efficiency.

5.A.1.1	Management of AI/ML monitoring

TS 28.105 specifies use cases where predefined thresholds are monitored to trigger training initiation/retraining by the producer and defines indicators to be monitored during validation and testing, i.e., monitoring of ML model management aspects.

In addition, TS 28.105 specifies network-side monitoring associated with AI/ML inference (e.g., NG-RAN AI/ML-based distributed Energy Saving, Mobility Optimization, and Load Balancing), where monitored conditions can trigger activation/deactivation of the corresponding AI/ML Inference Functions—i.e., monitoring of network management aspects.

When an E2E LCM mechanism employs ML workflows to manage the ML model lifecycle, multiple dimensions of management data/information may need to be monitored. During the lifecycle, the MnS Consumer may wish to monitor targets including, but not limited to:

- Network conditions,

- Network performance (e.g., PM/KPI, service performance, etc.),

- ML model performance (e.g., ML model validation/testing performance, AI/ML inference performance per TS 28.105), and ML model I/O,

- ML LCM components (e.g., ML repository, ML Training Function, ML Testing Function, AI/ML inference emulation function, ML update function, AI/ML inference function, etc.),

- ML workflow artifacts (e.g., ML workflows, ML pipelines, and ML policies, etc.).

The MnS Consumer may aim to optimize the ML model by considering multi-dimensional monitored data/information and to receive suggested actions when the monitoring reflects a global or joint event. In such case, the MnS consumer should have the capability to request AI/ML monitoring that contains multiple monitoring target, based on which the producer would create AI/ML monitoring instances. The consumer can receive the analysis result which may contain the suggested actions by the producer according to the monitored data/information from a specific monitoring instance.

5.A.1.3	Potential requirements

REQ-AIML_MON-1: The AI/ML monitoring MnS producer should have a capability to enable an authorized MnS consumer to request the creation of AI/ML monitoring instances that contains multiple monitoring targets, e.g., network conditions, network performance, ML model performance, ML LCM related components and ML workflow artifacts.

REQ-AIML_MON-2: The AI/ML monitoring MnS producer should support a capability to enable an authorized consumer to define the monitoring characteristics related to a specific instance of AI/ML monitoring request.

REQ-AIML_MON-3: The AI/ML monitoring MnS producer may have a capability to identify a global or joint event according to AI/ML monitoring analysis result.

REQ-AIML_MON-4: The AI/ML monitoring may have a capability to report to an authorized consumer with the analysis result that contains the suggested actions by the MnS producer according to the monitored data/information from a specific monitoring instance.

5.A.1.4	Possible solutions

5.A.1.5	Potential requirements

* * * End of Changes * * * *