3GPP TSG SA WG5 Meeting #163	S5-254478

Wuhan, CHINA 13 - 17 October 2025

Source:	Huawei

Title:	Rel-20 pCR on TR 28.881 Add use case for the investigation on coordination between conflict resolution mechanisms

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	V0.1.0

Work Item:	FS_IDMS_MN_Ph4

Comments

Intent utility function is one kind of intent conflict resolution mechanism. There are other two kinds of intent conflict resolution mechanisms: intent conflict based on pre-emption and intent priority. Thus, a conflict handling guideline is needed for these three conflict resolution methods.

Therefore, this tdoc is to add use case for the investigation on coordination between conflict resolution mechanisms.

This contribution related to WT-3.

Proposed Changes

* * * First Change * * * *

4.X Use case #X: Investigation on coordination between conflict resolution mechanisms

4.X.1 Description

Three conflict resolution mechanisms are mentioned in TS 28.312[y], including conflict resolution based on intent priority, conflict resolution based on intent utility functions and conflict resolution based on preemption.. The Priority mechanism resolves conflicts by discarding or modifying the intent with the lower priority (1=highest, 100=lowest). The Utility mechanism involves the MnS producer selecting the outcome that maximizes overall utility based on the utility information provided by the MnS consumer. The Pre-emption mechanism uses the intentPreemptionCapability attribute to determine which intent is modified or deleted. While each mechanism provides a valid approach, their independent operation may lead to suboptimal resource allocation for the MnS Consumer. Therefore, a comprehensive conflict handling guideline is necessary. The MnS consumer can provide its preference for these conflict resolution mechanisms, which includes a preferred conflict resolution mechanism or the application order of two or more conflict resolution mechanisms. The MnS producer can handle the conflicting targets based on the received conflict resolution mechanisms.

4.X.2 Potential requirements

REQ-IDMS_IntentConflict-CON-01: The intent driven MnS producer should allow an authorized MnS consumer to specify the preference for different conflict resolution mechanism, including conflict resolution based on intent priority, conflict resolution based on intent utility functions and conflict resolution based on preemption.

4.X.3 Potential solutions

This solution proposes to enhance the existing Intent IOC defined in 3GPP TS 28.312 [y].

To support REQ-IDMS_IntentConflict-CON-01, an attribute "preferredConflictResolutionMechanism" needs to be introduced in Intent IOC to represent the MnS consumer's preference for different conflict resolution mechanism, including conflict resolution based on intent priority, conflict resolution based on intent utility functions and conflict resolution based on preemption. This attribute can indicate a preferred conflict resolution mechanism or the application order of  two or more conflict resolution mechanisms. The MnS producer can handle the conflicting targets based on the received preferredConflictResolutionMechanism.

### 4.X.4 Evaluation of potential solutions

TBD

* * * End of Changes * * * *