3GPP TSG-SA5 Meeting #163	S5-254492

Wuhan, China, 13 - 17 October 2025

# Source:	Huawei

# Title:	Rel-20 pCR 32.872 Introduce the usecase1 on Local Breakout Inter CHFs scenario

# Document for:	Approval

Agenda Item:	7.5.2

1	Decision/action requested

This is a pCR to TR 32.872 Introduce a usecase on Local Breakout Inter CHFs scenario.

2	References

[1]	3GPP TR 32.872: "Study on 5GA roaming charging reliability enhancement".

3	Rationale

This pCR proposes to introduce a new use case on the topic of Local Breakout Inter CHFs scenario.

4	Detailed proposal

Propose to incorporate the following change into the TR 32.872 [1]

First change

5.x	Topic x: Local Breakout inter CHFs scenario

5.x.1	Use Cases

5.x.1.1	Use Case #1a: V-CHF detected the failure of CTF

In the LBO N40+N107 charging architecture, for a specific PDU session, there is a charging session between the V-SMF(CTF) and V-CHF, and there is also a charging session between the V-CHF and H-CHF. When the V-CHF detects the failure of V-SMF(CTF), how the V-CHF handles the charging session with V-SMF(CTF) is specified in clause 5.5.1.2 of 3GPP TS 32.290[3], how the V-CHF handles the charging session with H-CHF should be specified.

The potential charging requirements for this UC are: REQ-3GPPCH-LBIC-1a.

5.x.1.2	Use Case #1b: CTF detected the failure of V-CHF

In the LBO N40+N107 architecture, for a specific PDU session, there is a charging session between the V-SMF(CTF) and V-CHF, and there is also a charging session between the V-CHF and H-CHF. when the V-SMF(CTF) detects the failure of V-CHF, the V-SMF(CTF) may send the charging message to an alternative V-CHF as specified in clause 5.5.2 of 3GPP TS 32.290[3]. In this case, how the interaction between the alternative V-CHF and H-CHF should be specified.

Figure 5.x.1.2-1: An example of CTF detected failure of V-CHF in LBO roaming

The potential charging requirements for this UC are: REQ-3GPPCH-LBIC-1b.

5.x.2	Potential charging requirements

REQ-3GPPCH-LBIC-1a: Charging system shall support handling the charging session between V-CHF and H-CHF when V-CHF detected a failure of the CTF.

REQ-3GPPCH-LBIC-1b: Charging system shall support handling the interaction between alternative V-CHF and H-CHF when V-CHF switched

5.x.3	Key issue

5.x.3.1	Key issue #1: Failure handling enhancement for failures detected between the CTF and the V-CHF

This key issue is for investigating how to support REQ-3GPPCH-LBIC-1a, REQ-3GPPCH-LBIC-1b. This investigation covers the following:

-	Identification of the failure handling enhancement for the scenario in which a failure is detected between the CTF and the V-CHF;

End of change