3GPP TSG-SA5 Meeting #163	S5-254512

Wuhan, China, 13 - 17 October 2025

Source:	ZTE Corporation

Title:	Pseudo-CR on TR 28.882 Add New Use Case on Management Support for Dataset and Model Parameters Exchange

Document for:	Approval

Agenda item:	6.20.2

Spec:	3GPP TR 28.882

Version:	V0.0.0

Work Item:	FS_AIML_MGT_Ph3

Comments

This contribution propose to Add New Use Case on Management of Vertical Federated Learning.

Proposed Changes

* * * First Change * * * *

5.X	Use cases

5.X.1         Management Support for Dataset and Model Parameters Exchange

Two-sided AI/ML CSI compression have investigated an AI/ML-based, two-sided spatial-frequency CSI-compression scheme. A CSI-generation model is deployed at the UE and a matching CSI-reconstruction model at the gNB. The two models cooperate to compress, feedback and reconstruct the CSI. This allows the UE to compress the CSI drastically while still guaranteeing that the gNB obtains a highly accurate channel estimate, thereby improving resource-scheduling and other subsequent operations.

To support UE-side offline training, it’s expected that the NW-side can share model parameters/dataset to the UE or UE-side via standardized approach. Management based solution can be developed according to the existing management mechanism to enable dataset/model parameter transfer from NW to UE training entity. In the study, both the following two approaches shall be taken into consideration:

- Non-OTA approaches, i.e., ‘NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)’. In the non-OTA approach, two options should be considered:

  - Option 1: OAM -> UE-side training entity (a server inside MNO or an OTT server), where OAM is NW-side dataset/model parameter collection entity.

  - Option 2: gNB -> OAM -> UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity.

The difference for the above two options is who first originates the model parameter/dataset. In option 1, management system itself is the collection entity and pushes the model parameter/dataset directly to the UE-side training server over the management interface without gNB involved. But in option 2, the gNB acts as the collection entity and it first delivers the model parameter/dataset to management system, which then forwards it to the UE-side training server. In this case, management system acts purely as a relay while the gNB remains the original source of dataset and model parameters.

- OTA approaches, i.e., ‘gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server)’. For the OTA approaches, the feasibility of OAM -> UE via gNB path should be investigated.

5.X.3	Potential Requirements

REQ-VFL_MGMT-01: The 3GPP management system shall have a capability supporting the dataset and model parameters collection and sharing via both OTA and non-OTA approaches.